3GPP TSG-RAN WG4 Meeting #104-bis-e                                                       R4-2215550

# Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NTN SAN PUCCH demodulation

Agenda item: 	4.2.7.2.2

Document for:	Discussion

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present simulation results for PUCCH demodulation requirements for NTN.

# Discussion

We discuss open issues related to the SAN demodulation requirements in our accompanying paper [2].

# Simulation parameters

    - PUCCH Format 0:

We use parameters in Table 1 taken from agreed way forward [3] for NTN PUCCH demodulation requirements.

Table 1 Simulation parameters for PUCCH format 0 demodulation requirements

| Parameter | Test |
| --- | --- |
| PUCCH format | 0 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Number of OFDM symbols | 2 |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | ACK missed detection probability <1% |

We show results for ACK missed detection probability for 15KHz SCS 5 MHZ CBW and 30KHz SCS 10 MHZ CBW in figure 1. We observe from Fig. 1 that the BLER performance is better with a smaller bandwidth and a higher number of Rx antennas, i.e., SCS 15Khz CBW 5 Mhz and 2 Rx antennas, respectively.

Figure 1 ACK missed detection probability for 15KHz SCS 5 MHZ CBW and 30KHz SCS 10 MHZ CBW with Table 1 configuration.

Observation 1: The ACK missed detection probability performance of the case with 1 Rx antenna is significantly worse than 2 Rx antennas.

    - PUCCH Format 2 (UCI BLER):

We use parameters in Table 2 taken from agreed way forward [3] for NTN PUCCH demodulation requirements.

Table 2 Simulation parameters for PUCCH format 2 (UCI BLER) demodulation requirements

| Parameter | Value |
| --- | --- |
| PUCCH format | 2 |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | [1], 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

We show results for block error probability for 15KHz SCS 5 MHZ CBW and 30KHz SCS 10 MHZ CBW in figure 2. We observe from fig. 2 that the BLER performance is better with a larger bandwidth and a higher number of Rx antennas, i.e., SCS 30Khz CBW 10 Mhz and 2 Rx antennas, respectively.

Figure 2 UCI block error probability for 15KHz SCS 5 MHZ CBW and 30KHz SCS 10 MHZ CBW with Table 2 configuration.

Observation 2: The BLER performance of the case with 1 Rx antenna is significantly worse than 2 Rx antennas.

    - PUCCH Format 3:

We use parameters in Table 3 taken from agreed way forward [3] for NTN PUCCH demodulation requirements.

Table 3 Simulation parameters for PUCCH format 3 demodulation requirements

| Parameter | Value |
| --- | --- |
| PUCCH format | 3 |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Cyclic Prefix | Normal |
| Additional DM-RS configuration | No additional DM-RS, Additional DM-RS |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | UCI block error probability<1% |

We show results for UCI block error probability for 15KHz SCS 5 MHZ CBW and 30KHz SCS 10 MHZ CBW in figure 3.

Figure 3 BLER for 15KHz SCS 5 MHZ CBW and 30KHz SCS 10 MHZ CBW with Table 3 configuration.

Observation 3: When the number of Rx antenna equals 1, the performance, i.e., BLER, is only slightly different with or without additional DMRS. When the number of Rx antenna equals 2, the BLER of the with additional DMRS (i.e., 2 DMRS) is better than that as compared to the case without additional DMRS (i.e., 1 DMRS).

Proposal 1:  Take results shown in this document into account when defining requirements for NTN demodulation

# Conclusion

This contribution discusses aspects related to NTN SAN demodulation requirements and has the following proposal and observations:

Observation 1: For PUCCH format 0, the ACK missed detection probability performance of the case with 1 Rx antenna is significantly worse than 2 Rx antennas.

Observation 2: For PUCCH format 2, the BLER performance of the case with 1 Rx antenna is significantly worse than 2 Rx antennas.

Observation 3: For PUCCH format 3, when the number of Rx antenna equals 1, the performance, i.e., BLER, is only slightly different with or without additional DMRS. When the number of Rx antenna equals 2, the BLER of the with additional DMRS is better than that as compared to the case without additional DMRS.

Proposal 1:  Take results shown in this document into account when defining requirements for NTN demodulation

# References

- RP-222556, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2215551, Discussion on NTN SAN PUCCH demodulation requirements, Nokia, Nokia Shanghai Bell

- R4-2214387, WF on NTN SAN demodulation requirements, Huawei