3GPP TSG-RAN WG4 Meeting # 104-bis-e                                		R4-2215559

Electronic Meeting, 15– 26 August, 2022

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.718-11-21: DC_8A_n3A-n257A/G/H/I/J/K/L/M

Agenda Item:	5.6.3

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE band B8 with NR bands n3 and n257 for TR 37.718-11-21[1].

# Reference

- Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL°

# Text Proposal

----- Start of TP -----

## 6.x	 DC_8_n3-n257

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n257 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.x.2	Configuration for DC

Table 6.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n3A-n257A DC_8A_n3A-n257G DC_8A_n3A-n257H DC_8A_n3A-n257I DC_8A_n3A-n257J DC_8A_n3A-n257K DC_8A_n3A-n257L DC_8A_n3A-n257M | DC_8A_n3A DC_8A_n257A |

### 6.x.3	Co-existence studies

For DC_8A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_8A_n3A is already given in TR 37.716-11-11.

### 6.x.4	∆TIB and ∆RIB values

For DC_1_n8-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n257 | 8 | 0.3 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n257 | 8 | 0 |
|  | n3 | 0 |
|  | n257 | 0 |

### 6.x.5	MSD

MSD analysis is already available for DC_8A_n3A is already given in TR 37.716-11-11.

No MSD analysis is needed for DC_8A_n257A, as there is no coexistence issue.

----- End of TP -----