3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215573

Electronic Meeting, 10 – 19 October 2022

Agenda Item:	4.3.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposal on measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The revised work item for extending current NR operation to 71 GHz was approved at TSG RAN#97-e [1]. One of the objectives of this work item is to define BS conformance testing requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. This topic was discussed at TSG RAN4#103-e and the WF was agreed [2-3].

This contribution proposes an approach to decide the measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz based on the agreed WF in [2].

# 2.	Discussion

The following points relating to measurement uncertainty and calibration were agreed in [2]:

  - The measurement capabilities resulting from different test equipment implementations should be studied and provided by test equipment vendors to decide the achievable and satisfactory measurement performance for BS type 2-O testing in the frequency range between 52.6 GHz and 71 GHz.

  - Measurement uncertainty and test tolerance analysis needs to be properly captured and therefore update to WID is needed to include TR 37.941.

  - Use of LNA if applicable should be in MU budget for FR2-2 except TX OFF power measurement. It is FFS whether mixer can be considered for some of test cases if applicable.

  - MU values for network analyser

The following information have been provided:

    - UE conformance testing, the MU of 1 sigma of 0.85 dB is assumed for the frequency range 40.8 GHz to 80 GHz.

    - AAS BS conformance testing (TR 37.941), the MU of 1 sigma of 0.30 dB is assumed for the frequency range up to 60 GHz.

Further analysis is required regarding what value to assume for FR2-2 MU evaluation.

  - As general principle, if a component is needed, it should be taken into account in MU budget.

  - The following aspect are FFS for FR2-2:

    - Add intermediate calibration stage of spectrum analyser absolute power accuracy for DL EIRP measurement, with the intension to break the trend with very large measurement uncertainties for high frequencies.

    - For test system measurement uncertainty, extend measurement procedures to enable for test setups to break the trend where measurement uncertainty grows rapidly as function of frequency.

    - If required introduce external mixer stage test setup and corresponding calibration procedures for receiver requirements and out-of-band requirements to improve measurement uncertainty.

    - Use of power sensor/meter for signal leveling should not be mandated for test requirements and related test setups.

It can be seen from there are numerous aspects to be studied for measurement uncertainty and calibration in the agreed WF in [2]. As the next TSG RAN4#105 meeting is scheduled to be the last one before the completion of the conformance testing part of the work item, to arrive on a timely agreement for the measurement uncertainty in the frequency range between 52.6GHz and 71GHz (currently denoted as FR2-2), it is proposed to adopt a pragmatic approach to obtain the measurement uncertainty for transmitter requirements in FR2-2 based on the existing ones in FR2-1 (formerly denoted as FR2), which are specified in clause 4.1.2.2 of TS 38.141-2 [4].

The measurement uncertainty for transmitter requirements in FR2-1 are provided in table 4.1.2.2-2 of TS 38.141-2, which is copied below.

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR2 OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) |
| 6.3 OTA base station output power | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | 1% |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | 600 kHz |
| 6.7.3 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |
| 6.7.4 OTA operating band unwanted emissions | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| 6.7.5.4 OTA transmitter spurious emissions, additional requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

It can be seen in table 4.1.2.2-2 that there are only 0.2 – 0.3 dB differences between the uncertainty values in different frequency ranges in FR2-1, except the 5.0 dB uncertainty value for transmitter spurious emissions which covers up to 60 GHz. These values have been decided based on detailed studies and discussions [5-7]. Considering the time constraint to complete the work item as targeted, it is proposed to allow additional 0.3 dB measurement uncertainty of BS OTA transmitter requirements in FR2-2 compared to the existing ones for the frequency range (43.5 GHz < f ≤ 48.2 GHz), except the 5.0 dB uncertainty value for transmitter spurious emissions which should be able to cover up to 71 GHz, i.e., table 4.1.2.2-2 of TS 38.141-2 should be updated as follows.

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR2 OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) ±2.5 dB (52.6 GHz < f ≤ 71 GHz) |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.8 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.3 OTA base station output power | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) ±2.9 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.9 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | 1% |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | 600 kHz |
| 6.7.3 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.1 dB (52.6 GHz < f ≤ 71 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.2 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.7.4 OTA operating band unwanted emissions | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.2 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 71 GHz |
| 6.7.5.4 OTA transmitter spurious emissions, additional requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 71 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

# 3.	Conclusion

This contribution has proposed an approach to decide the measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz based on the agreed WF, which is summarized as follows.

Proposal: To allow additional 0.3 dB measurement uncertainty of BS OTA transmitter requirements in FR2-2 compared to the existing ones for the frequency range (43.5 GHz < f ≤ 48.2 GHz), except the 5.0 dB uncertainty value for transmitter spurious emissions which should be able to cover up to 71 GHz.

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR2 OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) ±2.5 dB (52.6 GHz < f ≤ 71 GHz) |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.8 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.3 OTA base station output power | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) ±2.9 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.4.2 OTA RE power control dynamic range | N/A |
| 6.4.3 OTA total power dynamic range | ±0.4 dB |
| 6.5.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.9 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.5.2 OTA transmitter transient period | N/A |
| 6.6.2 OTA frequency error | ±12 Hz |
| 6.6.3 OTA modulation quality | 1% |
| 6.6.4 OTA time alignment error | ±25 ns |
| 6.7.2 OTA occupied bandwidth | 600 kHz |
| 6.7.3 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.1 dB (52.6 GHz < f ≤ 71 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.2 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.7.4 OTA operating band unwanted emissions | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) ±3.2 dB (52.6 GHz < f ≤ 71 GHz) |
| 6.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 71 GHz |
| 6.7.5.4 OTA transmitter spurious emissions, additional requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 71 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

# References

[1]	RP-222656, “Revised WID: Extending current NR operation to 71 GHz”, Qualcomm.

[2]	R4-2214373, “WF on FR2-2 BS test environments”, Ericsson.

[3]	R4-2214374, “WF on FR2-2 BS conformance testing”, Nokia, Nokia Shanghai Bell.

[4]	3GPP TR 38.141-2 v17.6.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.

[5]	R4-1811772, “WF on MU and TT for transmission in-band TRP emission and directional requirements for FR2”, Ericsson.

[6]	R4-1811889, “MU and TT for FR1 and FR2 Tx/Rx TRP spurious emissions”, Huawei.

[7]	R4-2016881, “WF on BS MU/TT for n262”, Nokia, Nokia Shanghai Bell, Ericsson.