3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215603

# Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda item:	4.2.5.1

Source:	Apple

Title:	On measurement procedure for NTN UE

Document for:	Discussion

# Introduction

In RAN4 #104e meeting, the WF[1] for NTN RRM was agreed, but there are still some open issues not fully addressed as followings,

### Issue 4. Fully Overlapping Concurrent MGs Agreement: - Option 1: Do not define requirements for fully overlapping concurrent MGs - Option 2: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2 - Option 2A: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms. - A MG with the lowest ID, i.e. 0, gets priority over the other, and the dropping rule starts from SFN=0, i.e. MG-ID#0 is selected and MG-ID#1 is dropped at the first collision instance after SFN=0, and it alternates afterwards. - [RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.] - Option 2B: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms. - RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only. - Option 2C: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

In this contribution, we discuss the above issues for concurrent MGs fully overlapped cases and some other potential issues to the current spec.

# Discussion

### Issue 4. Fully Overlapping Concurrent MGs

Regarding the overlapped MG cases, we think the round-robin method could be used for UE to measure all MGs rather than using priority rule. As discussed in last meeting, using priority rule would result in that one of the MG has no chance to be measured if two MGs are fully overlapped on time domain. In last meeting, some companies proposed that fully overlapping concurrent MGs is not configured for requirement design, but that’s a hard limitation on network implementation, which means high priority MG must have larger periodicity with low priority MG if they are somehow partially overlapped (low priority MG can only be used on those non-overlapped occasions), e.g., as mentioned in last meeting, it’s not possible to configure 160ms for both MGRPs. As long as Infra vendor has such configuration possibility, it makes less sense to preclude such scenario in the requirement design. And also, we didn’t see the benefit to use priority rule with restriction on MG configurations.

During the last meeting discussion, if the scaling method is used, there were 3 sub-options so far and the controversial part among those three options is: whether new UE capability of supporting “fully overlapping concurrent MGs” is needed or not, and whether a fixed pattern of using MG is needed or not. In our view, such fixed pattern is a kind of UE implementation restriction on NTN measurement, and therefore we prefer to leave the measurement behavior to UE implementation as long as the measurement is specified. Regarding the UE capability, we don’t understand on the motivation and benefit to have such capability since measurement order itself is up to UE implementation.

Proposal 1: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2.

- It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

### Measurement period scaling due to proximity between SMTC and MG

In last meeting, RAN4 has discussed the proximity rule between MG and SMTC, and agreed that,

For the case where one SMTC is inside MG and the other SMTC is outside the MG, if the proximity distance between the MG and SMTC outside the MG is smaller than or equal to the proximity distance threshold, i.e. 4ms, the two SMTCs are considered as colliding SMTCs.

### And during the discussion, other companies also mentioned that how to reflect such agreement in the spec is not very clear. We observed some potential impact to the existing NTN RRM requirement after considering this proximity conclusion, e.g., for the measurement requirement when SMTC are partially overlapped with MG.

### In TS38.133 section 9.2C.5.1(Intra-frequency without MG), it was specified that,

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1 When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

### In previous TN network, the partially overlapping has relation with the SMTC periodicity and  MGRP, i.e., partially overlapping happens when SMTC period < MGRP; and therefore the Kp is defined by using the SMTC periodicity and MGRP. As an example in the following figure 1, even though the SMTC periodicity < MGRP, all the SMTCs meet the proximity distance from MG, and therefore, this case shall be still treated as fully overlapping case.

Figure 1. Example of fully overlapping when SMTC period<MGRP

Observation: in NTN RRM measurement, even though the SMTC periodicity < MGRP, it can still be a fully overlapping case between SMTC and MG if all the SMTCs meet the proximity distance from MG.

Moreover, for some partially overlapping cases, it cannot only rely on SMTC period and MGRP to determine the Kp. As illustrated in the figure 2, 50% of the SMTCs for intra-freq measurement without MG cannot be used in such partially overlapping case, and therefore Kp=2. However, if we use MGRP and SMTC period, the Kp will be 1/(1-(10/40))=4/3. Furthermore, when NTN concurrent MG is used, the case will more complicated and also the Kp shall be not based on MGRP period or SMTC periodicity.

Figure 2. Example of partially overlapping case (Kp=2 but not 4/3)

### Based on the analysis above, the Kp for partially overlapping case shall be updated to considering the proximity between MG and SMTCs. The methodology used in MG enhancement can be one of the solutions here. We propose to define the Kp for intra-frequency and inter-frequency measurement without MG as following,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer. Otherwise, MGRP max is the MGRP of configured measurement gap.

-  Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1C.8.3.

Kp = 1 when Navailable_SAN = 0.

Proposal 2: Specify the following Kp definition for NTN intra-frequency measurement without MG and inter-frequency measurement without MG together with a definition of overlapping between SMTC and MG (based on agreement of proximity between SMTC and MG in RAN4 #104e),

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows: - For a window W of duration max(SMTC period, MGRP_max), where - If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer. Otherwise, MGRP max is the MGRP of configured measurement gap. - Starting from the beginning of any SMTC occasion: - Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and - Navailable_SAN is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1C.8.3. Kp = 1 when Navailable_SAN = 0.

### One layer definition for concurrent MG

### In RAN4#103e meeting, RAN4 sent LS(R4-2210611) to RAN2 to clarify that,

RAN4 thanks RAN2 for the LS sent in R2-2204114 asking about the feasibility for NR NTN when one frequency layer is associated to both concurrent measurement gaps with the same gap type. RAN4 reached consensus in this matter that one frequency layer can be associated to both concurrent measurement gaps with the same gap type. There is no need to define additional NTN UE capability for this association.

### In last RAN2 meeting, 2 CRs related with such reply LS information were agreed: R2-2209247 is for TS38.331 and R2-2208788 is for TS38.306.

### TS38.331

### TS38.306

### However, RAN4 has not discussed any MG based or MG-less based CSI-RS L3 measurement in NTN, and the frequency layer in RAN4 LS is concluded for SSB based RRM measurement rather than CSI-RS L3 measurement, in our understanding. Also, RAN4 didn’t discuss the definition of frequency layer for CSI-RS L3 in NTN or mixed SSB and CSI-RS RRM measurement. In order to make the capability and signaling clear and to avoid ambiguity/mismatch between RAN4 and RAN2, we propose to send LS to RAN2 to further clarify that the frequency layer in last RAN4 LS is based on the conclusion of SSB based RRM measurement. The following draft LS is proposed.

### Proposal 3: RAN4 to send a follow-up LS to RAN2 for previous LS(R4-2210611) that,

### One frequency layer can be associated to both concurrent measurement gaps with the same gap type for SSB based RRM measurement. RAN4 has no discussion on CSI-RS L3 measurement requirement for NTN in R17.

# Conclusion

In this contribution, we discuss the above issues for concurrent MGs fully overlapped cases and some other potential issues to the current spec.

Proposal 1: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2.

- It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

Observation: in NTN RRM measurement, even though the SMTC periodicity < MGRP, it can still be a fully overlapping case between SMTC and MG if all the SMTCs meet the proximity distance from MG.

Proposal 2: Specify the following Kp definition for NTN intra-frequency measurement without MG and inter-frequency measurement without MG together with a definition of overlapping between SMTC and MG (based on agreement of proximity between SMTC and MG in RAN4 #104e),

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows: - For a window W of duration max(SMTC period, MGRP_max), where - If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer. Otherwise, MGRP max is the MGRP of configured measurement gap. - Starting from the beginning of any SMTC occasion: - Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and - Navailable_SAN is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1C.8.3. Kp = 1 when Navailable_SAN = 0.

### Proposal 3: RAN4 to send a follow-up LS to RAN2 for previous LS(R4-2210611) that,

### One frequency layer can be associated to both concurrent measurement gaps with the same gap type for SSB based RRM measurement. RAN4 has no discussion on CSI-RS L3 measurement requirement for NTN in R17.

# References

[1] R4-2214471, WF on NR NTN RRM requirements, Qualcomm, RAN4 #104e.