3GPP RAN WG4 Meeting #104bis		R4-2215648

Electronic meeting, October 10 – 19th, 2022

Agenda item:	6.28.2

Source:	Apple

Title:	On enabling first 20MHz for bands n96 and n102

WI/SI:	NR_unlic_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which has been approved recently in different regulatory regions. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI [2], TSG RAN#96 meeting approved a new WI [3] with the main motivation of enabling the LPI and VLP modes in countries not covered by previous efforts. In addition to providing support for the LPI and VLP modes, the new Rel-18 WI added another objective to enable first 20MHz chunk of spectrum, i.e. 5925-5945MHz, for bands n96 and n102. The matter is that while both bands n96 and n102 starts at 5925MHz, there are no channel raster points covering 5925-5945MHz range.

During the RAN4#104 meeting companies discussed about the best way to proceed with the first 20MHz chunk of spectrum and several options were identified as summarised in [4]. In this discussion paper we present our initial considerations on adding support for the first 20MHz with associated problems and challenges.

# 2	5925-5945MHz for bands n96 and n102

3GPP bands n96 and n102 are the two bands defined for the shared spectrum access in the whole and lower part of the 6GHz band. Referring to an excerpt from TS 38.101-1, both bands start at 5925MHz and the only difference between them is the upper edge and the geographical areas where they can be used. However, even though both bands start at 5925MHz, there are no corresponding channel raster points that would allow utilising first 20MHz chunk of spectrum, i.e. 5925-5945MHz.

Table 2-1: Band n96 and n102 ranges.

| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| --- | --- | --- | --- |
| n9715 | 2300 MHz – 2400 MHz | N/A | SUL |
| n9815 | 1880 MHz – 1920 MHz | N/A | SUL |
| n9916 | 1626.5 MHz – 1660.5 MHz | N/A | SUL |
| n100 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |
| n10214 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |

In principle there are no obstacles to define additional channel raster points so that first 20MHz can be used for 20MHz (and larger) NR-U channels. However, one of the major concerns is that, as discussed in Rel-16 and Rel-17, some countries have quite strict out-of-band emission requirements. Referring to the summary of regulatory requirements in Annex A, there are countries such as EU/CEPT, South Korea and US that define quite strict out-of-band requirements, which in turn results in quite larger A-MPR back-off values.  Referring to some preliminary A-MPR results captured for the PC5 VLP mode in South Korea (see sub-clause 6.1.1.2.2 in TR 38.849), the UL channel residing right at the edge of the band might require approximately 3dB higher back-off comparing to the centre channel. And the same phenomena is observed for the edge and centre channel A-MPR values simulated for EU/CEPT. Based on that it is somewhat questionable whether there is any benefit in configuring the UL channel at the edge of the band in a situation when the unlicensed band offers plenty of opportunities for other locations. At the same time, the edge channel still can be used for the DL operation because it is usually the case that the base station transmitter can ensure much better protection of neighbouring bands. In other words, the edge channel can always be used as a secondary cell in the multi-carrier configuration for the UE.

Based on these considerations the following options were identified during the RAN4#104 meeting:

-	Option 1: Channel at band edge which have larger A-MPR than current edge channels shall be downlink only

-	Option 2: Enable first 20MHz for DL only

-	Option 3: Do not define the additional channel raster points

Based on the presented considerations we suggest adopting Option 2 as a starting point, i.e. enable first 20MHz only for DL channels for all countries. Option 1 can be explored further to understand better how it can be captured in the specifications and/or which specification impact it will have.

Proposal 1:	As a baseline, enable first 20MHz for band n96 and n102 only for NR-U DL channels.

Proposal 2:	First 20MHz can be considered for NR-U UL channels if it does not result in higher A-MPR than inner channels (depending on the progress of the technical work and/or number of countries that can benefit from this enhancement).

# 3	Conclusions

In this contribution we have presented our further considerations on enabling first 20MHz channel for the NR-U unlicensed bands n96 and n102. As explained in the paper, the easiest way is to enable first 20MHz channel only for the DL operation (covering all countries). Other options can be further explored to understand which countries can benefit from the UL operation in that frequency range and how it can be supported with in specifications.

Proposal 1:	As a baseline, enable first 20MHz for band n96 and n102 only for NR-U DL channels.

Proposal 2:	First 20MHz can be considered for NR-U UL channels if it does not result in higher A-MPR than inner channels (depending on the progress of the technical work and/or number of countries that can benefit from this enhancement).

# 4	References

- RP-192926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-213180, "Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2214439, "WF on channel raster extension for NR-U 6GHz bands", Apple Inc.

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |