3GPP RAN WG4 Meeting #104bis-e		R4-2215662

Online, October 10th – 19th, 2022

Agenda item:	6.2.2

Source:	Apple

Title:	On CA_n5-n8 architectures

WI/SI:	FS_NR_700800900_combo_enh

Release:	Rel-18

Document for:	Approval

# 1	Introduction

Carrier aggregation has been successful in progressively achieving higher data rate since its inception in LTE Rel-10. Despite the number of integrated bands in FR1 range alone has been up to 6, the sub-1GHz band combinations have not yet gained much traction in smartphones, due to the known implementation challenges in RF front-end multiplexing filter design for closely spaced frequency bands and limited form factor to provide sufficient antenna frequency range coverage. Nonetheless, the desire for combining the narrowly and fragmentally allocated sub-1GHz spectrum to achieve higher data rate still remains which has prompted a new study item in RAN #96 meeting to identify the issues and investigate the solution to enable the support of band combinations made up of 700/800/900MHz for a smartphone, specifically for the three example band combinations, CA_n5-n8, CA_n5-n28, and CA_n8-n20-n28 [1]. In last RAN4 meeting, the general aspects of the frequency range restriction, architecture assumption, channel band width support, and UL configuration for the three intended band combinations have been first discussed which were concluded with an approved WF [2]. To follow up on the WF, in this contribution we share our views on the potential architecture variants and their implications on UE RF requirements for CA_n5-n8 based on the WF architecture assumption for the number of antenna.

# Discussion

The main issue for CA_n5-n8 is that there is substantial frequency overlap between n5 DL range and n8 UL range, as shown in Figure 2-1. As a result, there would be no feasible filter to isolate n5 DL and n8 UL. On the other hand, although the proponent of this CA combination had clarified that the operation frequency ranges of the combination are restricted to:

n5: UL 824 - 835 MHz; DL 869 - 880 MHz

n8: UL 904 - 915 MHz; DL 949 - 960 MHz

which avoids the overlap between n5 DL and n8 UL. However, such restriction cannot be used as the RF multiplexer implementation guideline as the filter design should accommodate the full-range operation when being operated as a single band for all the constituent bands.

Figure 2-1 CA_n5-n8 spectrum range

Proposal 1: The frequency range restriction of the combination shall not be used as the RF multiplexer implementation guideline as the filter design should accommodate the full-range operation when being operated as a single band for all the constituent bands.

On the UE architecture assumption, both 2 and 3 antenna implementations have been considered in the approved WF in last RAN4 meeting [2]. Due to the frequency range overlap between n5 DL and n8 UL, one possibility to enable CA_n5-n8 operation with 2-antenna implementation is to allow only non-simultaneous Rx/Tx between n5 DL and n8 UL. With that the filter isolation between n5 DL and n8 UL would no longer be needed and a triplexer can be used to provide the semi-full-duplex carrier aggregation to a single antenna in the main path, as is shown in Figure 2-2. On the other hand, since the triplexer center band range is over 5% bandwidth ratio (band range divided by center frequency) and the frequency gap between center band and n8 DL is only 10 MHz which may pose design challenge on achieving acceptable filter isolation and insertion loss, the feasibility of a single triplexer for CA_n5-n8 needs to be studied.

Figure 2-2 Possible CA_n5-n8 UE architecture and operation diagram for semi-full-duplex CA

Observation 1: One possibility to enable CA_n5-n8 operation with 2-antenna implementation is to allow only non-simultaneous Rx/Tx between n5 DL and n8 UL in conjunction with a triplexer in main path.

Proposal 2: For CA_n5-n8 with 2-antenna implementation, the feasibility of a single triplexer for CA_n5-n8 with acceptable filter isolation and insertion loss needs to be studied.

For the 3-antenna implementation, the two of the three antenna are used in the main path to aggregate the n5 and n8 signals over the air, as shown in Figure 2-3. As n5 and n8 signals do not need to be combined through a multiplexer, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation.

Observation 2: For CA_n5-n8 with 3-antenna implementation, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation since n5 and n8 signals do not need to be combined through a multiplexer.

Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation which can be up to 348 MHz (from APT600 band to n8) to support all the sub-1GHz bands above 600 MHz as defined in 3GPP. Nonetheless, using two antenna to aggregate n5 and n8 may not be the optimal antenna arrangement to divide the sub-1GHz band frequency range as the lower frequency antenna still needs to cover up to 282 MHz (from APT600 band to n5). On the other hand, the additional antenna cannot be added without occupying more phone space. Therefore, the feasibility on placing more than two low-band antenna in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

Observation 3: Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation.

Observation 4: Using two antenna to aggregate n5 and n8 may not be the optimal antenna arrangement to divide the sub-1GHz band frequency range as the lower frequency antenna still needs to cover up to 282 MHz (from APT600 band to n5)

Proposal 3: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

Figure 2-3 CA_n5-n8 UE architecture based on the 3-antenna implementation

Lastly, despite the 3-antenna implementation can help reduce n8 UL interfering n5 DL via antenna isolation which however is far from sufficient even under the restricted frequency ranges for CA operation where n8 UL does not overlap with n5 DL. Therefore, semi-full-duplex operation with non-simultaneous Rx/Tx between n5 DL and n8 UL as diagramed in Figure 2-2 is still required for the CA combination.

Proposal 4: Semi-full-duplex operation with non-simultaneous Rx/Tx between n5 DL and n8 UL is required for CA_n5-n8 irrespective of 2- or 3-antenna implementation.

# 3	Conclusion

In this contribution we share our views on the potential architecture variants and their implications on UE RF requirements for CA_n5-n8 based on the WF architecture assumption for the number of antenna.

Observation 1: One possibility to enable CA_n5-n8 operation with 2-antenna implementation is to allow only non-simultaneous Rx/Tx between n5 DL and n8 UL in conjunction with a triplexer in main path.

Observation 2: For CA_n5-n8 with 3-antenna implementation, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation since n5 and n8 signals do not need to be combined through a multiplexer.

Observation 3: Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation.

Observation 4: Using two antenna to aggregate n5 and n8 may not be the optimal antenna arrangement to divide the sub-1GHz band frequency range as the lower frequency antenna still needs to cover up to 282 MHz (from APT600 band to n5).

Proposal 1: The frequency range restriction of the combination shall not be used as the RF multiplexer implementation guideline as the filter design should accommodate the full-range operation when being operated as a single band for all the constituent bands.

Proposal 2: For CA_n5-n8 with 2-antenna implementation, the feasibility of a single triplexer for CA_n5-n8 with acceptable filter isolation and insertion loss needs to be studied.

Proposal 3: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor.

Proposal 4: Semi-full-duplex operation with non-simultaneous Rx/Tx between n5 DL and n8 UL is required for CA_n5-n8 irrespective of 2- or 3-antenna implementation.

# 4	References

RP-221464 “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, CATT, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6th – 9th, 2022

R4-2214445 “WF on study on FS_NR_700800900”, CATT, 3GPP TSG-RAN WG4 Meeting #104-e, online, August 15th – 26th, 2022