3GPP RAN WG4 Meeting #104bis-e		R4-2215669

Online, October 10th – 19th, 2022

Agenda item:	5.27.2

Source:	Apple

Title:	Refsens and blocking for the 600MHz APT band

WI/SI:	NR_600MHz_APT-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

In RAN#96 a new work item to define a new band for APT 600MHz has been agreed in RP-221778 [1]. It proposes to define a new band based on the B1 option in the study item 38.860 [2] using a DL frequency range from 612-652 MHz and an UL frequency range from 663-703MHz. It is proposed to use a single duplexer. This document describes some RX issues and proposes solutions to overcome these issues.

# Discussion

## 2.1	Support of n105 and n71

Millions of n71 capable devices have already been sold and are available on the market. The new band will use the same frequency range as n71 but with 5MHz added at the lower end of the RX band and 5 MHz added at the upper end of the TX band.

Figure 1: The frequency arrangement of the new band n105 and n71

UEs are expected to support both bands, the upcoming n105 and n71, using the same RF components in order to support roaming. Additionally n71 is also used in Asia, i.e. the area of the APT, already, so it is clear that there cannot be UEs that only support n105 and not n71. Therefore the requirements for n105 need to be specified in a way that also the n71 requirements can be supported with the same RF hardware including the duplexer.

Observation 1: n71 is already used in the APT region, not only in the US, so a device used in the APT region needs to be able to support n105 and n71.

Proposal 1: RAN4 should specify the RF performance parameters for n105 in such a way that the same hardware can support n105 and n71.

## 2.2	RX blocking for n71

As we have seen above, the n105 hardware, in this case the duplexer, also needs to be used for n71, therefore the UE needs to pass the blocking test for n71 using the n105 duplexer.

The blocking test for n71 requires the UE to receive the wanted signal in the channel above 617MHz, while a blocker going up to 608MHz at a power level of -15dBm is present. At the same time the bandwidth of the RX filter of the duplexer is extended by 5MHz towards the blocker down to 612MHz. The remaining gap is just 4MHz, within which the attenuation needs to change from passband with low loss to the stop band with the best possible rejection to suppress the blocker.

At the same time there will be shifts of the attenuation curves due to the process variations as well as temperature, even temperature compensated SAW filters are not perfectly independent of temperature but show some small shifts which result in issues when using such small gaps. It is absolutely necessary to take production tolerances and temperature drifts into account, when defining 3GPP minimum requirements. It is not possible to base these 3GPP minimum requirements on an ideal tolerance free simulation of a duplexer.

So the main issues for such a duplexer are:

- Production tolerances resulting in decreased performance (higher IL, lower stopband attenuation)

- Imperfections of temperature compensation resulting in decreased performance

- Reduced attenuation at 608MHz due to the transition to the passband. We can see performance degradations of up to 20dB due to process and temperature variations

- Increased insertion loss at 612MHz and above due to the transition from the passband to the stopband, we can see about 1dB worse insertion loss at 612MHz compared to 617MHz.

- The better the rejection is against the TV blocker, the larger the insertion loss is degrading Refsens on the lowest channel

Proposal 2: RAN4 should specify the RF performance parameters for n105 taking into account the production tolerances, process spread as well as temperature drifts of duplexers supporting both, n105 and n71.

## 2.3	Refsens for n105

Due to the difficulties to design a duplexer as discussed above, we do not see any room for improvement of the Refsens compared to n71 devices. The baseline of the Refsens shall be the same as for n71. The additional 5MHz from 612MHz to 617MHz are especially difficult due to the n71 blocking requirement resulting in up to 1dB of insertion loss degradation at 612MHz compared to 617MHz. Therefore we propose to add 0.5dB to the Refsens requirement within 612 … 617MHz to compensate at least for half of the additional losses due to the increased bandwidth of the n105 duplexer.

Observation 2: The performance of a n105 duplexer is much more difficult to achieve, as it is not a dual duplexer solution as specified for n71 but a single duplexer with a much larger bandwidth/duplex gap ratio and it has to fulfill a the same blocking requirement 5 MHz closer to the passband.

Proposal 3: RAN4 should specify the Refsens for n105 by re-using the Refsens values from n71 and adding +0.5dB for the frequency range 612-617MHz to account for the much tighter requirements on the duplexer compared to n71.

## 2.4	RX blocking for n105

There has been a big discussion in RAN4#104 about the RX blocking for n105. While some companies proposed to just copy the requirements from n71, it was overlooked, that the interfering TV stations have a maximum of 200kW compared to the 1000kW of the TV stations in the US, that have been taken into account for n71. Since the TV stations are 7dB lower in power, it seems reasonable to also reduce the blocker level from -15dBm to -22dBm.

Observation 3: The TV stations in the APT region have 200kW compared to 1MW in the US. The blocker level can be relaxed accordingly.

Proposal 4: RAN4 should specify the in-band blocker level range 3 for n105 by reducing the -15dBm from n71 to -22dBm for n105 due to the 7dB lower TV power levels within the APT region.

In the last meeting there has been a text proposal for the APT600 TR citing a study from APT on TV interference. Although it has not been agreed to add this to the TR without adapting the text to the TR, there was some interesting information on the blockers included, for example the frequency offsets used for the study as copied in the table below:

Table A1 – Options and cases evaluated for DTT and mobile co-existence

| Option | DL DTT channel style/band | DL mobile |
| --- | --- | --- |
| B1: 40 MHz Full Band duplexer | 6 MHz Channel Raster/Spacing 596-602 MHz (CH 35) | 612-652 MHz |
|  | 8 MHz Channel Raster/Spacing 590-598 MHz (CH 36) 598-606 MHz (CH 37) |  |
|  | 7 MHz Channel Raster/Spacing 596-603 MHz (CH 38) |  |

The question coming up from this table is: What is the highest frequency of a TV blocker in the APT region? According to this table from the APT study, channel 36 with the 6MHz raster is not used for TV transmissions, so that the highest blocker frequency used by a TV transmitter would be 606MHz using 8MHz channel 37. This results in 6MHz gap from the upper edge of the TV channel to the lower band edge of n105.

It seems reasonable not to use channel 36 in a 6MHz TV channel raster configuration, since it would not only be an issue for the n105 receiver, but a TV receiver at channel 36 would also suffer from a blocking issue with the neighboring NR basestation which can have high signal levels at the upper two MHz of the adjacent channel. Since TV receivers are broadband and don’t use SAW filters to reduce the NR interference, most likely this TV channel cannot be used by the TV stations, since the TV receivers would get significant interference from the NR BS. It seems useful not to take usage of 6MHz channel 36 into account and use 606MHz as the upper edge of the interference.

Observation 4: A TV receiver receiving the 6MHz channel 36 from 602-608MHz will most likely suffer from significant interference from the BS at the lower edge of the n105 band and may not be used for TV broadcasts.

Proposal 5: RAN4 should specify the in-band blocker center frequency for range 3 blocking for n105 to be 603MHz, using the 8MHz channel 37 up to 606MHz as the closest TV signal frequency.

# 3	Conclusion

In this contribution, we share our views on the issues and the potential solutions for implementing the new band for APT 600MHz.

Observation 1: n71 is already used in the APT region, not only in the US, so a device used in the APT region needs to be able to support n105 and n71.

Observation 2: The performance of a n105 duplexer is much more difficult to achieve, as it is not a dual duplexer solution as specified for n71 but a single duplexer with a much larger bandwidth/duplex gap ratio and it has to fulfill a the same blocking requirement 5 MHz closer to the passband.

Observation 3: The TV stations in the APT region have 200kW compared to 1MW in the US. The blocker level can be relaxed accordingly.

Observation 4: A TV receiver receiving the 6MHz channel 36 from 602-608MHz will most likely suffer from significant interference from the BS at the lower edge of the n105 band and may not be used for TV broadcasts.

Proposal 1: RAN4 should specify the RF performance parameters for n105 in such a way that the same hardware can support n105 and n71.

Proposal 2: RAN4 should specify the RF performance parameters for n105 taking into account the production tolerances, process spread as well as temperature drifts of duplexers supporting both, n105 and n71.

Proposal 3: RAN4 should specify the Refsens for n105 by re-using the Refsens values from n71 and adding +0.5dB for the frequency range 612-617MHz to account for the much tighter requirements on the duplexer compared to n71.

Proposal 4: RAN4 should specify the in-band blocker level range 3 for n105 by reducing the -15dBm from n71 to -22dBm for n105 due to the 7dB lower TV power levels within the APT region.

Proposal 5: RAN4 should specify the in-band blocker frequency for range 3 for n105 to be 603MHz, using the 8MHz channel 37 up to 606MHz as the closest TV signal frequency.

# 4	References

RP-221778 “New WID on APT 600 MHz NR band”; Spark NZ, Qualcomm Inc;  3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6th – 9th, 2022

3GPP TR 38.860, Study on extended 600MHz NR band

3GPP TS 38.101-1 V17.9.0 (2022-09)