3GPP TSG-RAN4 Meeting #104-bis-e	R4-2215670

Online, , October 10 – 19th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on power class fallback for shared spectrum bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic_enh. |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of power class fallback rule set is required for unlicenced spectrum access. For licensed spectrum the power class fallback rule set is defined in clause 6.2.1. It defines the applicability of power class requirements if a UE supports a different power class than the default UE power class. In case a UE supports a different power class than the default UE power class for the band and the supported power class enables higher maximum output power than that of the default power class then the requirements of a lower power class might be applicable in case the network signals a certain P-Max limit or configures an UL duty cycle above 50%. With the introduction of power class 3 for shared spectrum access the fallback approach of NR licenced should be followed. A dedicated rule set for shared spectrum access is introduced as the default power class is power class 5 while licensed access has power class 3 as default. Since power class 5 does not exist for licensed spectrum access the rule set in clause 6.2.1 cannot cover the needs of shared spectrum access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Power class fallback rule set is introduced for unlicenced spectrum access. Rule set considers power class 5 as default with power class 3 being the higher power class. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Power class fallback is not properly defined for unlicenced spectrum access and requirements remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.1, 6.2F.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

## 6.2F	Transmitter power for shared spectrum channel access

### 6.2F.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of shared spectrum channel access carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2F.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2F.1-2 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
| n102 | NS_58 | 20, 40, 60, 80 | 5945 – 6425 | 10 |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2F.4;

-	else shall apply all requirements for power class 3 to the supported power class and set the configured transmitted power as specified in clause 6.2F.4;

### 6.2F.1A	UE maximum output power for CA

<<< Unchanged parts are skipped >>>

### 6.2F.4	Configured transmitted power

The requirements for configured maximum output power in clause 6.2.4 apply. The definition of ΔPPowerClass is updated to the following:

ΔPPowerClass = 3 dB for a power class 3 capable UE when P-max of 20 dBm or lower is indicated (The exact evaluation period is no less than one radio frame).

6.2F.4D Configured transmitted power UL MIMO

<<< End of changed sections >>>