3GPP TSG-RAN WG4 Meeting #104bis-e 	R4-2215677

Electronic meeting, 10th – 19th October, 2022

Source:	Ericsson

Title:	Simulation results for NTN SAN PUSCH demodulation requirements

Agenda item:	4.2.7.2.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NTN NR SAN demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

## 2.1	Normal PUSCH requirement

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Antenna Configuration |  | 1Tx1Rx and 1Tx2Rx |
| Channel model |  | NTN-TDLA100-200 NTN-TDLC5-200 |
| MCS |  | MCS4 |
| Transform precoding |  | Disabled and enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15kHz 5MHz 30kHz 10MHz |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Table 2.1-2 Simulation results for normal PUSCH requirements, precoding disabled, 15kHz SCS and 5MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | 0.8 |
|  |  |  |  |  | B | 1 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -0.6 |
|  |  |  |  |  | B | -0.6 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -3.2 |
|  |  |  |  |  | B | -3 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -4 |
|  |  |  |  |  | B | -4 |

Table 2.1-3 Simulation results for normal PUSCH requirements, precoding disabled, 30kHz SCS and 10MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | 0.8 |
|  |  |  |  |  | B | 0.8 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -0.8 |
|  |  |  |  |  | B | -0.8 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -3.2 |
|  |  |  |  |  | B | -3.2 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -4 |
|  |  |  |  |  | B | -4 |

Table 2.1-4 Simulation results for normal PUSCH requirements, precoding enabled, 15kHz SCS and 5MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | 1.2 |
|  |  |  |  |  | B | 1.2 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -0.6 |
|  |  |  |  |  | B | -0.6 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -3 |
|  |  |  |  |  | B | -3 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -4 |
|  |  |  |  |  | B | -4 |

Table 2.1-5 Simulation results for normal PUSCH requirements, precoding enabled, 30kHz SCS and 10MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | 1.2 |
|  |  |  |  |  | B | 1.2 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -0.8 |
|  |  |  |  |  | B | -0.8 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -3 |
|  |  |  |  |  | B | -3 |
|  |  |  | NTN-TDLC5-200 | 70% | A | -4 |
|  |  |  |  |  | B | -4 |

## 2.2 UL TA requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | CSRS = 5, BSRS =0, for 20 RB |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| NOTE 1. The transmission of SRS is optional. The transmission comb is configured as KTC = 2. The SRS periodic is configured as TSRS = 10 for 15kHz SCS and 20 for 30kHz SCS respectively. |  |  |

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA100-200 Low |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS |

Table 2.1-2 Simulation results for UL TA requirements, 15kHz SCS and 5MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | 1.44 |
|  |  |  |  |  | B | 1.48 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -2.83 |
|  |  |  |  |  | B | -2.85 |

Table 2.1-3 Simulation results for UL TA requirements, 30kHz SCS and 10MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | 1.31 |
|  |  |  |  |  | B | 1.29 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -2.84 |
|  |  |  |  |  | B | -2.84 |

## 2.3 PUSCH repetition type A requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Table 2.1-2 Simulation results for PUSCH repetition type A requirements, 15kHz SCS and 5MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | -7.5 |
|  |  |  |  |  | B | -7.5 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -10.8 |
|  |  |  |  |  | B | -10.8 |

Table 2.1-3 Simulation results for PUSCH repetition type A requirements, 30kHz SCS and 10MHz CBW

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 | 70% | A | -7.2 |
|  |  |  |  |  | B | -7.2 |
| 1 | 2 | Normal | NTN-TDLA100-200 | 70% | A | -10.5 |
|  |  |  |  |  | B | -10.5 |

# Conclusions

Simulation results for NR SAN PUSCH demodulation requirements are delivered.

# References

[1]	R4-2214387, WF for NTN NR SAN demodulation, Huawei HI Silicon, RAN4#104-e

# Annex A Channel model for NTN

Table A-1 NTN-TDL-A100 delay profile (resolution 5ns)

| Tap # | Tap delay [ns] | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.675 | Rayleigh |
| 3 | 285 | -6.482 | Rayleigh |

Table A-2 NTN-TDLC5 delay profile (resolution 5ns)

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -8.668 | Rayleigh |
| 2 | 60 | -21.423 | Rayleigh |
| NOTE: The first tap follows a Rician distribution with a K-factor of K1 = 8.05 dB and a mean power of 0dB. |  |  |  |