3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215759

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item: 6.8.3.1

Source: MediaTek Inc.

Title: Discussion on simultaneous DL reception from different directions for general issues

Document for: Discussion

- Introduction

In the last meeting, RAN4 has a preliminary discussion for FR2 multi-Rx and some open issues are identified in WF [1]. Besides, in this meeting, this WI is divided into four agenda items to be discussed : (1) general issues, (2) L3 measurement, (3) L1 measurement and (4) TCI state switching. Our view on first agenda item “general issues” is provided in this paper.

- Discussion

In the following sections, below issues will be discussed sequentially.

- Basic assumption

- Scenario

- Feasibility of enhancement for L3 measurement

- Requirement applicability

- UE capability

2.1 Basic assumption

In this section, our view on following sub-issues are provided : (1) receive time difference, (2) UL transmission, (3) measurement accuracy, (4) received signal power difference between two panels and (5) detectable condition

2.1.1 Receive time difference

To our understanding, the key point of whether UE can measure the signals with timing difference larger than on CP is not only related to the number of the UE Rx panels. It also involves the number of FFT and timing estimation/tracking/compensation modules in the baseband. If UE wants to measure the signals by different panels with timing difference larger than one CP, then the number of modules mentioned above should be doubled.

Observation 1: For timing difference between different panels, the number of the FFT module, timing estimation/tracking/compensation will be doubled.

Proposal 1: The timing difference between different panels is at least within one CP. FFS whether to define requirements with timing difference larger than one CP.

2.1.2 UL transmission

According to WID [2] as below, only DL reception is captured. Therefore, UL transmission is out of scope.

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs)

Observation 2: Only DL reception is considered in the WID of R18 FR2 multi-Rx.

Proposal 2: Not to discuss UL transmission in R18 FR2 multi-Rx.

2.1.3 Measurement accuracy

In the last meeting, some companies suggest RAN4 can aim to reduce the delay requirement while maintaining the existing accuracy requirements.

From our point of view, even a UE which is capable of simultaneous 2 Rx panel is not mandated to turn on 2 panels all the time. UE may decide to switch off one of the panel. In that case, it is desirable to make the accuracy requirement unchanged, i.e., the accuracy requirement should be the same regardless of that single panel or multiple panels are active. Otherwise, the performance between single active panel and multiple active panels may be different. So, we are fine to define the requirement based on the same accuracy as legacy requirement first. However, whether to reduce the delay can be further discussed in each sub-topics.

Proposal 3: The same measurement accuracy as legacy requirement is reused for R18 FR2 multi-Rx. Whether the delay requirement can be reduced should be discussed in each sub-topics, e.g. L1 measurement or TCI state switch.

2.1.4 Received signal power difference between two panels

Since there are two UE panels to be activated at a time in this WI, some companies have concern on the received power difference between the two panels. For example, whether the signals can be received accurately if the received power difference on two panels is too larger. However, to our understanding, it is a RF issue and should be discussed in RF session.

Proposal 4: Leave the received signals power difference between two panels to RF session. No further discussions are needed in RRM session.

2.1.5 Detectable condition

As below, there is one open issue regarding detectable condition in the last meeting. Some companies think not only the target signals but also the associated signals in the QCL information should be detectable.

Issue 1-1-2-10: RS signals that shall remain detectable during measurement period - Proposals - Option 1: FFS In an improved RRM requirement relying on a certain QCL relationship (QCL type D), not only the target signal but also the associated signal in the QCL information shall remain detectable during the entire measurement period.

In general, we are fine with the proposal. Here, one more example is provided. Typically, tracking RS may be a source RS in one TCI state. Since the flied “repetition” cannot be configured for the tracking RS, UE can take the other RS (e.g. SSB) which is in the same QCL chain as the tracking RS to perform UE Rx beam selection. Therefore, it would be more reliable if all RSs in the same QCL chain remain detectable during the entire measurement period. Furthermore, as same QCL Type-D means that the 2 RS are transmitted from the same Tx spatial filter, we tend to believe that the detectability should be the same for all RS in the same QCL chain. So, the following proposal is suggested.

Proposal 5: For detectable condition, all RSs in the same QCL chain for the target TCI state should remain detectable during the entire measurement/switch period.

2.2 Scenario

In this section, two sub-issues (1) intra/inter - cell operation and (2) definition of single carrier in WID are discussed in section 2.2.1 and 2.2.2, respectively.

2.2.1 Intra/Inter-cell operation

Whether multiple UE Rx panels can be applied in both intra and inter cell operation is discussed without conclusion yet. To our understanding, the intention of multi-panel in this WI is for 4 MIMO layers data transmission. Comparing with intra-cell operation, inter-cell operation is not a typical case. Because, for inter-cell operation, UE will use two panels to perform 4 MIMO layers data transmission from two different serving cells at a time. In that case, UE is located in the middle of two cells which means UE is on the cell edge of both cells. It is high probable that both channels for two cells are not good. So, the UE may perform handover rather than 4 MIMO layer data transmission. Therefore, we do not see the necessity to introduce the multi-panel requirement for inter-cell operation.

Observation 3: For inter-cell operation, typically, UE is located in the middle of two cells which means the UE is on the cell edge of both cells. It is high probable that handover will be performed rather than 4 MIMO layer data transmission.

Proposal 6: Not to consider inter-cell operation in R18 multi-Rx UE.

2.2.2 Definition of single carrier in WID

In the WID [2], “single carrier” is explicitly pointed out as the scope of R18 multi-Rx. However, in the last meeting, companies have different understanding on the “single carrier” as below.

Issue 1-1-2-1: Single carrier for defining RRM requirements - Proposals - Option 1: RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier, by excluding downlink CA operation. - Option 2: UE can be configured with multiple carriers but multi-Rx chain is enabled on only one of the carriers.

For this issue, we tend to believe the meaning of “single carrier” in WID is closer to option 1 in the above issue according to our understanding. Furthermore, Option 1 is the starting point before getting to Option 2. No matter RAN4 wants to proceed on Option 2 or not, RAN4 should definitely work on Option 1 first.

Proposal 7: RRM requirement discussion shall start from the case with different QCL TypeD RSs on a single carrier, by excluding downlink CA operation.

2.3 Feasibility of enhancement for L3 measurement

The discussion of whether to enhance L3 measurement is triggered in the last meeting. For L3 measurement, it is a cell-level measurement and it is used for mobility. UE may regularly measure SSB on the frequency layer provided by RRC and perform handover procedure if the L3-RSRP measurement results of neighboring cell is better than the serving cell. However, to our understanding, there is no much gain if UE measures the SSB from neighboring more frequently with multi-panels. Fast L3 measurements are typically needed only when UE is moving faster. However, we do not see any difference in terms of UE mobility between UE with single activated panel and UE with multiple activated panels.

Proposal 8: Not to enhance L3 measurement requirement in R18 multi-panel WI.

2.4 Requirement applicability

In this section, our view on the following sub-issues are provided : (1) UE behaviour using a single panel, (2) simultaneous L3 and L1 measurement, (3) simultaneous reception of measured RS and data, (4) applicability of FR2-2 and (5) applicability of QCL Type

2.4.1 UE behaviour using a single panel

According to the WID [2], the number of active panels for a UE is two at a time. However, as Figure 1, because UE may move or rotate, it is possible that the best UE receiving panel for two different QCL-Type D RSs is the same.

Figure 1. Both different QCL-Type D RSs are received by same UE Rx panel at a time

In the last meeting, some companies provide the information that the discussion of applicability condition (e.g. AoA difference) is triggered in RF session. So, we are fine to define the requirement with some applicability conditions to avoid receiving two different QCL-Type D RSs by using one UE panel at a time.

Proposal 9: Define the requirement of multi-panel with some applicability conditions, e.g. the AoA difference between 2 signals is larger than the threshold which is to be concluded in RF session.

2.4.2 simultaneous L3 and L1 measurement

This issue is regarding whether UE can use different panels to simultaneously perform L3 and L1 measurement for a cell at a time. As our analysis in section 2.3, to our understanding, L3 enhancement is for mobility. However, the intention of this WI is to increase the MIMO layer up to 4 to improve data throughput. So, we tend to believe L3 measurement enhancement is not needed.

Besides, in order to achieve simultaneous L3 and L1 measurement, joint consideration between L3 and L1 measurement in UE implementation is needed. It will hugely increase UE complexity because UE needs to control different measurements on different panels at a time.

Proposal 10: In R18 multi-Rx, UE is not required to perform both L3 measurements and L1 measurements at a time.

2.4.3 simultaneous reception of measured RS and data

In this section, we would like to discuss whether the scheduling restriction is needed or not even though UE has two active panels at a time. In the legacy R15/R16 requirement, because only one active panel is assumed, it is simple to discuss the UE behaviour when measurement RS and data are collided in the same OFDM symbol. For the sake of handling the collision, scheduling restriction is defined in TS 38.133.

However, in R18 multi-Rx, two active panels at a time are allowed, i.e. one additional dimension “panel” is considered. It is a question mark whether the scheduling restriction can be directly removed from R18 multi-Rx. To our understanding, if network completely know when/how UE receives the PDSCH (e.g. which panel and which beam), the scheduling restriction can be removed from R18 multi-Rx. To clearly explain our point, we have an example below.

Assume panel #1 and panel #2 are active at a time and they are used for data reception and measurement. In this case, there are four beams on each panel. In addition, UE has already performed Rx beam selection for PDSCH before, and beam #3 on panel #1 is the best configuration to receive PDSCH. Now, network schedules the PDSCH in the same symbol as SSB. Under this assumption, UE measurement scheduling can be as Table 1. (Note: assume RS and PDSCH have same SCS.)

Table 1. one possible UE measurement scheduling

| Time (ms) | 0 |  | 20 |  | 40 |  | 60 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB occasion index | 0 |  | 1 |  | 2 |  | 3 |  |
| Panel index | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Beam index for L1 measurement | 1 |  | 2 |  | 3 |  | 4 |  |
| Whether UE can receive both SSB and PDSCH at a time | X | X | X | X | O | X | X | X |

In SSB occasion #2, scheduling restriction is not needed because both SSB and PDSCH can be received by the same beam on the same panel. However, to our understanding, which panel is used for measurement (i.e. UE beam sweeping) is up to UE implementation. In other words, network never knows which panel/beam will be used by UE to measure the next SSB occasion.

For example, as red index in Table 2, another one UE may have reverse beam index on panel #1 for beam sweeping. In this case, UE cannot receive the PDSCH anymore because UE applies beam #2 on panel #1 during SSB occasion #2. (Note: beam #3 on panel #1 is the best configuration to receive PDSCH).

Table 2. another possible UE measurement scheduling

| Time (ms) | 0 |  | 20 |  | 40 |  | 60 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB occasion index | 0 |  | 1 |  | 2 |  | 3 |  |
| Panel index | 1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Beam index for L1 measurement | 4 | 1 | 3 | 2 | 2 | 3 | 1 | 4 |
| Whether UE can receive both SSB and PDSCH at a time | X | X | X | X | X | X | X | X |

Observation 4: For a multiple panels UE, beam sweeping on SSB is still usually needed. How to use panel to perform beam sweeping in measurement is up to UE implementation and unknown to network.

Therefore, based on Observation 4 and above analysis, sometimes UE can use same beam and panel for reception of SSB and PDSCH on the same OFDM symbol while sometime UE cannot. But all in all, network never knows when UE can do so.

Besides, one possible question here is “if panel #1 cannot be used for PDSCH reception, could we use panel #2 to receive the PDSCH?” To our understanding, it cannot work because the channels on two panels are different. So, based on above analysis, we would like to trigger a discussion that “how to guarantee network can know when to have schedule restriction or when not to?” We tend to believe, even though UE has two active panels at a time, scheduling restriction is still needed if network cannot know when to have schedule restriction.

Proposal 11: RAN4 to further study how to guarantee that network can know when to apply schedule restriction or when not to.

2.4.4 applicability of FR2-2

One open issue regarding FR2-1 and FR2-2 is captured in WF [1] as below.

Issue 1-1-2-6: Applicability of new requirements for UE with the capability of simultaneous reception from different directions - Proposals - Option 1: A clarification is added in clause 3.6.13 of TS 38.133 (Applicability of requirements for FR2) that the new requirements defined in this WI are applicable only for FR2-1. - Option 2: New requirements defined in the multi-Rx chain WI are applicable for FR2-1 only.

For this issue, option 2 which means to add a clarification “applicable for FR2-1 only” in the new requirement is preferred. The reason is because in option 1 the original motivation to add a clarification in clause 3.6.13 is to avoid direct inheriting from FR2 requirements in previous releases. But now we are introducing a new requirement in later release. So, we think the situation is different. Based on this observation, the following proposal is suggested.

Proposal 12: To clarify the requirement in new section that R18 multi-Rx is applicable for FR2-1 only, i.e. no FR2-2.

2.4.5 applicability of QCL Type

In the last meeting, there is one open issue regarding QCL Type in WF [1] as below.

Issue 1-1-2-8: Applicability of new requirements to different QCL types - Proposals - Option 1: FFS RAN4 to discuss whether the new requirements will also be applicable when QCL type D is configured together with QCL type A and QCL type C

Some companies think QCL type D can also be configured together with QCL type A or together with QCL type C. Therefore, the requirement should be also applicable when two QCL types (A+D or C+D) are configured. To our understanding, similar logic seems already captured in existing requirement of TCI state switch. The corresponding evidence is provided as below.

### 8.10.3 MAC-CE based TCI state switch delay If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state - TSSB-proc = 2 ms; - TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

So, based on the above analysis, we are fine with the proposal. But, we are just wondering what is the explicit spec impact if the proposal is agreed?

2.5 UE capability

The discussion of UE capability simultaneousRxDataSSB-DiffNumerology is triggered in the last meeting. As far as we know, whether UE can support the simultaneousRxDataSSB-DiffNumerology has nothing to do with the number of panels. Because the mix numerology will be handled in the baseband. So, based on this observation, it seems the existing requirement can be reused.

Proposal 13: The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used in R18 multi-panel WI.

- Summary

In this paper, the discussion of FR2 multiple panels is provided. We have the following proposal:

Observation 1: For timing difference between different panels, the number of the FFT module, timing estimation/tracking/compensation will be doubled.

Proposal 1: The timing difference between different panels is at least within one CP. FFS whether to define requirements with timing difference larger than one CP.

Observation 2: Only DL reception is considered in the WID of R18 FR2 multi-Rx.

Proposal 2: Not to discuss UL transmission in R18 FR2 multi-Rx.

Proposal 3: The same measurement accuracy as legacy requirement is reused for R18 FR2 multi-Rx. Whether the delay requirement can be reduced should be discussed in each sub-topics, e.g. L1 measurement or TCI state switch.

Proposal 4: Leave the received signals power difference between two panels to RF session. No further discussions are needed in RRM session.

Proposal 5: For detectable condition, all RSs in the same QCL chain for the target TCI state should remain detectable during the entire measurement/switch period

Observation 3: For inter-cell operation, typically, UE is located in the middle of two cells which means the UE is on the cell edge of both cells. It is high probable that handover will be performed rather than 4 MIMO layer data transmission.

Proposal 6: Not to consider inter-cell operation in R18 multi-Rx UE.

Proposal 7: RRM requirement discussion shall start from the case with different QCL TypeD RSs on a single carrier, by excluding downlink CA operation.

Proposal 8: Not to enhance L3 measurement requirement in R18 multi-panel WI.

Proposal 9: Define the requirement of multi-panel with some applicability conditions, e.g. the AoA difference between 2 signals is larger than the threshold which is to be concluded in RF session.

Proposal 10: In R18 multi-Rx, UE is not required to perform both L3 measurements and L1 measurements at a time.

Observation 4: For a multiple panels UE, beam sweeping on SSB is still usually needed. How to use panel to perform beam sweeping in measurement is up to UE implementation and unknown to network

Proposal 11: RAN4 to further study how to guarantee that network can know when to apply schedule restriction or when not to.

Proposal 12: To clarify the requirement in new section that R18 multi-Rx is applicable for FR2-1 only, i.e. no FR2-2.

Proposal 13: The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used in R18 multi-panel WI.

- Reference

- R4-2214344, WF on FR2 multi-Rx RRM, vivo, RAN4 #104-e

- RP-221753, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Incorporated, RAN #96