3GPP TSG-RAN WG4 Meeting # 104-bis-e                              R4-2215794

# Electronic Meeting, Oct 10 – Oct 19, 2022

Title: 	Discussion on additionalSpectrumEmission for NS mapping and CRs associated with Canada and US band n77

Agenda Item:	9.4

Source: 	MediaTek Inc.

Document for:	Approval

Introduction

Regarding intra-band UL CA associated with Canada and US band n77, there is discussion about NS_55, NS_57, CA_NC_NS_01, and CA_NS_01 with additionalSpectrumEmission and signaling mapping in RAN-Plenary#97e[1][2] and RAN4#104-e [3]. In the contribution, the proposal for additionalSpectrumEmission and signaling mapping is provided.

Discussion

In RAN-Plenary#97-e meeting, regarding intra-band UL CA operation in the n77 frequency range in US, there is decision [1] about requirements for NCCA in n77 where NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_01 or NS_55 for another uplink component carrier in the range 3450-3550 MHz. On top of that, in TS 38.101-1 clause 6.2A.3.1.2, texts will be inserted to clarify that for UEs configured with NCCA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3700-3980 MHz and NS_01 or NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed additional spurious emission and A-MPR requirements are according to CA_NC_NS_01.

New added texts for TS 38.101-1 clause 6.2A.3.1.2: For UEs configured with intra-band non-contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3700-3980 MHz and NS_01 or NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NC_NS_01.

As for intra-band UL CA operation in the n77 frequency range in Canada, there is also decision [2] about requirements for CCA in n77 where NS_01 for an uplink component carrier in the range 3450-3650 MHz and NS_01 or NS_57 for another uplink component carrier in the range 3650-3980 MHz. On top of that, in TS 38.101-1 clause 6.2A.3.1.1, texts will be inserted to clarify that for UEs configured with CCA in n77.

New added texts for TS 38.101-1 clause 6.2A.3.1.1: For UEs configured with intra-band contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3450-3650 MHz and NS_01 or NS_57 for another uplink component carrier in the range 3650-3980 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NS_01.

The new added texts in TS 38.101-1 clauses 6.2A.3.1.1 and 6.2A.3.1.2 above also follow the agreed RAN2 CRs R2-2209139 and R2-2209137, respectively.

Observation 1: Regarding intra-band UL CA for band n77 in US or Canada, the operation of NS_55, NS_57, CA_NC_NS_01, and CA_NS_01 with additionalSpectrumEmission and signalling is clearly indicated in RP-222682, RP-222683, R2-2209137 and R2-2209139. Additionally, the RP-222682 and RP-222683 CRs’ new added texts also follow the agreed RAN2 CRs R2-2209137 and R2-2209139, respectively.

In RAN4#104e, another issue of NS mapping from RAN2 to RAN4 as indicated in [3] is shown below.

It can be seen that the RAN2 specification only refers back to Table 6.2.3.1-1A for 38.101-1 which contains the single carrier NS. The RAN2 specifications do NOT refer back to clause 6.2A.3.1.1 for CA_NS or clause 6.2A.3.1.2 for CA_NC_NS and further state that a common additionalSpectrumEmission is applicable for all uplink carriers in the NC or CCA configuration. Two possible options for interpretations are considered below. - Only the single carrier NS mapping from the PCC applies (cell specific NS signalled in SIB). The mapping to CA_NS and CA_NC_NS must conform to the NS mapping as defined in Table 6.2.3.1-1A. For example, assume that the basestation indicates additionalSpectrumEmission = 1 in Band n41. According to Table 6.2.3.1-1A, this maps to NS_04. When CA is configured, the same additionalSpectrumEmission = 1 applies and checking Table 6.2.3.1.1-2 for CCA, it can be seen that CA_NS_04 applies to the CA_n41 configuration. However, if instead additionalSpectrumEmission = 2 had been indicated, the single carrier mapping would be to NS_47 but there is no mapping for CA_NS and CA_NC_NS. The RAN4 specification should be corrected to add the mappings for CA_NS and CA_NC_NS corresponding to additionalSpectrumEmission = 2; for example, this might mean to define CA_NS_47 and CA_NC_NS_47. All future CA_NS and CA_NC_NS should ensure there is a 1:1 mapping according to the NS mapping. Alternative, since the mapping is 1:1, the CA_NS and CA_NC_NS designations are redundant and can be removed referring instead only to the NS (of course, there are different A-MPR tables for CCA and NCCA and potentially different emissions, but they can still be indexed by NS rather than CA_NS or CA_NC_NS). Option 1: The additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update needed. - It is the CA_NS or CA_NC_NS that applies when the SCC is configured and activated. It was the intention that a separate and distinct additionalSpectrumEmission is conveyed to the UE for CA (via dedicated signalling) when the SCC is configured and therefore RAN4 defined independent tables for CA_NS and CA_NC_NS. This SCC additionalSpectrumEmission is mapped according to Table 6.2A.3.1.1-2 (38.101 spec has an error and incorrectly refers to this table as 6.2.3.1.1-2 without the “A” suffix for CA) and Table 6.2A.3.1.2-2 for CCA and NC CA, respectively. RAN2 specification should be updated to point to clause 6.2A.3.1.1 for CA_NS or clause 6.2A.3.1.2 for CA_NC_NS. Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. RAN2 specification update needed. The RAN2 agreements (i.e., CRs of R2-2209139 and R2-2209137) can be accommodated by either option 1 or option 2 although the implementation may slightly differ.

Observation 2: Regarding NS mapping from RAN2 and RAN4, the RAN2 agreements (i.e., CRs of R2-2209137 and R2-2209139) can be accommodated by either option 1 or option 2 in [3] although the implementation may slightly differ.

Option 1: The additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update needed.

Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. RAN2 specification update needed.

Proposal 1: Based on the agreed intra-band UL CA CRs (i.e., RP-222682 and RP-222683) associated with Canada and US band n77, the RAN4 specification update and NS mapping in TS 38.101-1 is already applicable. As for NS (i.e., NS, CA_NS, and CA_NC_NS) mapping issue associated with additionalSpectrumEmission, it can be solved by either option 1 or option 2 from R4-2214409. As for additionalSpectrumEmission NS mapping on n77 or other bands, option according to RAN4 solution below is recommended as it is straightforward and updated directly in RAN4 specification if needed.

Option: For intra-band UL CA on band n77 or other bands, the additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update if needed.

Conclusion

In the contribution, regarding intra-band UL CA signalling and additionalSpectrumEmission associated with signaling mapping, the observations for band n77 in US or Canada are provided. The proposal about general signalling mapping solution for band n77 or other bands is also provided.

Observation 1: Regarding intra-band UL CA for band n77 in US or Canada, the operation of NS_55, NS_57, CA_NC_NS_01, and CA_NS_01 with additionalSpectrumEmission and signalling is clearly indicated in RP-222682, RP-222683, R2-2209137 and R2-2209139. Additionally, the RP-222682 and RP-222683 CRs’ new added texts also follow the agreed RAN2 CRs R2-2209137 and R2-2209139, respectively.

Observation 2: Regarding NS mapping from RAN2 and RAN4, the RAN2 agreements (i.e., CRs of R2-2209137 and R2-2209139) can be accommodated by either option 1 or option 2 in [3] although the implementation may slightly differ.

Option 1: The additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update needed.

Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS. RAN2 specification update needed.

Proposal 1: Based on the agreed intra-band UL CA CRs (i.e., RP-222682 and RP-222683) associated with Canada and US band n77, the RAN4 specification update and NS mapping in TS 38.101-1 is already applicable. As for NS (i.e., NS, CA_NS, and CA_NC_NS) mapping issue associated with additionalSpectrumEmission, it can be solved by either option 1 or option 2 from R4-2214409. As for additionalSpectrumEmission NS mapping on n77 or other bands, option according to RAN4 solution below is recommended as it is straightforward and updated directly in RAN4 specification if needed.

Option: For intra-band UL CA on band n77 or other bands, the additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS. RAN4 specification update if needed.

- References

[1] RP-222682, “Extension of operation in the n77 frequency range in US”, Nokia

[2] RP-222683, “Extension of operation in the n77 frequency range in Canada”, Nokia

[3] R4-2214409, “WF on clarification of the NS mapping from RAN2 to RAN4”, Qualcomm Inc.