3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215804

# Electronic Meeting, October 10 – 19, 2022

Source:	LG Electronics

Title:	Discussion on L3 measurement related RRM for simultaneous DL reception from different directions

Agenda item:	6.8.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on L3 measurement related RRM for simultaneous DL reception from different directions based on the agreed WF [1] in the last meeting.

Discussion

In the last RAN4 meeting, the feasibility of L3 measurement related RRM requirements in this WI was discussed.

Issue 1-1-4: Feasibility/necessity of enhancing requirements for L3 measurements

Based on the WID, L3 measurement delay enhancement is one of the objectives, and the starting point is the enhancements related to L1-RSRP measurement under the condition that if necessary.

From the UE behaviour perspective, after triggering simultaneous DL signal reception from multiple TRP, UE would activate multi-antenna panels if the AoA between two DL signals is larger than a certain level. So, for the L3 measurement period, multi-antenna panels are still activated, and UE does not need to perform only a single antenna panel for L3 measurement. When multi-antenna panels are activated, L3 measurement through multi-antenna panels is a natural UE behaviour. Therefore, L3 measurement enhancements should be considered in this WID.

- Proposal 1: L3 measurement enhancements should be considered since L3 measurement through multi-antenna panels is a natural UE behaviour when multi-antenna panels are activated.

In the last RAN4 meeting, the following cases for L3 measurement related requirements to be enhanced were listed.

- RRM measurement requirements for RRC-IDLE state and RRC_INACTIVE state.

- Handover to FR2-1 unknown cell

- FR2-1 unknown SCell activation

- FR2-1 unknown PSCell addition and release

- For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG).

If there are work-load issues, RAN4 can start L3 measurement enhancement in connected mode first.

- Proposal 2: Introduce the following case for L3 measurement enhancement first

  - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG).

Conclusion

In this contribution, we provide our views on L3 measurement related RRM for simultaneous DL reception from different directions, and we propose

- Proposal 1: L3 measurement enhancements should be considered since L3 measurement through multi-antenna panels is a natural UE behaviour when multi-antenna panels are activated.

- Proposal 2: Introduce the following case for L3 measurement enhancement first

  - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG).

Reference

[1] R4-2214344, “WF on FR2 multi-Rx RRM,” vivo.