3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215912

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.2

Source:	Ericsson

Title:	The remaining issues for PDCCH and PBCH requirements in FR2-2

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the remaining issues for PDCCH and PBCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #104-e, the propagation environments have been agreed to align with BS requirements and provided in the Appendix.

For PDCCH Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring - Agreement: RAN4 to follow the mandatory behaviour for multi-slot PDCCH scheduling and schedule PDCCH with (4,1) for 480kHz and (8,1) for 960kHz if agreed Issue 2-1-2: Set of PDCCH Test Cases Agreement: Define PDCCH requirements according to the table in WF [1] Issue 2-1-1: Whether to align propagation conditions with PDSCH - Agreement: Yes For PBCH Issue 2-2-1: SSB index assumption - Agreement: Only with not known SSB index Issue 2-2-2: Test setup for PBCH requirements - Define requirements for the tests below. Propagation condition to be further discussed; Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; [TDLA10-200]; New; unknown. 1-2; 480kHz/400MHz; [TDLA10-200]; New; unknown.

In this paper, we bring our view on the remaining issues related to PDCCH and PBCH demodulation.

# 2	Remaining issues for PDCCH requirements

As it has been stated in WF [1], the set of PDCCH test cases still need to be defined using the agreed propagation environments. Based on our simulation results in [2], we limit the test cases to the following Table in Proposal 1.

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA30-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA30-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA30-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA30-650 | 2x2 ULA Low | 1_0 | 40 | New |
| Test 1-5 | 3 | 2 | 60 | 2 | 16 | TDLD10-200 | 2x2 ULA Low | 1_0 | 40 | New |

# 3	Remaining issues for PBCH requirements

As it has been stated in WF [1], the set of PDCCH test cases still need to be defined using the agreed propagation environments. Based on our simulation results in [xx], we limit the test cases to the following Table in Proposal 2.

Proposal 2: Define PBCH demodulation requirements with the following test setup

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-650 | New | Unknown |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | New | Unknown |

# 4	Summary

In this paper, we tried to contribute with the following proposals for further discussions:

Proposal 1: Define PDCCH demodulation requirements for UE in FR2-2 with the following test setup.

| Test case | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNRBB (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | TDLA30-650 | 1x2 ULA Low | 1_0 | 40 | New |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | TDLA30-650 | 1x2 ULA Low | 1_1 | 56 | New |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | TDLA30-650 | 2x2 ULA Low | 1_1 | 56 | New |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | TDLA30-650 | 2x2 ULA Low | 1_0 | 40 | New |
| Test 1-5 | 3 | 2 | 60 | 2 | 16 | TDLD10-200 | 2x2 ULA Low | 1_0 | 40 | New |

Proposal 2: Define PBCH demodulation requirements with the following test setup

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-650 | New | Unknown |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | New | Unknown |

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- R4-2215913, “Simulation results for PDCCH and PBCH demodulation in FR2-2”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDCCH

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | TBD |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

## A.2	PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 Note 2: as specified in clause 11.1 of TS 38.213 |  |  |

See 38.101-4 Table 7.4.2.2-1

## A.3	Channel models

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |