3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215917

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.4

Source:	Ericsson

Title:	Simulation results for CSI reporting requirements in FR2-2

Document for:	Discussion

# 1	Introduction

In this paper and based on the progress summarized in WF [1], we present the simulation results on the CSI reporting which has been limited CQI reporting under static conditions in the frequency range 52.6 GHz to 71 GHz.

Issue 3-2-1: Scope of the FR2-2 CQI Requirements: - Agreement (GTW August 22, 2022): Only for SCS 120kHz, CBW=100MHz, further discuss test metric and test SNR points. Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements: - Option 1: Up to 16 QAM; - Option 2: Include 64QAM; Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz: - Agreement: For FR2-2 CQI requirements for 120kHz/100MHz, reuse test setup and test metrics from FR2-1; - Further discuss whether FR2-1 SNR test points can be reused - Option 1: Yes; - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1dB for QPSK;

# 2	Simulation Results for CQI Reporting under static conditions

We consider here

- Numerologies: 120 KHz/100 MHz (66 RBs), Rank 1.

- Tx EVM = 6%.

- Consider both PN model in TR 38.808 [2], at UE side only

- Use CPE compensation only

We will consider the following table throughout the simulations (same as in FR2-1)

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |  |  |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |  |  |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Available RE-s |  |  |  | 7590 | 7590 |  |  |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |  |  |  |  |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 |  |  |  |  |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |  |  |  |  |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |  |  |  |  |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |  |  |  |  |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |  |  |  |  |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |  |  |  |  |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |  |  |  |  |
| 8 | 1.9141 | 13 |  | 14343 | 28680 |  |  |  |  |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |  |  |  |  |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |  |  |  |  |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |  |  |  |  |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |  |  |  |  |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |  |  |  |  |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |  |  |  |  |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |  |  |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-1 of TS 38.214 |  |  |  |  |  |  |  |  |  |

It is worth reminding that:

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

## 2.1 Simulation results

Under PN Set 1 Effects

| SNR | Median CQI | Pr(Median CQI – 1 < CQI < Median CQI+1) [%] | BLER of Median CQI-1 [%] | BLER of Median CQI [%] | BLER of Median CQI+1 [%] |
| --- | --- | --- | --- | --- | --- |
| -5 | 3 | 100 | 0 | 0 | 100 |
| -4 | 4 | 99.7 | 0 | 1.2 | 100 |
| -3 | 4 | 99.7 | 0 | 0 | 100 |
| -2 | 5 | 99.7 | 0 | 0.1 | 100 |
| -1 | 5 | 100 | 0 | 0 | 100 |
| 0 | 6 | 99.7 | 0 | 0.7 | 100 |
| 1 | 6 | 100 | 0 | 0 | 100 |
| 2 | 7 | 100 | 0 | 1.4 | 100 |
| 3 | 7 | 100 | 0 | 0 | 100 |
| 4 | 8 | 100 | 0 | 2.0 | 100 |
| 5 | 8 | 100 | 0 | 0 | 72.7 |
| 6 | 9 | 100 | 0 | 0 | 100 |
| 7 | 9 | 100 | 0 | 0 | 72.7 |
| 8 | 10 | 100 | 0 | 0 | 100 |
| 9 | 10 | 100 | 0 | 0 | 90.1 |
| 10 | 11 | 100 | 0 | 0.4 | 100 |
| 11 | 11 | 100 | 0 | 0 | 94.9 |
| 12 | 12 | 100 | 0 | 28.3 | 100 |
| 13 | 12 | 100 | 0 | 0 | 100 |
| 14 | 13 | 100 | 0 | 78.0 | 100 |
| 15 | 13 | 100 | 0 | 12.6 | 100 |
| 16 | 13 | 100 | 0 | 0.4 | 100 |
| 17 | 13 | 100 | 0 | 0.1 | 99.7 |
| 18 | 14 | 100 | 0 | 72.0 | 100 |
| 19 | 14 | 100 | 0 | 30.5 | 100 |
| 20 | 14 | 100 | 0 | 10.6 | 100 |
| 21 | 14 | 100 | 0 | 4.2 | 100 |
| 22 | 15 | 99.7 | 1.8 | 99.9 | NaN |
| 23 | 15 | 100 | 1.3 | 98.5 | NaN |
| 24 | 15 | 100 | 0.7 | 93.6 | NaN |
| 25 | 15 | 100 | 0.7 | 87.7 | NaN |

Under PN Set 2 Effects

| SNR | Median CQI | Pr(Median CQI – 1 < CQI < Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -5 | 3 | 99.7 | 0 | 0 | 100 |
| -4 | 4 | 100 | 0 | 1.9 | 100 |
| -3 | 4 | 100 | 0 | 0 | 100 |
| -2 | 5 | 100 | 0 | 0.1 | 100 |
| -1 | 5 | 100 | 0 | 0 | 100 |
| 0 | 6 | 100 | 0 | 3.3 | 100 |
| 1 | 6 | 100 | 0 | 0 | 100 |
| 2 | 7 | 100 | 0 | 6.2 | 100 |
| 3 | 7 | 100 | 0 | 0 | 100 |
| 4 | 8 | 100 | 0 | 9.8 | 100 |
| 5 | 8 | 100 | 0 | 0 | 77.1 |
| 6 | 9 | 100 | 0 | 1.1 | 100 |
| 7 | 9 | 100 | 0 | 0 | 77.6 |
| 8 | 10 | 100 | 0 | 31.1 | 100 |
| 9 | 10 | 100 | 0 | 0.1 | 100 |
| 10 | 11 | 100 | 0 | 60.9 | 100 |
| 11 | 11 | 100 | 0 | 3.2 | 100 |
| 12 | 12 | 100 | 0 | 90.1 | 100 |
| 13 | 12 | 100 | 0 | 55.5 | 100 |
| 14 | 12 | 100 | 0 | 19.1 | 99.9 |
| 15 | 13 | 100 | 6.0 | 96.5 | 100 |
| 16 | 13 | 100 | 2.2 | 82.8 | 100 |
| 17 | 13 | 100 | 1.4 | 61.1 | 100 |
| 18 | 14 | 100 | 43.7 | 100 | 100 |
| 19 | 14 | 100 | 31.5 | 100 | 100 |
| 20 | 14 | 100 | 24.3 | 99.3 | 100 |
| 21 | 14 | 100 | 20.1 | 97.9 | 100 |
| 22 | 15 | 96.4 | 96.0 | 100 | NaN |
| 23 | 15 | 98.0 | 93.5 | 100 | NaN |
| 24 | 15 | 99.3 | 92.0 | 100 | NaN |
| 25 | 15 | 99.0 | 89.6 | 100 | NaN |

The median CQI as function of SNR under PN effects are depicted below.

(a) CQI reporting under PN Set 1                   (b) CQI reporting under PN Set 2

Fig. 1 Simulation results depicting the median CQI as function of SNR using 120 KHz/100 MHz (66 PRBs) under PN effects.

Observation 1: The results show that the lowest SNR to report efficiently a CQI related to

- 64QAM is at SNR = 8 dB (CQI 10).

- 16QAM is at SNR = 2 dB (CQI 7).

- QPSK is at SNR below -5dB.

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: The results show that the lowest SNR to report efficiently a CQI related to

- 64QAM is at SNR = 8 dB (CQI 10).

- 16QAM is at SNR = 2 dB (CQI 7).

- QPSK is at SNR below -5dB.

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

# Appendix	Simulation assumption

## A.1	CQI reporting in static condition

### A.1.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

| For FR2-2 Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |
| SNRBB |  |  | dB | TBD |
| Propagation channel |  |  |  | AWGN |
| Antenna configuration |  |  |  | 2x2 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |
| ReportConfigType |  |  |  | Periodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |
| aperiodicTriggeringOffset |  |  |  | Not configured |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Measurement channel |  |  |  | TBD |