3GPP TSG-WG RAN4 Meeting #104-bis-e	R4-2215932

# Online, 10th – 19th October, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 				LTE based 5G UE RF requirement

Agenda Item: 	7.4.2

Document for:	Approval

# Introduction

RAN4 work on LTE based 5G broadcast is now kicked off after the completion of part 1 work by RAN1/2/3 [1]. In this contribution, open issues on UE RF requirements are discussed according to the agreed WF [6].

# Discussion

Coexistence aspects, ACS and Blocking requirement for UE

The LTE based 5G broadcast feature is already specified for legacy bands and channel bandwidths; however, coexistence in HPHT deployment has not been studied in the part 1 work item w.r.t. the RF scenarios for the adjacent channel (of the same band) or the service (on the adjacent band) deployments.

For the UE coexistence with adjacent channel or service, there are two possible issues for HPHT deployment.

- UE ACS requirement for the coexistence of 5G broadcast BS and 5G broadcast UE in adjacent channels.

- UE blocking requirement for the coexistence of IMT BS and 5G broadcast UE in adjacent bands.

WID states that “New (dedicated) channel filters (e.g. 6/7/8 MHz filters) are not considered for the UE” and “Reuse existing requirements for 10 MHz as much as possible.”

This may be troublesome to meet the necessary coexistence requirement against adjacent channel and service, as the channel filter can overlap with interferers if 10 MHz filer is assumed.

Observation 1: 10 MHz channel filter may overlap with the adjacent channel or adjacent band interferes, which degrades UE ACS and blocking performance.

Another option, if a new channel filter is not used, is to reuse 5 MHz channel filter with UE specific channel bandwidth or BWP, similar to the overlapping channel bandwidth methods discussed for irregular channel bandwidth work item.

Observation 2: Reusing 5 MHz channel filter can be another option to meet adjacent channel selectivity and blocking requirement.

For the existing DTT receivers in EU standards, the ACS/blocking requirements are specified in EN 303 340 (Table 5 and 6 below). This may be used as a baseline to match the 5G broadcast receiver performance with the existing receivers, so that the coexistence scenario is not affected.

The minimum required I/C limit is 25 dB for adjacent channels (+/-8 MHz), 36 dB for 10 MHz, 38 dB for 18 MHz and 43 dB for 73 MHz.

ACS level 33 dB for E-UTRA bands and in-band blocking requirement in TS 36.101 do not look inconsistent with these requirements.

In the cellular systems, non-collocated deployment imposes stringent ACS requirement, which may not be the case for coordinated DTT deployment. However, for the conventional DTT, normal scenario is the rooftop antenna reception with Yagi array antenna. As concluded in TR 38.776, it would be challenging to provide service-level coverage for 95% of users for indoor and in-car receivers. This coverage issue for mobile reception may require a gap filler to provide more coverage for 5G broadcast. Then, non-collocated deployment (though not fully uncoordinated) may need to be further considered, which can require higher ACS.

Observation 3: It should be further checked if ACS/in-band blocking level can be kept for 6/7/8 MHz channels in TS 36.101, as more ACS demanding scenario may be needed if uncoordinated deployment is considered.

The blocking requirement is specified in Table 6, where -25 dBm interfere level is specified for wanted signal at REFSEN+6 dB. Similar scenario would be needed for 5G broadcast band to meet the existing receiver requirement. This is stricter than E-UTRA out-of-band blocking region 2, where -30 dBm is specified.

Observation 4: Further check what blocker level may be needed for out-of-band blocking.

UE REFSENS

There was a proposal in RAN4#104e to reuse band 71 sensitivity requirement for 5G terrestrial broadcast, however, the band 71 REFSENS is affected by duplexer insertion loss and desensitization due to uplink. The band for 5G terrestrial broadcast is standalone downlink only (SDO). If sub-band filters are assumed, the insertion loss can be minimized and a similar requirement as other downlink only bands (band 32 and 69, etc) would be feasible. It is proposed to investigate to minimize the insertion loss with possible sub-band filters if a single wideband filter requires a large insertion loss.

Observation 5: FDD Band 71 REFSENS is not a good example to reuse for SDO band.

Proposal 1: Investigate to minimize the insertion loss with possible sub-band filters if a single wideband filter requires a large insertion loss.

UE noise figure is assumed to be 6 dB for rooftop and car-mounted scenarios and 9 dB for in-car and indoor scenarios in SI evaluation TR 38.776. If different types of Receive-Only Mode (ROM) devices should be considered for 5G broadcast, it may be possible to specify more sensitive receiver for rooftop scenario and another for mobile receiver. Power classes (like PC1 with directive antenna and PC3 with non-directive one) cannot be used to differentiate UE type for this purpose, but it is possible to introduce something new for SDO band.

Observation 6: Two different noise figures (for rooftop/car-mounted and in-car/indoor) are assumed during the system evaluations.

Proposal 2: Agree what UE types are specified in this part 2 WI.

UE Maximum input level

The maximum input level for existing E-UTRA bands is specified for the RF scenario for macro cellular deployment. For HPHT, the maximum input level needs to be studies for the mobile reception.

This is not an issue for the rooftop reception, as an attenuator can be used to suppress the signal if the input level is high.

Observation 7: Maximum input level needs to be studied for the mobile reception of HPHT scenario.

# Conclusion

Observation 1: 10 MHz channel filter may overlap with the adjacent channel or adjacent band interferes, which degrades UE ACS and blocking performance.

Observation 2: Reusing 5 MHz channel filter can be another option to meet adjacent channel selectivity and blocking requirement.

Observation 3: It should be further checked if ACS/in-band blocking level can be kept for 6/7/8 MHz channels in TS 36.101, as more ACS demanding scenario may be needed if uncoordinated deployment is considered.

Observation 4: Further check what blocker level may be needed for out-of-band blocking.

Observation 5: FDD Band 71 REFSENS is not a good example to reuse for SDO band.

Proposal 1: Investigate to minimize the insertion loss with possible sub-band filters if a single wideband filter requires a large insertion loss.

Observation 6: Two different noise figures (for rooftop/car-mounted and in-car/indoor) are assumed during the system evaluations.

Proposal 2: Agree what UE types are specified in this part 2 WI.

Observation 7: Maximum input level needs to be studied for the mobile reception of HPHT scenario.

# Reference

[1] RP-211144 Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 1

[2] RP-211145 New WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2

[3] RP-220508 RAN2 CRs to New bands and bandwidth allocation for 5G terrestrial broadcast - part 1

[4] R4-2214441 WF on the scope of a coexistence study for HPHT 5G broadcast	Ericsson

[5] R4-2214442 WF on band definition for 5G broadcast in UHF	Nokia

[6] R4-2214443 WF on UE RF requirements for 5G broadcast	SWR

[7] R4-2214444 WF on system parameters for 5G broadcast	Qualcomm Incorporated

[8] R4-2214384 WF on List of expected changes to TS 36.104 due to introduction of LTE based 5G terrestrial broadcast band(s) Nokia

[9] R4-2214385 WF on Coexisting studies between IMT service around DTT spectrum and reuse of existing regulatory agreements	Huawei, ZTE