3GPP TSG-RAN WG4 # 104-bis-e	R4-2215938

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.5.3.1.1

Source:	MediaTek Inc.

Title:	Discussion on demodulation requirement for Enhancement on HST-SFN deployment

Document for:	Discussion

# Introduction

In RAN4#104-e, the detail for the simulation assumption for HST-SFN scheme A was agreed [1]. We provide discussion with simulation results in this contribution.

# Discussion

The following options in Issue 2-2-2 were agreed for further discussion in the previous meeting [1] and are discussed in this document. HST-SFN scheme B simulation results are discussed in chapter 2.1. HST-SFN scheme A related issues and simulation results are discussed in chapter 2.2.

## 2.1 HST-SFN scheme B simulation results

The simulation settings are based on the agreed assumptions from 104-e WF [1], 103-e WF [2] and 102-e WF [3] documents and shown in Table 1. HST-SFN Rel-17 Scheme B is simulated with agreed MCS options 13 and 17 and maximum doppler 870Hz for FDD and 1667Hz for TDD with number of receiver antenna 2 and 4 as suggested in 104-e WF [1]. Pre-compensation model was agreed to be ideal in 104-e WF [1]. Simulation results are listed in Table 2.

Table 1: Simulation settings HST-SFN Rel-17 Scheme B.

| Parameter | Value | Value |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| BW (MHz) / SCS (kHz) | 10 / 15 | 40 / 30 |
| Propagation Condition | HST-SFN Rel-17 | HST-SFN Rel-17 |
| Rank | 2 | 2 |
| MCS | 13 or 17 | 13 or 17 |
| Antenna configuration | 2x2, 2x4 | 2x2, 2x4 |
| Maximum doppler | 870Hz | 1667Hz |
| Number of DMRS symbols | 3 | 3 |
| PRB bundling size | 2 | 2 |
| CSI-RS periodicity | 10 | 10 |
| CSI-RS slot offsets | 1, 2 | 1, 2 |
| Pre-compensation model | Ideal | Ideal |

Table 2: Simulation results of HST-SFN Rel-17 Scheme B

| Duplex scheme | Antenna config | MCS | Maximum doppler | SNR [dB] |
| --- | --- | --- | --- | --- |
| FDD | 2x4 | 13 | 870Hz | 5.2 |
| FDD | 2x2 | 13 | 870Hz | 7.8 |
| FDD | 2x4 | 17 | 870Hz | 8.8 |
| FDD | 2x2 | 17 | 870Hz | 11.4 |
| TDD | 2x4 | 13 | 1667Hz | 5.8 |
| TDD | 2x2 | 13 | 1667Hz | 8.4 |
| TDD | 2x4 | 17 | 1667Hz | 9.6 |
| TDD | 2x2 | 17 | 1667Hz | 11.8 |

## 2.2 HST-SFN scheme A related issues and simulation results

The simulation settings are based on the agreed assumptions from 104-e WF [1], 103-e WF [2] and 102-e WF [3] documents and shown in Table 3. HST-SFN Rel-17 Scheme A is simulated with agreed MCS options 13 and 17 and maximum doppler 870Hz for FDD and 1667Hz for TDD with number of receiver antenna 2 and 4 as suggested in 104-e WF [1].

Table 3: Simulation settings HST-SFN Rel-17 Scheme A.

| Parameter | Value | Value |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| BW (MHz) / SCS (kHz) | 10 / 15 | 40 / 30 |
| Propagation Condition | HST-SFN Rel-17 | HST-SFN Rel-17 |
| Rank | 2 | 2 |
| MCS | 13 or 17 | 13 or 17 |
| Antenna configuration | 2x2, 2x4 | 2x2, 2x4 |
| Maximum doppler | 870Hz | 1667Hz |
| Number of DMRS symbols | 3 | 3 |
| PRB bundling size | 2 | 2 |
| CSI-RS periodicity | 10 | 10 |
| CSI-RS slot offsets | 1, 2 | 1, 2 |

The simulation results are shown in the following Figures 1 and 2, and 70% relative throughput numbers are collected in Table 4. Figure 1 shows results of FDD with maximum doppler 870Hz and Figure 2 shows results of TDD with maximum doppler 1667Hz. Both figures include both MCS options 13 and 17, and number of receiver antennas 2 and 4.

Figure 1: HST-SFN Rel-17, FDD, BW 10 MHz, SCS 15kHz, max doppler 870Hz simulation results

Figure 2: HST-SFN Rel-17, TDD, BW 40 MHz, SCS 30 kHz, max doppler 1667 Hz simulation results

Table 4 collects required SNR for relative throughput of 70%. We can make observation from Figure 1 and Figure 2.

Observation #1: UE has problems to achieve relative throughput of higher than 90% with Rank2 MCS17.

Table 4: Simulation results of HST-SFN Rel-17 Scheme A

| Duplex scheme | Antenna config | MCS | Maximum doppler | SNR [dB] |
| --- | --- | --- | --- | --- |
| FDD | 2x4 | 13 | 870Hz | 6.4 |
| FDD | 2x2 | 13 | 870Hz | 8.5 |
| FDD | 2x4 | 17 | 870Hz | 13.3 |
| FDD | 2x2 | 17 | 870Hz | 17.1 |
| TDD | 2x4 | 13 | 1667Hz | 8.6 |
| TDD | 2x2 | 13 | 1667Hz | 11.0 |
| TDD | 2x4 | 17 | 1667Hz | 15.0 |
| TDD | 2x2 | 17 | 1667Hz | 19.3 |

Next here are discussions and proposals for HST-SFN scheme A related Issue 2-2-2.

Issue 2-2-2: MCS & Rank

- Further discuss and decide in next Ran4 meeting with following options

    - MCS 13 with Rank 2

    - MCS 17 with Rank 2

    - It’s encouraged companies can provide results for both options under FDD and TDD modes.

    - RAN4 will decide based on the alignment outcome among companies’ results and whether peak TP can be achieved or not.

As seen in Observation #1 UE has problems to achieve relative throughput higher than 90% with Rank2 corresponding to 10% BLER. This is indication that higher MCS17 is in the limits of reasonable performance. Also, in the practical network where OLLA would be targeting 10% BLER actual MCS would be lowered due to too high BLER. Also, in previous meeting 104-e WF [1] it was agreed to check if peak throughput can be achieved or not. Therefore, we are proposing to define test case with adequate Rank2 and MCS 13.

Proposal #1: We support MCS 13 and Rank 2.

# Conclusion

In this paper we provided the view on the requirements definition for HST-SFN Rel-17 enhancements. The following observations and proposals are made:

Observation #1: UE has problems to achieve relative throughput of higher than 90% with Rank2 MCS17.

Proposal #1: We support MCS 13 and Rank 2.

# Reference

- R4-2214397, “WF on UE demodulation and CSI requirement for FeMIMO”, Samsung

- R4-2210670, “WF on demodulation requirement for Enhancement on HST-SFN deployment”, Ericsson

- R4-2207208, “WF on demodulation requirement for Enhancement on HST-SFN deployment”, Ericsson