3GPP TSG-RAN WG4 # 104-bis-e	R4-2215939

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.5.3.1.2

Source:	MediaTek Inc.

Title:	Discussion on demodulation performance requirements definition for Rel17 multi-TRP

Document for:	Discussion

# Introduction

In RAN4#104-e, the detail for the simulation assumption for multi-TRP PDCCH with FDM repetition scheme was agreed [1]. We provide discussion with simulation results in this contribution.

# Discussion

The following options in Issue 1-1-1 was agreed for further discussion in the previous meeting [1] and is discussed in this document.

Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission

Agreements:

- Specify requirements without specific receiver assumption, additional margin will be included:

  - Additional margin

    - Option 1: [0.8] dB

    - Option 2: [1] dB

    - Other options not precluded

There exist several UE implementation assumptions that may or may not include UE soft-combining. It is known that performance with UE soft-combining is better than without UE soft-combining. If requirements would be defined based on UE soft-combining assumption that may be a problem to UEs not implementing soft-combining. Therefore, we are proposing to define requirements with high enough additional margin to support all UE implementation assumptions. The exact margin can be determined from simulation result collection from all companies.

Proposal #1: We support to define PDCCH requirement for multi-TRP repetition transmission scheme with high enough additional margin to support all UE implementation assumptions.

The simulation settings are based on the agreed assumptions from 104-e [1], 103-e WF [2] and 102-e WF [3] documents and shown in Table 1. Multi-transmission point (TRP) PDCCH repetition is simulated with agreed Aggregation Level (AL) 2 with number of receiver antenna 2 and 4 as suggested in 104-e WF [1].

Table 1: Simulation settings.

| Parameter | Value | Value |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| Repetition transmission schemes | FDM | FDM |
| CBW | 10 MHz | 40 MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 | 2 |
| Aggregation level for repetition | 2 | 2 |
| Propagation Condition | TDLA30-10 | TDLA30-10 |
| Timing offset | -0.5us | -0.25us |
| Frequency offset | 200Hz | 300Hz |
| Antenna configuration | 2x2, 2x4 | 2x2, 2x4 |
| CCE to REG mapping type | nonInterleaved | nonInterleaved |
| REG bundle size | 6 | 6 |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) | 1% of Pm-dsg (%) |

The first set of simulation results are collected in Table 2 where SNR setting for each TRP uses Balanced SNR. This means the SNRs for TRP #1 and TRP #2 are assumed to be balanced without additional scaling factor for the transmitted signal from each TRP as agreed in 104-e WF [1].

Table 2: Simulation results of SNR setting for each TRP using Option 1 Balanced SNR

| Duplex scheme | Antenna config | Aggregation level | SNR [dB] with UE soft-combining Ideal | SNR [dB] without UE soft-combining Ideal | SNR [dB] with UE soft-combining Impairment | SNR [dB] without UE soft-combining Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 2x2 | 2 | 2.41 | 4.03 | 4.41 | 6.03 |
| FDD | 2x4 | 2 | -1.50 | 0.07 | 0.50 | 2.07 |
| TDD | 2x2 | 2 | 2.57 | 4.15 | 4.57 | 6.15 |
| TDD | 2x4 | 2 | -1.40 | 0.11 | 0.60 | 2.11 |

# Conclusion

In this paper we provided the view on the requirements definition for multi-TRP Rel-17 enhancements. The following proposal is made:

Proposal #1: We support to define PDCCH requirement for multi-TRP repetition transmission scheme with high enough additional margin to support all UE implementation assumptions.

# Reference

- R4-2214397, “WF on UE demodulation and CSI requirement for FeMIMO”, Samsung

- R4-2210671, “WF on demodulation requirement for Enhancement on Multi-TRP”, Huawei, HiSilicon

- R4-2207207, “WF on demodulation requirement for Enhancement on Multi-TRP”, Huawei, HiSilicon