3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215981

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Discussion on satellite NTN demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	4.2.7.2.1

Document for:	Discussion

Background

During RAN#104-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we share our views about NTN SAN PUSCH demodulation requirements.

Discussion

PUSCH

Test applicability rule for different antenna configuration

Test applicability rule for different antenna configuration Candidate options: - Option 1: Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. - Option 2: Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy) by same polarization type, the tests with low MIMO correlation level shall apply only for either one connector or the second lowest number of supported connecters, in addition to the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration - Option 3: Only 1Rx should be considered for SAN.

The only remained issue for NTN SAN PUSCH demodulation requirements is the test applicability rule for different antenna configuration. Firstly, based on the agreements achieved in RAN4#103-e, both 1Rx and 2Rx shall be considered for NTN SAN PUSCH demodulation requirements. We think this agreement should be kept since we don’t see any technology concern for blocking defining either antenna configuration above.

Keep previous agreement that both 1Rx and 2Rx shall be considered for NTN SAN PUSCH demodulation requirements.

For the test applicability rule between 1Rx and 2Rx cases, Option 1 is proposed to reduce the test effort since there is similar UE processing for 1Rx and 2Rx from the demodulation point of view. Option 2 is proposed to reuse the method from Rel-16 HST WI.

From our understanding, the UE processing on demodulation under NTN scenario can be verified by either 1Rx cases or 2Rx cases. Also considering that there is no impact on demodulation for the polarization type, we propose to use following test applicability rule: (Option 1)

Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Use following test applicability rule: (Option 1)

- Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Proposals

In this contribution, we discuss on SAN PUSCH demodulation requirements for NTN. Our observations and proposals are:

- Keep previous agreement that both 1Rx and 2Rx shall be considered for NTN SAN PUSCH demodulation requirements.

Use following test applicability rule: (Option 1)

- Unless otherwise stated, for a SAN supporting different numbers of antenna connectors (for SAN type 1-C) or TAB connectors (for SAN type 1-H) (see D.xxx in table yyy), the tests with low MIMO correlation level shall apply only for the highest number of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

Reference

R4-2214387, WF on NTN SAN demodulation requirements, RAN4#104-e, Huawei, HiSilicon