3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215992

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Discussion on UE NTN demod PDSCH

Source: 	Huawei, HiSilicon

Agenda item:	4.2.7.3.1

Document for:	Discussion

Background

During RAN#104-e meeting, WF [1] for NTN demodulation requirements – general and PDSCH was approved. In this contribution, we share our views about NTN UE PDSCH demodulation requirements.

Discussion

  - Modulation order

- Proposals: to confirm the following tentative agreements: - Option 1: Do not specify PDSCH for performance requirements for 64QAM - Option 2: Specify PDSCH for performance requirements for 64QAM - Agreement: - FFS whether specify PDSCH requirements for 64QAM in Rel-17, operators’ feedback on the demand are encouraged.

In RAN4#101-e meeting, agreement was achieved from the RF side that

- Include 64QAM as optional with manufacture declaration basis for SAN.

- Include 64QAM (DL and UL ) for NTN satellite UE as optional feature with granularity [per UE]

From our understanding, 64QAM is an important feature to provide higher throughput data transmission. Based on the link budget evaluation as following Table 2.1-1 and Table 2.1-2, 64QAM is feasible at least for rank1 and MCS17. Also considering RF side has introduced 64QAM feature, we propose to define PDSCH performance requirements for 64QAM.

Table 2.1-1 Link budget analysis for NTN PDSCH

|  | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban | LEO600-rural | LEO600-urban | LEO1200-rural | LEO1200-urban | GEO-rural | GEO-urban |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sattellite Max Tx power (dBm) based on TR38.863 | 46.81 | 46.81 | 52.81 | 52.81 | 50.81 | 50.81 | 46.64 | 46.64 | 52.64 | 52.64 | 50.64 | 50.64 |
| Coupling loss 50%-tile based on R4-2115628 (dB) | 125.65 | 125.6 | 131.48 | 131.43 | 140.08 | 139.98 | 125.65 | 125.6 | 131.48 | 131.43 | 140.08 | 139.98 |
| Received power level at UE antenna connector (dBm) | -78.84 | -78.79 | -78.67 | -78.62 | -89.27 | -89.17 | -79.01 | -78.96 | -78.84 | -78.79 | -89.44 | -89.34 |
| Noise figure for NTN UE (dB) | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| DL RB allocation (RB) | 106 | 106 | 106 | 106 | 106 | 106 | 51 | 51 | 51 | 51 | 51 | 51 |
| SCS (kHz) | 15 | 15 | 15 | 15 | 15 | 15 | 30 | 30 | 30 | 30 | 30 | 30 |
| 10log10 (kT) for dBm/Hz | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| Thermal Noise | -92.19 | -92.19 | -92.19 | -92.19 | -92.19 | -92.19 | -92.36 | -92.36 | -92.36 | -92.36 | -92.36 | -92.36 |
| SNR | 13.35 | 13.40 | 13.52 | 13.57 | 2.92 | 3.02 | 13.35 | 13.40 | 13.52 | 13.57 | 2.92 | 3.02 |

Table 2.1-2 Link level simulation results for NTN PDSCH (Rank1, MCS17)

| Case Number | SCS (kHz) / CBW (MHz) | Propagation condition | Antenna configuration | SNR (dB) @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Disabled HARQ | HARQ process 32 |
| 1 | 15 / 5 | NTN-TDLA100-200 | 2x2 | 10.59 | 10.08 |
| 2 | 15 / 5 | NTN-TDLC100-200 | 2x2 | 11.05 | 10.34 |
| 3 | 30 / 10 | NTN-TDLA100-200 | 2x2 | 8.85 | 8.30 |
| 4 | 30 / 10 | NTN-TDLC100-200 | 2x2 | 9.18 | 8.47 |

64QAM is feasible at least for rank1 and MCS17 with higher throughput based on the link budget analysed by RAN4 RF and us as shown in Table 2.1-1.

Define PDSCH performance requirements for 64QAM.

  - SCS/CBW set

- Proposals: to confirm the following tentative agreements: - Option 1: Do not specify requirements with additional 30KHz SCS for NTN. - Option 2: Specify requirements with additional 30KHz SCS for NTN

As per RF agreements, two new NTN satellite bands n256/n255 is introduced with {5, 10, 15, 20}MHz bandwidth configuration for 15kHz SCS and {10, 15, 20}MHz bandwidth configuration for 30kHz/60kHz SCS. For the normal UE performance requirements, only 10MHz and 20MHz bandwidth configuration case is defined for 15kHz SCS and 30kHz SCS respectively. For NTN UE performance requirements definition, we propose to use same typical bandwidth and SCS configuration as normal TN UE performance requirements, i.e. 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS. Also considering that 30kHz SCS has been introduced in satellite network performance requirements, it is reasonable to define 30kHz SCS performance requirements for NTN UE to keep consistence between NTN UE and SAN..

For NTN UE performance requirements, select both 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS.

Simulation assumption for PDSCH performance with disabled HARQ process feedback

In last meeting, the simulation assumption for PDSCH performance with disabled HARQ is agreed as following Table 2.3-1.

Table 2.3-1 Simulation assumption for PDSCH performance requirements with disabled HARQ Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. CSI-RS for tracking; CSI-RS periodicity; Slots; 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 [30 kHz SCS: 40 for CSI-RS resource 1,2,3,4]. CSI-RS offset; Slots; 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 [30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4]. Number of HARQ Processes; Disabled. The number of slots between PDSCH and corresponding HARQ-ACK information; N/A. Maximum number of HARQ transmission; 1.

Currently, it is not clear that the number of HARQ process is disabled [1]. Based on RAN1 design, it should be HARQ process feedback disabled rather than HARQ process disabled. From our understanding, the number of HARQ processes is still needed to be specified. So we propose to use the clause title in the specification as “Minimum requirements for PDSCH with disabled HARQ process feedback” and update the simulation assumption for PDSCH performance with disabled HARQ process feedback as following:

| Number of HARQ processes |  | 16 |
| --- | --- | --- |
| HARQ process feedback |  | Disabled for all HARQ processes |

Use the clause title in the specification as “Minimum requirements for PDSCH with disabled HARQ process feedback”.

Update the simulation assumption for PDSCH performance with disabled HARQ process feedback as following:

| Number of HARQ processes |  | 16 |
| --- | --- | --- |
| HARQ process feedback |  | Disabled for all HARQ processes |

Proposals

In this contribution, we discuss on UE PDSCH demodulation requirements for NTN. Our observations and proposals are:

- 64QAM is feasible at least for rank1 and MCS17 with higher throughput based on the link budget analysed by RAN4 RF and us as shown in Table 2.1-1.

- Define PDSCH performance requirements for 64QAM.

For NTN UE performance requirements, select both 10MHz bandwidth for 15kHz SCS and 20MHz bandwidth for 30kHz SCS.

Use the clause title in the specification as “Minimum requirements for PDSCH with disabled HARQ process feedback”.

Update the simulation assumption for PDSCH performance with disabled HARQ process feedback as following:

| Number of HARQ processes |  | 16 |
| --- | --- | --- |
| HARQ process feedback |  | Disabled for all HARQ processes |

Reference

R4-2214386, WF for NTN demodulation requirements – general and PDSCH, RAN4#104-e, Qualcomm Incorporated