3GPP TSG-RAN WG4 Meeting # 104bis-e									  R4-2216039

Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion on feasibility study for CA_n5-n8

Source: 	Xiaomi

Agenda item:	6.2.2.1

Document for:	Approval

- Introduction

In RAN4#104-e meeting, the feasibility of two low band CA including CA_n5-n8 was discussed but still need further study. This contribution will further provide our view on CA_n5-n8.

- Discussion

In the last meeting, the agreements and way forward on CA_n5-n8 are given in [1] as below:

### 2.2.1 Spectrum restriction assumption for the analysis Agreement: - UE should support the full range of spectrum for single band operations on both n5 and n8 - Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8. UL; DL. Frequency 1 (800MHz); 824MHz - 835MHz; 869MHz - 880MHz. Frequency 2 (900MHz); 904MHz - 915MHz; 949MHz - 960MHz. ### 2.2.2 UE RF architecture assumption The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8. 2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands. ### 2.2.3 CBW assumption The following is the assumption of BCS0: n5: {5,10}MHz, n8{5, 10}MHz ### 2.2.4 RF parameters assumption Continue the RF parameters discussion when the UE RF architecture and CBW are agreed. ### 2.2.5 Feasibility issues need to be analysed The following issues will be analyzed in the feasibility study. - The MSD due to IMD and cross band isolation. - The impact on n5 out-of-band blocking requirement. ### 2.2.6 Other observations and proposals The following RF requirement framework applies to CA_n5-n8, CA_n5-n28 and CA_n8-n20-28. - Operating bands for CA - Channel bandwidths per operating band for CA - UE co-existence requirement - ∆TIB and ∆RIB values - REFSENS requirements (MSD) - Maximum output power ### 2.2.7 UL configuration The CBW of UL configuration CA_n5-n8 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

The spectrum restriction is introduced based on operators’ request to solve frequency overlapping between n5 and n8.

|  | UL | DL |
| --- | --- | --- |
| Band n5 | 824MHz - 835MHz | 869MHz - 880MHz |
| Band n8 | 904MHz - 915MHz | 949MHz - 960MHz |

With the spectrum restriction, there is no IMD issue for CA_n5-n8 with two uplink transmission. So no MSD due to IMD should be introduced.

Observation 1: With the spectrum restriction there is no IMD issue for CA_n5-n8 with two uplink transmission.

For CA_n5-n8, two candidate RF architectures are agreed for further study. The UE RF architectures can be

- 3 antenna architecture (2 Tx/Rx main antenna, 1 Rx diversity antenna)

- 2 antenna architecture (1 Tx/Rx main antenna, 1 Rx diversity antenna)

Table 1: UE RF architecture for CA_n5A-n8A

| CA configuration | NR bands | 2 antenna |  | 3 antenna |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Tx/Rx capable LB Ant. 1 | Rx Diversity Ant. 2 | Tx/Rx Capable LB Ant. 1 | Tx/Rx Capable LB Ant. 2 | Rx Diversity Ant. 3 |
| CA_n5A-n8A | n5 | Quadplexer | n5 Rx + n8 Rx Duplexer | Duplexer |  | n5 Rx + n8 Rx Duplexer |
|  | n8 |  |  |  | Duplexer |  |

Figure 1: 2 antenna architecture for CA_n5A-n8A

Figure 2: 3 antenna architecture for CA_n5A-n8A

Based on previous agreements, RAN4 need to study such two RF architectures, i.e. 2 antenna and 3 antenna. The 2 antenna architecture and 3 antenna are illustrated in figure 1 and figure 2, respectively. It is proposed to use figure 1 and figure 2 as baseline for further study.

Proposal 1: It is proposed to use the RF front end architecture in figure 1 (2 antenna) and figure 2 (3 antenna) as baseline for further study.

For 2 antenna architecture, the number of antenna is reduced as much as possible by using a quadplexer so that the space issue is alleviated for smartphone. But a relatively large challenge would be caused for low band wideband antenna because a low band antenna is expected to cover a wide bandwidth across two bands.

For 3 antenna architecture, it is a feasible solution providing that 3 antenna architecture can be accommodated in a smartphone. Separate antenna architecture enables a good isolation but the trade-off between isolation and space should be evaluated. Based on the commercial smartphone design, 3 antenna architecture is able to be accommodated in a smartphone so the space of smartphone is not a problem. Currently 3 antenna architecture is much more preferred in practice.

Observation 2: 3 antenna architecture is able to be accommodated in a smartphone so the space of smartphone is not a problem based on the commercial smartphone design. Currently 3 antenna architecture is much more preferred in real implementation for CA_n5A-n8A.

Given that both 2 antenna and 3 antenna architectures are considered for further study, it is expected that two different sets of RF requirements will be obtained. In general, UE RF requirement specified in the specification is agnostic to UE RF architecture and specification does not restrict the UE implementation. As usual, RAN4 should define one set of RF requirements but it is still unclear which set of RF requirements will be considered. So it should be clarified which set of RF requirement will be used or the worst case can be used. In our understanding, it would be appropriate to use the worst case to define the minimum requirements by considering all UE RF architectures.

Proposal 2: For CA_n5A-n8A, RAN4 should clarify which set of RF requirement, e.g., worst case, will be used to define minimum requirements by considering all UE RF architecture.

- Conclusion

This contribution gives our views on the feasibility analysis for CA_n5A-n8A. The following observations and proposed are provided:

Observation 1: With the spectrum restriction, there is no IMD issue for CA_n5-n8 with two uplink transmission.

Observation 2: 3 antenna architecture is able to be accommodated in a smartphone so the space of smartphone is not a problem based on the commercial smartphone design. Currently 3 antenna architecture is much more preferred in real implementation for CA_n5A-n8A.

Proposal 1: It is proposed to use the RF front end architecture in figure 1 (2 antenna) and figure 2 (3 antenna) as baseline for further study.

Proposal 2: For CA_n5A-n8A, RAN4 should clarify which set of RF requirement, e.g., worst case, will be used to define minimum requirements by considering all UE RF architecture.

- Reference

- R4-2214445, WF on study on FS_NR_700800900, CATT, RAN4#104-e