3GPP TSG-RAN WG4 Meeting # 104bis-e									  R4-2216043

Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion on 2UL inter-band CA coexistence requirements

Source: 	Xiaomi

Agenda item:	6.3.3

Document for:	Discussion

- Introduction

In the last meeting, a WF on FR1 2UL inter-band CA coexistence requirements was approved. Whether to simplify the coexistence table with intersection principle was FFS. This contribution will provide our view on this open issue.

- Discussion

In general, the protected bands and frequency ranges for a band combination in principle is usually specified based on the intersection set from each constituent band coexistence requirements. But base on our reading on the latest spec, there are a lot of exceptions for protected band or frequency range.

We simply go through the spurious emission for UE coexistence in terms of several 2UL inter-band CA band combinations in the specification. A few of observations are given for spurious emission as below:

- CA_n1-n3: Band n104 are protected for single band n1 and n3 while band n104 is not protected for CA_n1-n3. The protected frequency ranges for CA_n1-n3 aggregates all the frequency ranges protected by n1 and n3.

- Both protected band and frequency range for CA_n1-n3 are not consistent with the intersection principle.

- CA_n1-n5: Band 22 is protected for single band n1 while not protected for single band n3, but Band 22 is protected for CA_n1-n5. No frequency range is protected for CA_n1-n5 but actually there are overlapping protected frequency range between n1 and n5.

- Both protected band and frequency range for CA_n1-n5 are not consistent with the intersection principle.

- CA_n1-n7: The protected bands for CA_n1-n7 are consistent with intersection principle. The protected frequency ranges for CA_n1-n7 aggregates all the frequency ranges protected by n1 and n7.

- The protected frequency range for CA_n1-n7 is not consistent with intersection principle.

Regarding the protected bands for 2UL inter-band CA, it is not always consistent with intersection principle if no errors are made in the spec. Probably one or several bands is exceptionally protected or not protected for some band combinations. It is not clear whether these are errors or exceptions. More clarification on these inconsistency is expected for these band combinations from the proponent.

Regarding the protected frequency ranges for 2UL inter-band CA, it is also not always consistent with intersection principle and the rule is diverse for different band combinations. Based on our reading on the spec, there are the following different cases, but not limited to:

- specified with intersection principle

- Specified as aggregation of all protected frequency range for each single band

- No protected frequency range even frequency ranges are protected by single band

- …

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79, n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15, XX |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79, n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n5, n89 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 103, NR Band n77, n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4,6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n1-n7 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76 NR Band n78, n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4. 7, 6 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4. 7, 6 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |

Observation 1: Regarding the protected bands for 2UL inter-band CA, it is not always consistent with intersection principle if no errors are made in the spec. More clarification on these inconsistency is expected for these band combinations from the proponent.

Observation 2: Regarding the protected frequency ranges for 2UL inter-band CA, it is not always consistent with intersection principle and the rule is diverse for different band combinations.

- Conclusion

This contribution mainly presents our observations on FR1 2UL inter-band CA coexistence requirements. The following observations are provided:

Observation 1: Regarding the protected bands for 2UL inter-band CA, it is not always consistent with intersection principle if no errors are made in the spec. It is not clear whether these are errors or exceptions. More clarification on these inconsistency is expected for these band combinations from the proponent.

Observation 2: Regarding the protected frequency ranges for 2UL inter-band CA, it is not always consistent with intersection principle and the rule is diverse for different band combinations.

- Reference

- R4-2214448, WF on FR1 2UL inter-band CA coexistence requirements, Apple, RAN4#104-e