3GPP TSG-RAN WG4 Meeting #104-bis-e                           R4-2216072

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP and discussion on CA_n5-n8

Agenda Item:	6.2.2.2

Document for:	Approval

Introduction

In RAN#96e meeting, a WF [1] was approved with the following outcomes.

1) Spectrum restriction assumption for the analysis

Agreement:

- UE should support the full range of spectrum for single band operations on both n5 and n8

- Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8.

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

2) UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8.

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

3) CBW assumption

The following is the assumption of BCS0:

n5: {5,10}MHz, n8{5, 10}MHz

4) RF parameters assumption

Continue the RF parameters discussion when the UE RF architecture and CBW are agreed.

5) Feasibility issues need to be analysed

The following issues will be analyzed in the feasibility study.

  - The MSD due to IMD and cross band isolation.

  - The impact on n5 out-of-band blocking requirement.

6) Other observations and proposals

The following RF requirement framework applies to CA_n5-n8, CA_n5-n8 and CA_n8-n20-28.

  - Operating bands for CA

  - Channel bandwidths per operating band for CA

  - UE co-existence requirement

  - ∆TIB and ∆RIB values

  - REFSENS requirements (MSD)

  - Maximum output power

7) UL configuration

The CBW of UL configuration CA_n5-n8 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

We analyzed and summarized all the RF requirements for CA_n5-n8. In this paper, we provide the following text proposals to be captured into TR.

- References

[1] R4-2214445, WF on study on FS_NR_700800900, CATT

- Text proposal

- <<Start of Change for TR 38.872>>

## 5.X	CA_n5-n8

- 5.X.1	UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

The reference RF architecture with three low band antennas for CA_n5-n8 is shown as below.

Figure 5.X.1-1:  The reference RF architecture with three low band antennas for CA_n5-n8

- 5.X.2	Common for 1 band UL and 2 bands UL of CA_n5-n8

- 5.X.2.1	Operating bands for CA

Since UL band n8 is overlapping with DL band n5, the frequency ranges for CA_n5-n8 need to be restricted. A solution with specific capability indication can be considered in case operators may have a different demand about frequency range restriction in the future.

Table 5.X.2.1-1:  CA band combination of band n5+n8

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 835 MHz | 869 MHz | – | 880 MHz | FDD |
| n8 | 904 MHz | – | 915 MHz | 949 MHz | – | 960 MHz | FDD |

- 5.X.2.2	Channel bandwidths per operating band for CA

Based on WF R4-2214445, BCS0 for CA_n5A-n8A can be specified as below.

Table 5.X.2.2-1: Supported bandwidths per CA band combination of band n5+n6

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |

- 5.X.2.3	UE co-existence studies

Table 5.X.2.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n8.

Table 5.X.2.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 | 4400 | 4575 |

Table 5.X.2.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 | 4625 | 4800 |

Based on the analysis above, there is no need to specify harmonics and harmonics mixing exception for CA_n5-n8 as we didn’t specify them in current spec.

- 5.X.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n8, the TIB,c and RIB,c are given in the tables below.

Table 5.X.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | FFS |
|  | n8 | FFS |

Table 5.X.2.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n8 | n5 | FFS |
|  | n8 | FFS |

- 5.X.2.5	REFSENS requirements

As CA_n5-n8 has been specified into current spec TS 38.101-1, there is no harmonics and harmonics mixing exception.

The REFSENS exception due to cross band isolation for CA_n5-n8 can be specified as below.

Table 5.X.2.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | FFS | >ACLR2 |

The following terminology is used to define the source of cross-band isolation interference:

- “ACLR1” indicates that the first adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

- “ACLR2” indicates that the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

- “>ACLR2” indicates that neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

- 5.X.3	Specific for 2 bands UL of CA_n5-n8

- 5.X.3.1	Maximum output power for inter-band CA

Table 5.X.3.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n8A | 23 | +2/-3 |

- 5.X.3.2	UE co-existence studies

Table 5.X.3.2-1 lists Band n5 + Band n8 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.X.3.2-1: Band n5 and Band n8 UL IMD products with frequency range restriction

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 835 | 904 | 915 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 69 | 91 | 1728 | 1750 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 973 | 1006 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 2585 | 2632 | 2665 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1557 | 1601 | 1877 | 1921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3420 | 3536 | 3580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 182 | 138 | 3456 | 3500 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2836 | 2781 | 2436 | 2381 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 1042 | 697 | 642 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4440 | 4495 | 4200 | 4255 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4360 | 4415 | 4280 | 4335 |

Based on Table 5.X.3.2-1, there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz	–	835 MHz	DL: 869 MHz	–	880 MHz;

n8	UL: 904 MHz	–	915 MHz	DL: 949 MHz	–	960 MHz.

Table 5.X.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.X.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n8 | E-UTRA Band 1, 28, 31, 34, 38, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

- 5.X.3.3	REFSENS requirements

There is no need to specify REFSENS exception due to IMD interference for CA_n5-n8.

- <<End of Change>>