3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2216073

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP and Discussion on test burden reduction

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In RAN4#104 meeting, we provided a discussion paper [1] to consider the potential directions about test burden reduction. The outcomes were captured into the Way forward [2] as below.

<Way forward>:

Proposal 1: Generally, if CA_nA-nB, DC_A_nB (DC_A_nD), DC_B_nA (DC_B_nC), DC_nB_A, DC_nA_B have same spurious emission requirements for UE to UE coexistence, there is no need to test the spurious emission requirements for UE to UE coexistence for each UL configuration again and again. Once one of these UL configurations is verified, the other UL configurations for different feature in same band combination can be considered as being capable of meeting these requirements.

NOTE: Band nC and nD are the corresponding SUL bands with same UL frequency range of band nA and nB, e.g. SUL band n80 has same UL frequency range of band n3.

FFS how to reduce the test burden for MOP requirements.

<Way forward>:

Proposal 2: reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations can be further simplified by using same approach of NR CA, as what we have done for R17 NR CA band combinations in BCS4 WI.

FFS how to address the different test configurations between band combinations.

<Way forward>:

Proposal 3: For band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB which have MSD due to IMD interference, FFS how to reduce the test burden.

FFS whether we only choose the worst MSD case.

<Way forward>:

Proposal 4: For band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB which doesn’t have any MSD requirements, it’s recommended to test one of them in order to reduce the test burden for REFSENS requirements.

FFS how to address the potential different delta Rib requirements.

In this meeting, we’d like to address the open issues and provide a text proposal.

Discussion on Tx RF requirements for different features

For Tx RF requirements of band combinations, the main requirements related to band combination-specific are spurious emission for UE-to-UE coexistence and MOP. As we discussed in last meeting, for example, MOP requirements for CA_n1A-n3A/DC_1A_n80A/DC_1A_n3A/DC_3A_n1A/DC_3A_n84A/DC_n3A_1A are same and the same RF architecture can be reused to achieve the same MOP requirements for different features, i.e. 23dBm for PC3 with +2/-3dB tolerance. Based on the latest specification TS 38.101-1-h60 and TS 38.101-3-h60, we summarized PC3 MOP requirements for all the UL NR CA, UL NR DC, EN-DC and NE-DC band combinations in table 1. For each row, the band combinations have the same frequency range for each band and same RF implementation in spite of different features. Thus, there is no need to test the MOP requirements for every individual band combination in the same row of table 1. It’s proposed to choose one band combination for MOP testing for each row. It’s recommended to preferentially test UL NR CA band combination. However, if there is no corresponding UL NR CA band combination, it’s recommended to choose one band combination randomly for MOP testing.

Proposal 1: For PC3 MOP requirements, there is no need to test the MOP requirements for every individual band combination in the same row of table 1 due to the same RF implementation.

Proposal 2: It’s proposed to choose one band combination for MOP testing for each row in table 1. It’s recommended to preferentially test UL NR CA band combination if UE support it. However, if there is no corresponding UL NR CA band combination, it’s recommended to randomly choose one band combination UE support for MOP testing.

Table 1 Band combination PC3 MOP requirements for the same frequency range with different features

| Uplink NR CA Configuration | Uplink NR DC Configuration | Uplink EN-DC Configuration | Uplink NE-DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | DC_n1A-n3A | DC_1A_n3A DC_3A_n1A DC_1A_n80A DC_3A_n84A | DC_n3A_1A | 23 | +2/-3 |
| CA_n1A-n5A |  | DC_1A_n5A |  | 23 | +2/-3 |
| CA_n1A-n7A | DC_n1A-n7A | DC_1A_n7A DC_7A_n1A |  | 23 | +2/-3 |
| CA_n1A-n8A |  | DC_1A_n8A DC_8A_n1A | DC_n8A_1A | 23 | +2/-3 |
| CA_n1A-n18A |  |  |  | 23 | +2/-3 |
| CA_n1A-n20A |  | DC_1A_n20A DC_20A_n1A |  | 23 | +2/-3 |
| CA_n1A-n28A | DC_n1A-n28A | DC_1A_n28A DC_28A_n1A | DC_n1A_28A | 23 | +2/-3 |
|  |  | DC_1A_n38A DC_38A_n1A |  | 23 | +2/-3 |
| CA_n1A-n40A |  | DC_1A_n40A DC_40A_n1A |  | 23 | +2/-3 |
| CA_n1A-n41A | DC_n1A-n41A | DC_1A_n41A DC_41A_n1A |  | 23 | +2/-3 |
|  |  | DC_1A_n50A |  | 23 | +2/-3 |
|  |  | DC_1A_n51A |  | 23 | +2/-3 |
|  |  | DC_1A_n71A |  | 23 | +2/-3 |
| CA_n1A-n74A |  |  |  | 23 | +2/-3 |
| CA_n1A-n77A | DC_n1A-n77A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | DC_n77A_1A | 23 | +2/-3 |
| CA_n1A-n78A | DC_n1A-n78A | DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | DC_n78A_1A | 23 | +2/-3 |
| CA_n1A-n79A | DC_n1A-n79A | DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
| CA_n2A-n5A | DC_n2A-n5A | DC_2A_n5A DC_5A_n2A |  | 23 | +2/-3 |
| CA_n2A-n7A |  | DC_2A_n7A DC_7A_n2A |  | 23 | +2/-3 |
| CA_n2A-n12A |  | DC_2A_n12A DC_12A_n2A |  | 23 | +2/-3 |
| CA_n2A-n14A |  | DC_14A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n25A |  | 23 | +2/-3 |
|  |  | DC_2A_n28A DC_28A_n2A |  | 23 | +2/-3 |
| CA_n2A-n30A |  | DC_2A_n30A DC_30A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n38A |  | 23 | +2/-3 |
|  |  | DC_2A_n41A |  | 23 | +2/-3 |
|  |  | DC_2A_n46A |  | 23 | +2/-3 |
| CA_n2A-n48A | DC_n2A-n48A | DC_2A_n48A DC_48A_n2A |  | 23 | +2/-3 |
| CA_n2A-n66A | DC_n2A-n66A | DC_2A_n66A DC_66A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n71A DC_71A_n2A |  | 23 | +2/-3 |
| CA_n2A-n77A | DC_n2A-n77A | DC_2A_n77A |  | 23 | +2/-3 |
| CA_n2A-n78A |  | DC_2A_n78A |  | 23 | +2/-3 |
| CA_n3A-n5A |  | DC_3A_n5A |  | 23 | +2/-3 |
| CA_n3A-n7A |  | DC_3A_n7A DC_7A_n3A DC_7A_n80A |  | 23 | +2/-3 |
| CA_n3A-n8A |  | DC_3A_n8A DC_8A_n3A DC_8A_n80A | DC_n3A_8A DC_n8A_3A | 23 | +2/-3 |
| CA_n3A-n18A |  | DC_18A_n3A |  | 23 | +2/-3 |
| CA_n3A-n20A |  | DC_3A_n20A DC_3A_n82A DC_20A_n3A DC_20A_n80A |  | 23 | +2/-3 |
| CA_n3A-n28A | DC_n3A-n28A | DC_3A_n28A DC_28A_n3A | DC_n28A_3A | 23 | +2/-3 |
| CA_n3A-n34A |  |  |  | 23 | +2/-3 |
| CA_n3-n38A |  | DC_3A_n38A DC_38A_n3A |  | 23 | +2/-3 |
| CA_n3A-n40A |  | DC_3A_n40A |  | 23 | +2/-3 |
| CA_n3A-n41A | DC_n3A-n41A | DC_3A_n41A DC_3A_n80A_ULSUP-TDM_n41 DC_41A_n3A | DC_n41A_3A | 23 | +2/-3 |
|  |  | DC_3A_n50A |  | 23 | +2/-3 |
|  |  | DC_3A_n51A |  | 23 | +2/-3 |
|  |  | DC_3A_n71A |  | 23 | +2/-3 |
| CA_n3A-n74A |  |  |  | 23 | +2/-3 |
| CA_n3A-n77A | DC_n3A-n77A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A | DC_n77A_3A | 23 | +2/-3 |
| CA_n3A-n78A | DC_n3A-n78A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A | DC_n78A_3A | 23 | +2/-3 |
| CA_n3A-n79A | DC_n3A-n79A | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
|  |  | DC_4A_n2A |  | 23 | +2/-3 |
|  |  | DC_4A_n5A |  | 23 | +2/-3 |
|  |  | DC_4A_n7A |  | 23 | +2/-3 |
|  |  | DC_4A_n28A |  | 23 | +2/-3 |
|  |  | DC_4A_n38A |  | 23 | +2/-3 |
|  |  | DC_4A_n41A |  | 23 | +2/-3 |
|  |  | DC_4A_n78A |  | 23 | +2/-3 |
| CA_n5A-n7A |  | DC_5A_n7A DC_7A_n5A |  | 23 | +2/-3 |
| CA_n5A-n12A |  | DC_5A_n12A DC_12A_n5A |  | 23 | +2/-3 |
| CA_n5A-n14A |  | DC_14A_n5A |  | 23 | +2/-3 |
| CA_n5A-n25A |  |  |  | 23 | +2/-3 |
| CA_n5A-n30A |  | DC_5A_n30A DC_30A_n5A |  | 23 | +2/-3 |
|  |  | DC_5A_n38A |  | 23 | +2/-3 |
| CA_n5A-n40A |  | DC_5A_n40A |  | 23 | +2/-3 |
| CA_n5A-n48A | DC_n5A-n48A | DC_5A_n48A DC_48A_n5A |  | 23 | +2/-3 |
| CA_n5A-n66A | DC_n5A-n66A | DC_5A_n66A DC_66A_n5A |  | 23 | +2/-3 |
|  |  | DC_5A_n71A DC_71A_n5A |  | 23 | +2/-3 |
| CA_n5A-n77A | DC_n5A-n77A | DC_5A_n77A |  | 23 | +2/-3 |
| CA_n5A-n78A |  | DC_5A_n78A | DC_n78A_5A | 23 | +2/-3 |
| CA_n5A-n79A |  | DC_5A_n79A |  | 23 | +2/-3 |
|  |  | DC_7A_n8A DC_8A_n7A |  | 23 | +2/-3 |
|  |  | DC_7A_n20A DC_20A_n7A |  | 23 | +2/-3 |
| CA_n7A-n25A |  | DC_7A_n25A |  | 23 | +2/-3 |
| CA_n7A-n28A |  | DC_7A_n28A DC_28A_n7A |  | 23 | +2/-3 |
| CA_n7A-n40A |  | DC_7A_n40A |  | 23 | +2/-3 |
| CA_n7A-n46A | DC_n7A-n46A |  |  | 23 | +2/-3 |
|  |  | DC_7A_n51A |  | 23 | +2/-3 |
| CA_n7A-n66A |  | DC_7A_n66A DC_66A_n7A |  | 23 | +2/-3 |
|  |  | DC_7A_n71A |  | 23 | +2/-3 |
| CA_n7A-n77A |  | DC_7A_n77A |  | 23 | +2/-3 |
| CA_n7A-n78A | DC_n7A-n78A | DC_7A_n78A | DC_n78A_7A | 23 | +2/-3 |
|  |  | DC_7A_n79A |  | 23 | +2/-3 |
|  |  | DC_8A_n2A |  | 23 | +2/-3 |
|  |  | DC_8A_n20A DC_20A_n8A |  | 23 | +2/-3 |
|  |  | DC_8A_n28A DC_28A_n8A | DC_n28A_8A |  |  |
| CA_n8A-n34A |  | DC_8A_n34A |  | 23 | +2/-3 |
| CA_n8A-n39A |  | DC_8A_n39A |  | 23 | +2/-3 |
| CA_n8A-n40A |  | DC_8A_n40A |  | 23 | +2/-3 |
| CA_n8A-n41A |  | DC_8A_n41A DC_8A_n81A_ULSUP-TDM_n41 | DC_n41A_8A | 23 | +2/-3 |
| CA_n8A-n77A |  | DC_8A_n77A | DC_n77A_8A | 23 | +2/-3 |
| CA_n8A-n78A |  | DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A | DC_n78A_8A | 23 | +2/-3 |
| CA_n8A-n79A |  | DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
|  |  | DC_11A_n1A |  | 23 | +2/-3 |
|  |  | DC_11A_n3A |  | 23 | +2/-3 |
|  |  | DC_11A_n28A |  | 23 | +2/-3 |
|  |  | DC_11A_n41A |  | 23 | +2/-3 |
|  |  | DC_11A_n77A |  | 23 | +2/-3 |
|  |  | DC_11A_n78A |  | 23 | +2/-3 |
|  |  | DC_11A_n79A |  | 23 | +2/-3 |
|  |  | DC_12A_n7A |  | 23 | +2/-3 |
|  |  | DC_12A_n25A |  | 23 | +2/-3 |
| CA_n12A-n30A |  | DC_12A_n30A |  | 23 | +2/-3 |
|  |  | DC_12A_n38A |  | 23 | +2/-3 |
|  |  | DC_12A_n41A |  | 23 | +2/-3 |
| CA_n12A-n66A |  | DC_12A_n66A DC_66A_n12A |  | 23 | +2/-3 |
|  |  | DC_12A_n71A |  | 23 | +2/-3 |
| CA_n12A-n77A |  | DC_12A_n77A |  | 23 | +2/-3 |
|  |  | DC_12A_n78A |  | 23 | +2/-3 |
|  |  | DC_13A_n2A |  | 23 | +2/-3 |
|  |  | DC_13A_n5A |  | 23 | +2/-3 |
|  |  | DC_13A_n7A |  | 23 | +2/-3 |
| CA_n13A-n25A |  | DC_13A_n25A |  | 23 | +2/-3 |
|  |  | DC_13A_n48A |  | 23 | +2/-3 |
| CA_n13A-n66A |  | DC_13A_n66A |  | 23 | +2/-3 |
|  |  | DC_13A_n71A |  | 23 | +2/-3 |
| CA_n13A-n77A |  | DC_13A_n77A |  | 23 | +2/-3 |
|  |  | DC_13A_n78A |  | 23 | +2/-3 |
| CA_n14A-n30A |  | DC_14A_n30A |  | 23 | +2/-3 |
| CA_n14A-n66A |  | DC_14A_n66A |  | 23 | +2/-3 |
| CA_n14A-n77A |  | DC_14A_n77A |  | 23 | +2/-3 |
| CA_n18A-n28A |  | DC_18A_n28A |  | 23 | +2/-3 |
| CA_n18A-n41A |  | DC_18A_n41A |  | 23 | +2/-3 |
| CA_n18A-n74A |  |  |  | 23 | +2/-3 |
| CA_n18A-n77A |  | DC_18A_n77A |  | 23 | +2/-3 |
| CA_n18A-n78A |  | DC_18A_n78A |  | 23 | +2/-3 |
|  |  | DC_18A_n79A |  | 23 | +2/-3 |
|  |  | DC_19A_n1A |  | 23 | +2/-3 |
|  |  | DC_19A_n77A |  | 23 | +2/-3 |
|  |  | DC_19A_n78A |  | 23 | +2/-3 |
|  |  | DC_19A_n79A |  | 23 | +2/-3 |
| CA_n20A-n28A |  | DC_20A_n28A DC_20A_n83A | DC_n28A_20A | 23 | +2/-3 |
|  |  | DC_20A_n38A |  | 23 | +2/-3 |
|  |  | DC_20A_n41A |  | 23 | +2/-3 |
|  |  | DC_20A_n50A |  | 23 | +2/-3 |
|  |  | DC_20A_n51A |  | 23 | +2/-3 |
|  |  | DC_20A_n77A |  | 23 | +2/-3 |
| CA_n20A-n78A |  | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
|  |  | DC_21A_n1A |  | 23 | +2/-3 |
|  |  | DC_21A_n28A |  | 23 | +2/-3 |
|  |  | DC_21A_n77A |  | 23 | +2/-3 |
|  |  | DC_21A_n78A |  | 23 | +2/-3 |
|  |  | DC_21A_n79A |  | 23 | +2/-3 |
| CA_n24A-n41A |  |  |  | 23 | +2/-3 |
| CA_n24A-n48A |  |  |  | 23 | +2/-3 |
| CA_n24A-n77A |  |  |  | 23 | +2/-3 |
| CA_n25A-n38A |  | DC_25A_n41A |  | 23 | +2/-3 |
| CA_n25A-n41A |  |  |  | 23 | +2/-3 |
| CA_25A-n48A |  | DC_48A_n25A |  | 23 | +2/-3 |
| CA_n25A-n66A |  | DC_66A_n25A |  | 23 | +2/-3 |
| CA_n25A-n77A |  | DC_25A_n77A |  | 23 | +2/-3 |
| CA_n25A-n78A |  | DC_25A_n78A |  | 23 | +2/-3 |
|  |  | DC_26A_n25A |  | 23 | +2/-3 |
|  |  | DC_26A_n41A |  | 23 | +2/-3 |
| CA_n26A-n66A |  |  |  | 23 | +2/-3 |
| CA_n26A-n70A |  |  |  | 23 | +2/-3 |
|  |  | DC_26A_n77A |  | 23 | +2/-3 |
|  |  | DC_26A_n78A | DC_n78A_26A | 23 | +2/-3 |
|  |  | DC_26A_n79A |  | 23 | +2/-3 |
|  |  | DC_28A_n5A |  | 23 | +2/-3 |
| CA_n28A-n34A |  |  | DC_n28A_34A | 23 | +2/-3 |
| CA_n28A-n39A |  |  | DC_n28A_39A | 23 | +2/-3 |
| CA_n28A-n40A |  | DC_28A_n40A | DC_n28A_40A | 23 | +2/-3 |
| CA_n28A-n41A | DC_n28A-n41A | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A DC_41A_n28A |  | 23 | +2/-3 |
| CA_n28A-n46A | DC_n28A-n46A |  |  | 23 | +2/-3 |
| CA_n28A-n50A |  | DC_28A_n50A |  | 23 | +2/-3 |
|  |  | DC_28A_n51A |  | 23 | +2/-3 |
|  |  | DC_28A_n66A DC_66A_n28A |  | 23 | +2/-3 |
| CA_n28A-n74A |  |  |  | 23 | +2/-3 |
| CA_n28A-n77A | DC_n28A-n77A | DC_28A_n77A |  | 23 | +2/-3 |
| CA_n28A-n78A | DC_n28A-n78A | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
| CA_n28A-n79A | DC_n28A-n79A | DC_28A_n79A |  | 23 | +2/-3 |
| CA_n34A-n79A |  |  |  | 23 | +2/-3 |
| CA_n30A-n66A |  | DC_30A_n66A DC_66A_n30A |  | 23 | +2/-3 |
| CA_n30A-n77A |  | DC_30A_n77A |  | 23 | +2/-3 |
| CA_n34A-n40A |  |  |  | 23 | +2/-3 |
| CA_n34A-n41A |  |  | DC_n41A_34A | 23 | +2/-3 |
|  |  | DC_38A_n8A |  | 23 | +2/-3 |
|  |  | DC_38A_n28A |  | 23 | +2/-3 |
| CA_n38A-n66A |  | DC_66A_n38A |  | 23 | +2/-3 |
| CA_n38A-n78A |  | DC_38A_n78A |  | 23 | +2/-3 |
|  |  | DC_38A_n79A |  | 23 | +2/-3 |
| CA_n39A-n40A |  | DC_39A_n40A |  | 23 | +2/-3 |
| CA_n39A-n41A |  | DC_39A_n41A | DC_n41A_39A | 23 | +2/-3 |
|  |  | DC_39A_n78A |  | 23 | +2/-3 |
| CA_n39A-n79A |  | DC_39A_n79A |  | 23 | +2/-3 |
| CA_n40A-n41A |  | DC_40A_n41A | DC_n41A_40A | 23 | +2/-3 |
| CA_n40A-n77A |  | DC_40A_n77A |  | 23 | +2/-3 |
| CA_n40A-n78A |  | DC_40A_n78A |  | 23 | +2/-3 |
| CA_n40A-n79A |  | DC_40A_n79A |  | 23 | +2/-3 |
| CA_n41A-n48A |  |  |  | 23 | +2/-3 |
| CA_n41A-n50A |  |  |  | 23 | +2/-3 |
| CA_n41A-n66A |  | DC_66A_n41A |  | 23 | +2/-3 |
| CA_n41A-n70A |  |  |  | 23 | +2/-3 |
| CA_n41A-n71A |  | DC_71A_n41A |  | 23 | +2/-3 |
| CA_n41A-n74A |  |  |  | 23 | +2/-3 |
| CA_n41A-n77A | DC_n41A-n77A | DC_41A_n77A |  | 23 | +2/-3 |
| CA_n41A-n78A | DC_n41A-n78A | DC_41A_n78A |  | 23 | +2/-3 |
| CA_n41A-n79A |  | DC_41A_n79A |  | 23 | +2/-3 |
|  |  | DC_42A_n1A |  | 23 | +2/-3 |
|  |  | DC_42A_n3A |  | 23 | +2/-3 |
|  |  | DC_42A_n28A |  | 23 | +2/-3 |
|  |  | DC_42A_n51A |  | 23 | +2/-3 |
|  |  | DC_42A_n77A |  | 23 | +2/-3 |
|  |  | DC_42A_n78A |  | 23 | +2/-3 |
|  |  | DC_42A_n79A |  | 23 | +2/-3 |
| CA_n46A-n48A | DC_n46A-n48A |  |  | 23 | +2/-3 |
| CA_n46A-n48B | DC_n46A-n48B |  |  | 23 | +2/-3 |
| CA_n46A-n78A | DC_n46A-n78A |  |  | 23 | +2/-3 |
|  |  | DC_48A_n12A |  | 23 | +2/-3 |
| CA_n48A-n66A | DC_n48A-n66A | DC_48A_n66A DC_66A_n48A |  | 23 | +2/-3 |
| CA_n48A-n70A | DC_n48A-n70A |  |  | 23 | +2/-3 |
| CA_n48A-n71A | DC_n48A-n71A | DC_48A_n71A DC_71A_n48A |  | 23 | +2/-3 |
| CA_n48A-n96A | DC_n48A-n96A |  |  | 23 | +2/-3 |
| CA_n48B-n96A | DC_n48B-n96A |  |  | 23 | +2/-3 |
| CA_n48A-n96B |  |  |  | 23 | +2/-3 |
| CA_n50A-n78A |  |  |  | 23 | +2/-3 |
|  |  | DC_66A_n46A |  | 23 | +2/-3 |
| CA_n66A-n71A |  | DC_66A_n71A DC_71A_n66A |  | 23 | +2/-3 |
| CA_n66A-n77A | DC_n66A-n77A | DC_66A_n77A |  | 23 | +2/-3 |
| CA_n66A-n78A |  | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
| CA_n70A-n71A |  |  |  | 23 | +2/-3 |
| CA_n70A-n78A |  |  |  | 23 | +2/-3 |
|  |  | DC_71A_n38A |  | 23 | +2/-3 |
| CA_n71A-n77A |  |  |  | 23 | +2/-3 |
| CA_n71A-n78A |  | DC_71A_n78A |  | 23 | +2/-3 |
| CA_n74A-n77A |  |  |  | 23 | +2/-3 |
| CA_n74A-n78A |  |  |  | 23 | +2/-3 |
| CA_n77A-n79A | DC_n77A-n79A |  |  | 23 | +2/-3 |
| CA_n78A-n79A |  |  |  | 23 | +2/-3 |
| CA_n78A-n92A |  |  |  | 23 | +2/-3 |

Discussion on Rx RF requirements for different features

For reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations, as we have agreed in last meeting, the reference sensitivity exception can be further simplified by using same approach of NR CA, as what we have done for R17 NR CA band combinations in BCS4 WI. As for how to address the different test configurations between band combinations, it’s recommended to follow the same principles that we have agreed in NR CA BCS4 WI.

Proposal 3: For reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations, it’s recommended to follow the same principles that we have agreed in NR CA BCS4 WI in WF R4-2210565.

As we have agreed that it’s recommended to test one of them in order to reduce the test burden for REFSENS requirements for band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB which don’t have any MSD requirements. For the case that band combinations with different features in same frequency range have different delta Rib requirements, it’s proposed for UE to meet the stricter delta Rib requirements. The reason is that the difference between different delta Rib requirements is neglected, but the same RF implementation is used to achieve these band combinations.

Proposal 4: For the case that band combinations with different features in same frequency range have different delta Rib requirements, it’s proposed for UE to meet the stricter delta Rib requirements. (It’s noted band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB don’t have any MSD requirements)

In total, we provide the corresponding text proposal to reflect the agreements which we have agreed and some solutions which are used to address the open issues.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: For PC3 MOP requirements, there is no need to test the MOP requirements for every individual band combination in the same row of table 1 due to the same RF implementation.

Proposal 2: It’s proposed to choose one band combination for MOP testing for each row in table 1. It’s recommended to preferentially test UL NR CA band combination if UE support it. However, if there is no corresponding UL NR CA band combination, it’s recommended to randomly choose one band combination UE support for MOP testing.

Proposal 3: For reference sensitivity exception due to harmonic/harmonic mixing or cross band isolation specified for ENDC band combinations, it’s recommended to follow the same principles that we have agreed in NR CA BCS4 WI in WF R4-2210565.

Proposal 4: For the case that band combinations with different features in same frequency range have different delta Rib requirements, it’s proposed for UE to meet the stricter delta Rib requirements. (It’s noted band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB don’t have any MSD requirements)

In total, we provide the corresponding text proposal to reflect the agreements which we have agreed and some solutions which are used to address the open issues.

- References

[1] R4-2213163, General discussion on Simplification of band combination specification, Huawei, HiSilicon

[2] R4-2214447, Way forward on test burden reduction for band combination, Huawei, HiSilicon

[3] R4-2210565, WF on criteria on Rel-17 enhanced MSD table format, Skyworks Solutions, Inc.

- Text proposal

- <<Start of TP for TR 38.846>>

- 7	Test burden reduction for band combinations

- 7.1	General

One of the objectives in this SI is to investigate the feasibility and optimize the specification structure and reduce the test burden. Currently, the main RF requirements related to specific band combinations include maximum output power (MOP), spurious emission for UE-to-UE coexistence, REFSENS and REFSENS exceptions due to harmonic/harmonic mixing/cross band isolation/IMD interference. Obviously, RF requirements for different features on the same band combination have some similarities and dependency. Especially, the RF implementations are similar and RF architectures can be reused for different features on the same band combination. It’s very meaningful to study the similarity and dependency of RF requirements for different features on the same band combination and find out the feasibility to further optimize the specification structure and reduce the test burden. For example, CA_nA-nB (NR CA), DC_nA-nB (NR-DC), DC_A_nB (EN-DC), DC_B_nA (EN-DC), DC_nB_A (NE-DC), DC_nA_B (NE-DC) (different features on same band combination) can use same RF implementation.

- 7.2	Similarity and Dependency of Tx RF requirements for different features on the same band combination

- 7.2.1	Maximum output power

We summarized PC3 MOP requirements for all the UL NR CA, UL NR DC, EN-DC and NE-DC band combinations in table 7.2.1-1 based on the TS 38.101-1-h60 and TS 38.101-3-h60. One band combination can be chosen to verify PC3 MOP testing for some band combinations in same row in table 7.2.1-1, as a result of reducing test burden. It’s recommended to preferentially test UL NR CA band combination if UE support it. However, if there is no corresponding UL NR CA band combination, it’s recommended to randomly choose one band combination that UE support for MOP testing.

Table 7.2.1-1 Band combination PC3 MOP requirements for the same frequency range with different features

| Uplink NR CA Configuration | Uplink NR DC Configuration | Uplink EN-DC Configuration | Uplink NE-DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | DC_n1A-n3A | DC_1A_n3A DC_3A_n1A DC_1A_n80A DC_3A_n84A | DC_n3A_1A | 23 | +2/-3 |
| CA_n1A-n5A |  | DC_1A_n5A |  | 23 | +2/-3 |
| CA_n1A-n7A | DC_n1A-n7A | DC_1A_n7A DC_7A_n1A |  | 23 | +2/-3 |
| CA_n1A-n8A |  | DC_1A_n8A DC_8A_n1A | DC_n8A_1A | 23 | +2/-3 |
| CA_n1A-n18A |  |  |  | 23 | +2/-3 |
| CA_n1A-n20A |  | DC_1A_n20A DC_20A_n1A |  | 23 | +2/-3 |
| CA_n1A-n28A | DC_n1A-n28A | DC_1A_n28A DC_28A_n1A | DC_n1A_28A | 23 | +2/-3 |
|  |  | DC_1A_n38A DC_38A_n1A |  | 23 | +2/-3 |
| CA_n1A-n40A |  | DC_1A_n40A DC_40A_n1A |  | 23 | +2/-3 |
| CA_n1A-n41A | DC_n1A-n41A | DC_1A_n41A DC_41A_n1A |  | 23 | +2/-3 |
|  |  | DC_1A_n50A |  | 23 | +2/-3 |
|  |  | DC_1A_n51A |  | 23 | +2/-3 |
|  |  | DC_1A_n71A |  | 23 | +2/-3 |
| CA_n1A-n74A |  |  |  | 23 | +2/-3 |
| CA_n1A-n77A | DC_n1A-n77A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | DC_n77A_1A | 23 | +2/-3 |
| CA_n1A-n78A | DC_n1A-n78A | DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | DC_n78A_1A | 23 | +2/-3 |
| CA_n1A-n79A | DC_n1A-n79A | DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
| CA_n2A-n5A | DC_n2A-n5A | DC_2A_n5A DC_5A_n2A |  | 23 | +2/-3 |
| CA_n2A-n7A |  | DC_2A_n7A DC_7A_n2A |  | 23 | +2/-3 |
| CA_n2A-n12A |  | DC_2A_n12A DC_12A_n2A |  | 23 | +2/-3 |
| CA_n2A-n14A |  | DC_14A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n25A |  | 23 | +2/-3 |
|  |  | DC_2A_n28A DC_28A_n2A |  | 23 | +2/-3 |
| CA_n2A-n30A |  | DC_2A_n30A DC_30A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n38A |  | 23 | +2/-3 |
|  |  | DC_2A_n41A |  | 23 | +2/-3 |
|  |  | DC_2A_n46A |  | 23 | +2/-3 |
| CA_n2A-n48A | DC_n2A-n48A | DC_2A_n48A DC_48A_n2A |  | 23 | +2/-3 |
| CA_n2A-n66A | DC_n2A-n66A | DC_2A_n66A DC_66A_n2A |  | 23 | +2/-3 |
|  |  | DC_2A_n71A DC_71A_n2A |  | 23 | +2/-3 |
| CA_n2A-n77A | DC_n2A-n77A | DC_2A_n77A |  | 23 | +2/-3 |
| CA_n2A-n78A |  | DC_2A_n78A |  | 23 | +2/-3 |
| CA_n3A-n5A |  | DC_3A_n5A |  | 23 | +2/-3 |
| CA_n3A-n7A |  | DC_3A_n7A DC_7A_n3A DC_7A_n80A |  | 23 | +2/-3 |
| CA_n3A-n8A |  | DC_3A_n8A DC_8A_n3A DC_8A_n80A | DC_n3A_8A DC_n8A_3A | 23 | +2/-3 |
| CA_n3A-n18A |  | DC_18A_n3A |  | 23 | +2/-3 |
| CA_n3A-n20A |  | DC_3A_n20A DC_3A_n82A DC_20A_n3A DC_20A_n80A |  | 23 | +2/-3 |
| CA_n3A-n28A | DC_n3A-n28A | DC_3A_n28A DC_28A_n3A | DC_n28A_3A | 23 | +2/-3 |
| CA_n3A-n34A |  |  |  | 23 | +2/-3 |
| CA_n3-n38A |  | DC_3A_n38A DC_38A_n3A |  | 23 | +2/-3 |
| CA_n3A-n40A |  | DC_3A_n40A |  | 23 | +2/-3 |
| CA_n3A-n41A | DC_n3A-n41A | DC_3A_n41A DC_3A_n80A_ULSUP-TDM_n41 DC_41A_n3A | DC_n41A_3A | 23 | +2/-3 |
|  |  | DC_3A_n50A |  | 23 | +2/-3 |
|  |  | DC_3A_n51A |  | 23 | +2/-3 |
|  |  | DC_3A_n71A |  | 23 | +2/-3 |
| CA_n3A-n74A |  |  |  | 23 | +2/-3 |
| CA_n3A-n77A | DC_n3A-n77A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A | DC_n77A_3A | 23 | +2/-3 |
| CA_n3A-n78A | DC_n3A-n78A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A | DC_n78A_3A | 23 | +2/-3 |
| CA_n3A-n79A | DC_n3A-n79A | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
|  |  | DC_4A_n2A |  | 23 | +2/-3 |
|  |  | DC_4A_n5A |  | 23 | +2/-3 |
|  |  | DC_4A_n7A |  | 23 | +2/-3 |
|  |  | DC_4A_n28A |  | 23 | +2/-3 |
|  |  | DC_4A_n38A |  | 23 | +2/-3 |
|  |  | DC_4A_n41A |  | 23 | +2/-3 |
|  |  | DC_4A_n78A |  | 23 | +2/-3 |
| CA_n5A-n7A |  | DC_5A_n7A DC_7A_n5A |  | 23 | +2/-3 |
| CA_n5A-n12A |  | DC_5A_n12A DC_12A_n5A |  | 23 | +2/-3 |
| CA_n5A-n14A |  | DC_14A_n5A |  | 23 | +2/-3 |
| CA_n5A-n25A |  |  |  | 23 | +2/-3 |
| CA_n5A-n30A |  | DC_5A_n30A DC_30A_n5A |  | 23 | +2/-3 |
|  |  | DC_5A_n38A |  | 23 | +2/-3 |
| CA_n5A-n40A |  | DC_5A_n40A |  | 23 | +2/-3 |
| CA_n5A-n48A | DC_n5A-n48A | DC_5A_n48A DC_48A_n5A |  | 23 | +2/-3 |
| CA_n5A-n66A | DC_n5A-n66A | DC_5A_n66A DC_66A_n5A |  | 23 | +2/-3 |
|  |  | DC_5A_n71A DC_71A_n5A |  | 23 | +2/-3 |
| CA_n5A-n77A | DC_n5A-n77A | DC_5A_n77A |  | 23 | +2/-3 |
| CA_n5A-n78A |  | DC_5A_n78A | DC_n78A_5A | 23 | +2/-3 |
| CA_n5A-n79A |  | DC_5A_n79A |  | 23 | +2/-3 |
|  |  | DC_7A_n8A DC_8A_n7A |  | 23 | +2/-3 |
|  |  | DC_7A_n20A DC_20A_n7A |  | 23 | +2/-3 |
| CA_n7A-n25A |  | DC_7A_n25A |  | 23 | +2/-3 |
| CA_n7A-n28A |  | DC_7A_n28A DC_28A_n7A |  | 23 | +2/-3 |
| CA_n7A-n40A |  | DC_7A_n40A |  | 23 | +2/-3 |
| CA_n7A-n46A | DC_n7A-n46A |  |  | 23 | +2/-3 |
|  |  | DC_7A_n51A |  | 23 | +2/-3 |
| CA_n7A-n66A |  | DC_7A_n66A DC_66A_n7A |  | 23 | +2/-3 |
|  |  | DC_7A_n71A |  | 23 | +2/-3 |
| CA_n7A-n77A |  | DC_7A_n77A |  | 23 | +2/-3 |
| CA_n7A-n78A | DC_n7A-n78A | DC_7A_n78A | DC_n78A_7A | 23 | +2/-3 |
|  |  | DC_7A_n79A |  | 23 | +2/-3 |
|  |  | DC_8A_n2A |  | 23 | +2/-3 |
|  |  | DC_8A_n20A DC_20A_n8A |  | 23 | +2/-3 |
|  |  | DC_8A_n28A DC_28A_n8A | DC_n28A_8A |  |  |
| CA_n8A-n34A |  | DC_8A_n34A |  | 23 | +2/-3 |
| CA_n8A-n39A |  | DC_8A_n39A |  | 23 | +2/-3 |
| CA_n8A-n40A |  | DC_8A_n40A |  | 23 | +2/-3 |
| CA_n8A-n41A |  | DC_8A_n41A DC_8A_n81A_ULSUP-TDM_n41 | DC_n41A_8A | 23 | +2/-3 |
| CA_n8A-n77A |  | DC_8A_n77A | DC_n77A_8A | 23 | +2/-3 |
| CA_n8A-n78A |  | DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A | DC_n78A_8A | 23 | +2/-3 |
| CA_n8A-n79A |  | DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A |  | 23 | +2/-3 |
|  |  | DC_11A_n1A |  | 23 | +2/-3 |
|  |  | DC_11A_n3A |  | 23 | +2/-3 |
|  |  | DC_11A_n28A |  | 23 | +2/-3 |
|  |  | DC_11A_n41A |  | 23 | +2/-3 |
|  |  | DC_11A_n77A |  | 23 | +2/-3 |
|  |  | DC_11A_n78A |  | 23 | +2/-3 |
|  |  | DC_11A_n79A |  | 23 | +2/-3 |
|  |  | DC_12A_n7A |  | 23 | +2/-3 |
|  |  | DC_12A_n25A |  | 23 | +2/-3 |
| CA_n12A-n30A |  | DC_12A_n30A |  | 23 | +2/-3 |
|  |  | DC_12A_n38A |  | 23 | +2/-3 |
|  |  | DC_12A_n41A |  | 23 | +2/-3 |
| CA_n12A-n66A |  | DC_12A_n66A DC_66A_n12A |  | 23 | +2/-3 |
|  |  | DC_12A_n71A |  | 23 | +2/-3 |
| CA_n12A-n77A |  | DC_12A_n77A |  | 23 | +2/-3 |
|  |  | DC_12A_n78A |  | 23 | +2/-3 |
|  |  | DC_13A_n2A |  | 23 | +2/-3 |
|  |  | DC_13A_n5A |  | 23 | +2/-3 |
|  |  | DC_13A_n7A |  | 23 | +2/-3 |
| CA_n13A-n25A |  | DC_13A_n25A |  | 23 | +2/-3 |
|  |  | DC_13A_n48A |  | 23 | +2/-3 |
| CA_n13A-n66A |  | DC_13A_n66A |  | 23 | +2/-3 |
|  |  | DC_13A_n71A |  | 23 | +2/-3 |
| CA_n13A-n77A |  | DC_13A_n77A |  | 23 | +2/-3 |
|  |  | DC_13A_n78A |  | 23 | +2/-3 |
| CA_n14A-n30A |  | DC_14A_n30A |  | 23 | +2/-3 |
| CA_n14A-n66A |  | DC_14A_n66A |  | 23 | +2/-3 |
| CA_n14A-n77A |  | DC_14A_n77A |  | 23 | +2/-3 |
| CA_n18A-n28A |  | DC_18A_n28A |  | 23 | +2/-3 |
| CA_n18A-n41A |  | DC_18A_n41A |  | 23 | +2/-3 |
| CA_n18A-n74A |  |  |  | 23 | +2/-3 |
| CA_n18A-n77A |  | DC_18A_n77A |  | 23 | +2/-3 |
| CA_n18A-n78A |  | DC_18A_n78A |  | 23 | +2/-3 |
|  |  | DC_18A_n79A |  | 23 | +2/-3 |
|  |  | DC_19A_n1A |  | 23 | +2/-3 |
|  |  | DC_19A_n77A |  | 23 | +2/-3 |
|  |  | DC_19A_n78A |  | 23 | +2/-3 |
|  |  | DC_19A_n79A |  | 23 | +2/-3 |
| CA_n20A-n28A |  | DC_20A_n28A DC_20A_n83A | DC_n28A_20A | 23 | +2/-3 |
|  |  | DC_20A_n38A |  | 23 | +2/-3 |
|  |  | DC_20A_n41A |  | 23 | +2/-3 |
|  |  | DC_20A_n50A |  | 23 | +2/-3 |
|  |  | DC_20A_n51A |  | 23 | +2/-3 |
|  |  | DC_20A_n77A |  | 23 | +2/-3 |
| CA_n20A-n78A |  | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
|  |  | DC_21A_n1A |  | 23 | +2/-3 |
|  |  | DC_21A_n28A |  | 23 | +2/-3 |
|  |  | DC_21A_n77A |  | 23 | +2/-3 |
|  |  | DC_21A_n78A |  | 23 | +2/-3 |
|  |  | DC_21A_n79A |  | 23 | +2/-3 |
| CA_n24A-n41A |  |  |  | 23 | +2/-3 |
| CA_n24A-n48A |  |  |  | 23 | +2/-3 |
| CA_n24A-n77A |  |  |  | 23 | +2/-3 |
| CA_n25A-n38A |  | DC_25A_n41A |  | 23 | +2/-3 |
| CA_n25A-n41A |  |  |  | 23 | +2/-3 |
| CA_25A-n48A |  | DC_48A_n25A |  | 23 | +2/-3 |
| CA_n25A-n66A |  | DC_66A_n25A |  | 23 | +2/-3 |
| CA_n25A-n77A |  | DC_25A_n77A |  | 23 | +2/-3 |
| CA_n25A-n78A |  | DC_25A_n78A |  | 23 | +2/-3 |
|  |  | DC_26A_n25A |  | 23 | +2/-3 |
|  |  | DC_26A_n41A |  | 23 | +2/-3 |
| CA_n26A-n66A |  |  |  | 23 | +2/-3 |
| CA_n26A-n70A |  |  |  | 23 | +2/-3 |
|  |  | DC_26A_n77A |  | 23 | +2/-3 |
|  |  | DC_26A_n78A | DC_n78A_26A | 23 | +2/-3 |
|  |  | DC_26A_n79A |  | 23 | +2/-3 |
|  |  | DC_28A_n5A |  | 23 | +2/-3 |
| CA_n28A-n34A |  |  | DC_n28A_34A | 23 | +2/-3 |
| CA_n28A-n39A |  |  | DC_n28A_39A | 23 | +2/-3 |
| CA_n28A-n40A |  | DC_28A_n40A | DC_n28A_40A | 23 | +2/-3 |
| CA_n28A-n41A | DC_n28A-n41A | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A DC_41A_n28A |  | 23 | +2/-3 |
| CA_n28A-n46A | DC_n28A-n46A |  |  | 23 | +2/-3 |
| CA_n28A-n50A |  | DC_28A_n50A |  | 23 | +2/-3 |
|  |  | DC_28A_n51A |  | 23 | +2/-3 |
|  |  | DC_28A_n66A DC_66A_n28A |  | 23 | +2/-3 |
| CA_n28A-n74A |  |  |  | 23 | +2/-3 |
| CA_n28A-n77A | DC_n28A-n77A | DC_28A_n77A |  | 23 | +2/-3 |
| CA_n28A-n78A | DC_n28A-n78A | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
| CA_n28A-n79A | DC_n28A-n79A | DC_28A_n79A |  | 23 | +2/-3 |
| CA_n34A-n79A |  |  |  | 23 | +2/-3 |
| CA_n30A-n66A |  | DC_30A_n66A DC_66A_n30A |  | 23 | +2/-3 |
| CA_n30A-n77A |  | DC_30A_n77A |  | 23 | +2/-3 |
| CA_n34A-n40A |  |  |  | 23 | +2/-3 |
| CA_n34A-n41A |  |  | DC_n41A_34A | 23 | +2/-3 |
|  |  | DC_38A_n8A |  | 23 | +2/-3 |
|  |  | DC_38A_n28A |  | 23 | +2/-3 |
| CA_n38A-n66A |  | DC_66A_n38A |  | 23 | +2/-3 |
| CA_n38A-n78A |  | DC_38A_n78A |  | 23 | +2/-3 |
|  |  | DC_38A_n79A |  | 23 | +2/-3 |
| CA_n39A-n40A |  | DC_39A_n40A |  | 23 | +2/-3 |
| CA_n39A-n41A |  | DC_39A_n41A | DC_n41A_39A | 23 | +2/-3 |
|  |  | DC_39A_n78A |  | 23 | +2/-3 |
| CA_n39A-n79A |  | DC_39A_n79A |  | 23 | +2/-3 |
| CA_n40A-n41A |  | DC_40A_n41A | DC_n41A_40A | 23 | +2/-3 |
| CA_n40A-n77A |  | DC_40A_n77A |  | 23 | +2/-3 |
| CA_n40A-n78A |  | DC_40A_n78A |  | 23 | +2/-3 |
| CA_n40A-n79A |  | DC_40A_n79A |  | 23 | +2/-3 |
| CA_n41A-n48A |  |  |  | 23 | +2/-3 |
| CA_n41A-n50A |  |  |  | 23 | +2/-3 |
| CA_n41A-n66A |  | DC_66A_n41A |  | 23 | +2/-3 |
| CA_n41A-n70A |  |  |  | 23 | +2/-3 |
| CA_n41A-n71A |  | DC_71A_n41A |  | 23 | +2/-3 |
| CA_n41A-n74A |  |  |  | 23 | +2/-3 |
| CA_n41A-n77A | DC_n41A-n77A | DC_41A_n77A |  | 23 | +2/-3 |
| CA_n41A-n78A | DC_n41A-n78A | DC_41A_n78A |  | 23 | +2/-3 |
| CA_n41A-n79A |  | DC_41A_n79A |  | 23 | +2/-3 |
|  |  | DC_42A_n1A |  | 23 | +2/-3 |
|  |  | DC_42A_n3A |  | 23 | +2/-3 |
|  |  | DC_42A_n28A |  | 23 | +2/-3 |
|  |  | DC_42A_n51A |  | 23 | +2/-3 |
|  |  | DC_42A_n77A |  | 23 | +2/-3 |
|  |  | DC_42A_n78A |  | 23 | +2/-3 |
|  |  | DC_42A_n79A |  | 23 | +2/-3 |
| CA_n46A-n48A | DC_n46A-n48A |  |  | 23 | +2/-3 |
| CA_n46A-n48B | DC_n46A-n48B |  |  | 23 | +2/-3 |
| CA_n46A-n78A | DC_n46A-n78A |  |  | 23 | +2/-3 |
|  |  | DC_48A_n12A |  | 23 | +2/-3 |
| CA_n48A-n66A | DC_n48A-n66A | DC_48A_n66A DC_66A_n48A |  | 23 | +2/-3 |
| CA_n48A-n70A | DC_n48A-n70A |  |  | 23 | +2/-3 |
| CA_n48A-n71A | DC_n48A-n71A | DC_48A_n71A DC_71A_n48A |  | 23 | +2/-3 |
| CA_n48A-n96A | DC_n48A-n96A |  |  | 23 | +2/-3 |
| CA_n48B-n96A | DC_n48B-n96A |  |  | 23 | +2/-3 |
| CA_n48A-n96B |  |  |  | 23 | +2/-3 |
| CA_n50A-n78A |  |  |  | 23 | +2/-3 |
|  |  | DC_66A_n46A |  | 23 | +2/-3 |
| CA_n66A-n71A |  | DC_66A_n71A DC_71A_n66A |  | 23 | +2/-3 |
| CA_n66A-n77A | DC_n66A-n77A | DC_66A_n77A |  | 23 | +2/-3 |
| CA_n66A-n78A |  | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |  | 23 | +2/-3 |
| CA_n70A-n71A |  |  |  | 23 | +2/-3 |
| CA_n70A-n78A |  |  |  | 23 | +2/-3 |
|  |  | DC_71A_n38A |  | 23 | +2/-3 |
| CA_n71A-n77A |  |  |  | 23 | +2/-3 |
| CA_n71A-n78A |  | DC_71A_n78A |  | 23 | +2/-3 |
| CA_n74A-n77A |  |  |  | 23 | +2/-3 |
| CA_n74A-n78A |  |  |  | 23 | +2/-3 |
| CA_n77A-n79A | DC_n77A-n79A |  |  | 23 | +2/-3 |
| CA_n78A-n79A |  |  |  | 23 | +2/-3 |
| CA_n78A-n92A |  |  |  | 23 | +2/-3 |

- 7.2.2	Spurious emission for UE-to-UE coexistence

Generally, if CA_nA-nB, DC_A_nB (DC_A_nD), DC_B_nA (DC_B_nC), DC_nB_A, DC_nA_B have same spurious emission requirements for UE to UE coexistence, there is no need to test the spurious emission requirements for UE to UE coexistence for each UL configuration again and again. Once one of these UL configurations is verified, the other UL configurations for different feature in same band combination can be considered as being capable of meeting these requirements.

NOTE: Band nC and nD are the corresponding SUL bands with same UL frequency range of band nA and nB, e.g. SUL band n80 has same UL frequency range of band n3.

- 7.3	Similarity and Dependency of Rx RF requirements for different features on the same band combination

- 7.3.1	REFSENS exception due to harmonic/harmonic mixing interference for inter-band combinations (two bands)

For reference sensitivity exception due to harmonic/harmonic mixing specified for ENDC/NEDC band combinations, it’s recommended to follow the same principles that we have agreed in NR CA BCS4 WI in WF R4-2210565. The specific guidelines for NR CA, ENDC/NEDC band combinations are shown below.

1) Guidelines for the DL band configuration:

- For both Tx harmonic direct hit and for Rx harmonic mixing, a maximum of two test points can be specified:

  - one test point to capture the lowest DL channel bandwidth (CBW) MSD,

  - one test point to capture the highest DL channel bandwidth (CBW) MSD. For this second test points, other DL configurations are not precluded.

- For near miss harmonic interference, one MSD test point can be considered to specify the lowest DL CBW MSD.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the clause that UE channel bandwidth per operating band for ENDC/NEDC or the BCS configuration tables of clause 5.5A from TS 38.101-1 for CA..

2) Guidelines for the UL band configuration.

For all MSD test points:

- Lcrb and RBstart specify the UL RB allocation which ensures that the DL affected band RX CBW entirely overlaps the harmonic interference power spectral density (PSD),

- The UL band is configured with the lowest UL CBW and the lowest SCS that can accommodate the specified UL Lcrb. For example, if the Lcrb is 25RB, the UL configuration should be 5MHz CBW, SCS15kHz.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the clause that UE channel bandwidth per operating band for ENDC/NEDC or the BCS configuration tables of clause 5.5A from TS 38.101-1 for CA..

3) Guidelines for the UL/DL harmonic order:

- The generic notation “ULn/DLm” should be used, where “m” indicates the DL band harmonic order, “n” indicates the UL band harmonic order.

- For Tx harmonic interference: the collision type is specified either as “direct-hit” or as “near-miss”. For example, “UL2/DL1 direct-hit” is used to indicate a DL band MSD test point due to direct hit of the UL band 2nd harmonic,

- For Rx harmonic mixing: the collision type is not needed. For example, “UL3/DL2” is used to indicate a DL band MSD test point is due to 2nd harmonic Rx mixing with third UL Tx harmonic.

4) There is no need to specify the UL band and the DL band carrier-centre frequencies as what we specified in clause 7.3A.4 from TS 38.101-1. For ENDC/NEDC band combinations, new MSD table format for Tx and Rx harmonic can refer to Table 7.3A.4-1 and Table 7.3A.4-4 from TS 38.101-1 respectively.

- 7.3.2	REFSENS exception due to cross band isolation interference for inter-band combinations (two bands)

For reference sensitivity exception due to cross band isolation specified for ENDC band combinations, it’s recommended to follow the same principles that we have agreed in NR CA BCS4 WI in WF R4-2210565. The specific guidelines for ENDC/NEDC band combinations are shown below.

1) Guidelines for the DL band configuration:

- The affected DL band carrier centre-frequency shall be configured closest to the UL band.

- Up to two test points can be specified:

  - the first test point should capture the lowest DL channel bandwidth (CBW) MSD,

  - the second test point is optional and may capture the highest DL channel bandwidth (CBW) MSD.

- It is not precluded that the second test point is specified for other DL CBW or DL carrier frequency to accommodate exceptions due to say, regional spectrum allocations specificities, or proponent concerns on specific CBW of interest etc. For example, for CA_n18-n28, both the 1st and the 2nd MSD test point may be specified for the smallest n28 DL CBW of 5MHz, but it is currently discussed that:

  - the first MSD test point may specify the MSD for the n28B DL frequency range,

  - while the second MSD test point may specify the MSD for the n28A frequency range.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the clause that UE channel bandwidth per operating band for ENDC/NEDC or the BCS configuration tables of clause 5.5A from TS 38.101-1 for CA.

2) Guidelines for the UL band configuration and for all MSD test points,

- For the first test point which evaluates the MSD for the lowest DL CBW, the UL band should be configured with the highest supported CBW. This ensures the UL band lowest IMD order has a maximum reach towards the DL affected band. This is key for inter-band CA/ENDC/NEDC combinations where the frequency separation distance is small relative to the UL CBW, for example in CA_n5-n28, CA_n1-n3 etc..

- For the second test point, the choice of UL/DL CBW remains open to account for exceptions or regional concerns, or proponents request.

- The UL SCS should be the lowest SCS that can be supported for the selected UL CBW. For example, if the UL CBW is 50MHz, then SCS15kHz should be specified.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the clause that UE channel bandwidth per operating band for ENDC/NEDC or the BCS configuration tables of clause 5.5A from TS 38.101-1 for CA.

- UL Lcrb is that specified in Table 7.3.2-3 from TS 38.101-1 for NR band and Table 7.3.1-2 from TS 36.101 for LTE band (UL configuration for UL Band REFSENS) for the corresponding UL band CBW.

- UL RBstart should ensure the UL RBs are positioned closest to the DL affected band,

- The UL band carrier centre frequency should be configured closest to the affected DL band.

3) Guidelines for cross-band interference source and MSD evaluation

- The interference source can be either “ACLR1”, “ACLR2”, or greater than ACLR2 (“>ACLR2”),

- For “ACLR1”, or “ACLR2” test points:

  - MSD may be neglected if the order of the IMD product of the modulated RB and its image that overlaps the affected DL band is greater than [9],

  - For RB/Image IMD order less or equal to [7], the MSD needs to be analysed taking into consideration RF-FE complexity (quadplexer, triplexer, etc..),

- For “>ACLR2” test points, aka “flat-band noise cases” / large separation distances:

  - MSD may need to be evaluated if the Tx noise rejection in the DL affected band is less than [40] dB. The underlying assumption is that the flat Tx noise PSD is -130dBm/Hz and DL affected CBW is 5MHz.

- 7.3.3	REFSENS exception due to inter-modulation distortion for inter-band combinations (two bands)

- 7.3.4	REFSENS requirements without any degradation for inter-band combinations (two bands)

One band combination can be chosen to verify REFSENS requirements for band combinations DL_nA-nB_UL_nA-nB / DL_B_nA_UL_B_nA / DL_A_nB_UL_A_nB / DL_nB_A_UL_nB_A / DL_nA_B_UL_nA_B which don’t have any MSD requirements, as a result of reducing test burden. For some special cases that band combinations with different features in same frequency range have different delta Rib requirements, UE need to meet the stricter delta Rib requirements for reducing test burden. The reason is that the difference between different delta Rib requirements is neglected, but the same Rx RF implementation is used to achieve these band combinations.

- <<End of TP>>