# 3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216117

Electronic Meeting, Oct. 10th – 19th, 2022

Source: 	vivo

Title: 	Analysis on improve MSD

Agenda Item:	6.6.4.1

Document for:	Discussion

Introduction

In RAN#96 meeting, the WI “Further RF requirements enhancement for NR frequency range 1” was agreed in [1]. In RAN4#104-e, a comprehensive discussion paper was submitted in [2]. The overall discussion process was documented in the summary [3], and a WF was also agreed in [4].

In this paper, some of the parameters used for traditional MSD analysis were discussed based on measurement results. Some other related issues were also discussed.

Discussion

The classic way of MSD analysis is step by step analysis based on typical architecture and a set of parameters. The interference would be calculated for some key paths, and be added to have a final MSD value. Some of the early LTE examples can reference to [5]. Though not officially confirmed, the traditional way of analyzing MSD seems still would play a part in study the MSD. In the last meeting, one comprehensive paper was submitted in [6], in which a very detailed study was done for the required improvement on various parameters in case a certain number of MSD improvement is needed.

In addition, in last meeting’s WF [4], the following contents were incorporated:

RF component assumptions and methods for lower MSD analysis

< WF Agreed in GTW2 >:

The following assumptions and methods can be considered for the MSD study in next meeting.

- Antenna isolation: 10~20dB

- PCB isolation: >/= 60dB,

- Other parameters, e.g. RF component linearity, diplexer/duplexer/filter rejection, etc. up to the choice of companies

- Other values for parameters are not precluded.

- For the MSD study, the parameters used in the analysis should be provided

- MSD reduction via UL power back-off is FFS, depending on further inputs and clarification in next meeting

Note: It is understood that practical implementations usually make different design trade-offs. The optimal values are unlikely to be achieved for all RF parameters and/or all band combinations simultaneously. And the feasibility for PCB isolation values can be discussed.

Here we also checked some measurement data for some of the key parameters here.

Antenna isolation:

Some models of commercial UE with EN-DC B3+n78 was checked for the antenna isolation data measured by network analyser between the antenna used for b3 and n78, and typical results are mostly ranged from 13~17dB. Considering it is quite similar for CA_n3-n78, the result could be served as a reference. 10dB is still a valid number for minimum requirement, and a slight enhanced value can also be considered based on the results.

Actually, in most cases, the antenna isolation do not play a key role as the MSD analysis.

Observation 1: Typical antenna isolation between n3 and n78 can be 13~17 dB. 10dB is still a valid number for minimum requirement, and an enhanced value such as [15]dB can also be considered based on the results.

PCB isolation:

This is one of the key parameters for the analysis, and in many cases,  this would be a gating factor as illustrated in [6]. However, it is usually not that easy to do direct measurements, and usually would only be optimised in case sever interference was observed. E.g. shields have to be removed to do some direct measurements, but removing shields sometimes would have a significant degradation to the isolation effect.

Still, based on some measurement history, 60dB is a not a good implementation between the aggressor PA out to victim LNA input, and 70~80dB can be feasible.

However, there are still many other factors in an actual UE implementation would face compared to the traditional MSD analysis. For example, the power supple VCC in the real UE sometimes would also have significant 2nd order harmonics interference in the real design, but this was never considered in the analysis process. There are even other interference sources from other components of UE other than the RF part, e.g. camera, would also become an interference source. So this kind of analysis do not need very much accurate.

Observation 2: A range of 70-80 dB PCB isolation can be considered based on implementation, but it may be hard and also unnecessary to be accurate, since there are measurements problem and also other unaccounted interference source and path in real implementation.

Others:

After checking the measurement of one PA, we found that the 2nd order harmonics power is -20dBm when operating at B3 output power 28.5dB in Tx out. This means a 48.5dB for PA H2 suppression, which is quite aligned with some of the previous contribution. However, the VCC also showed a -19.7dBm at the same harmonic frequency, which is not assumed anywhere before, and this is also in-line with previous observations that there are also other unaccounted interference sources.

Observation 3: B3 PA H2 up to 48dB is aligned with some implementation, but there are other unaccounted interference sources.

Based on these conditions, more analysis can be done based on traditional analysis, but these can only be rough references and the final effect on MSD values would be much more complicated.

Proposal: The parameters used for “traditional” MSD analysis can be based on previous observations which is also summarized here:

| Parameter | Values |
| --- | --- |
| Antenna Isolation | [15] dB |
| PCB Isolation | [70~80] dB |
| B3 PA H2 | [48] dB |

However, this MSD analysis can only be a reference. The final effect on MSD values would be much more complicated.

Conclusion

In this paper, some observations and proposal are provided.

Observation 1: Typical antenna isolation between n3 and n78 can be 13~17 dB. 10dB is still a valid number for minimum requirement, and an enhanced value such as [15]dB can also be considered based on the results.

Observation 2: A range of 70-80dB PCB isolation can be considered based on implementation, but it may be hard and also unnecessary to be accurate, since there are measurements problem and also other unaccounted interference source and path in real implementation.

Observation 3: B3 PA H2 up to 48dB is aligned with some implementation, but there are other unaccounted interference sources.

Proposal: The parameters used for “traditional” MSD analysis can be based on previous observations which is also summarized here:

| Parameter | Values |
| --- | --- |
| Antenna Isolation | [15] dB |
| PCB Isolation | [70~80] dB |
| B3 PA H2 | [48] dB |

However, this MSD analysis can only be a reference. The final effect on MSD values would be much more complicated.

- References

[1] RP-221496, Further RF requirements enhancement for NR frequency range 1 (FR1), RAN#96

[2] R4-2212805, General analysis on improve MSD, vivo, RAN4#104-e

[3] R4-2214243, Email discussion summary for [104-e][132] FR1_enh2, Moderator (Huawei, HiSilicon) , RAN4#104-e

[4] R4-2214452, WF on study for lower MSD, Huawei, HiSilicon, RAN4#104-e

[5] 36.860-12, LTE-Advanced dual uplink inter-band Carrier Aggregation (CA)

[6] R4-2212093, Possible approaches to improve MSD, Nokia, RAN4#104-e