# 3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216119

Electronic Meeting, Oct. 10th – 19th, 2022

Source: 	vivo

Title: 	Discussion on UE RF requirements for Tx Switching up to 3 or 4 bands

Agenda Item:	6.19.2

Document for:	Discussion

Introduction

The WI “Multi-carrier enhancements for NR” has been agreed in RAN#94-e and the latest update was approved in RAN#97e in [2].

In RAN4#104-e, RAN4 received a LS [2] from RAN1 regarding the complexity / switching period when extends the switching behaviour to 3 or 4 bands. After discussion, many conclusions were reached and a reply LS [3] has been approved and sent to RAN1. A WF for single TAG has also been agreed in [4] and the discussion was documented in the thread summary in [5].

In this contribution, continue discussion was done for single TAG case.

Discussion

Switching period for Tx switching across 3/4 bands

In the WF and LS, for the set of values and granularity for Tx switching period, the following agreements have been reached and documented in the LS:

On the length of switching period:

- For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL.

- The length of switching period is applied per band pair for each band combination.

- For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17.

  - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

- For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17.

There is one remaining issue whether a different value can be reported for the specific band pair in Rel-18 compared to Rel-16/17, as in the WF:

  - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching.

  - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

To our understanding, option 1 would be simpler but not that flexible to reflect the possible new schemes with the introduction of 3/4 bands. In addition, there are still possible factors that could lead to different numbers. The current situation is that many schemes and analyses are now discussed in RAN1, and there is a risk to make early restrictive agreements. So we propose to either choose option 2 or continue postponing this issue until more information is received from RAN1.

Proposal 1: Either select option 2 (different values can be reported for each band pair for Rel-18 and Rel-16/17) or wait until more information comes from RAN1’s ongoing study.

Impact from switching of one Tx chain on the other Tx chain

For the case that switching happened between multiple bands, the following issue has been discussed and GTW agreement has been reached in last RAN4 meeting in [5]:

- Issue description

  - When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”), is the other Tx chain is expected or not expected to be used for transmission on band C during the switching period?

Note: this is a new scenario/pattern not discussed in Rel-16/17 (NTT DOCOMO, China Telecom)

    - Option 1: Expected

    - Option 2: Not expected

GTW Agreement:

  - As baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

  - RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption.

As has been discussed, the option 1 would bring some “unsynchronized” scenario between different band pairs. Even regard as a kind of “optional advanced features”, this would bring unnecessary complexity, with quite a doubtful gain.

Proposal 2: Do not define other optional features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period.

For concurrent UL transmission on 2 bands

For single TAG case, the following agreements have been sent to RAN1:

For UL Tx switching across 3 and 4 bands, the support of concurrent UL transmission on 2 (out of 3 or 4) bands at least requires UL CA support on the corresponding band pair(s) by the UE.

This agreement has defined a prerequisite condition, for one band pair to support concurrent UL transmission, it has to support UL CA. From RAN4’s point, we may not have more prerequisites in RAN4, but other working groups may also have their own prerequisite conditions, e.g. whether “dualUL” be supported or not.

However, a sufficient condition is still useful and it would be beneficial to somehow define one. Based on this thinking, it is still proposed to somehow set a sufficient condition for this concurrent UL transmission at least in RAN4. RAN1 is also having some discussion on this issue.

Proposal 3: Further discuss whether UL support on the corresponding band pair(s) could be set to a sufficient condition of support of concurrent UL transmission, at least from RAN4’s point of view.

Number of bands supporting 2Tx/UL-MIMO

This is being discussed in RAN1, and the discussion in RAN4 can wait.

RF requirements

The following points have been reached in the

  - No need to define RF requirements for UL CA with UL simultaneous transmission on 3 and 4 bands in the WI.

  - For the next meeting, encourage analysis/identification of the RAN4 RF requirements needed for this WI.

It should be firstly noted that RAN4 requirements only consider the “Switched UL” option, and do not consider “dual UL” option as defined in RAN, that needs simultaneous transmission in multiple carriers. As one of the example figures below and the corresponding descriptions:

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Observation 1: RAN4 requirements currently only consider “switched UL” case, and doesn’t consider “dualUL” option which is defined in RAN1.

In RAN1 spec, more cases were defined, and the related behaviour are also defined (No transmission in switching period). It seems that the current way can be reused in Rel-18.

Proposal 4: For Rel-18, RAN4 can keep the REl-16/17 way that only define requirements “switched UL” case.

The current RAN4 requirements are as follows:

6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

6.3A.3.3.1	General

6.3A.3.3.2	Time mask for switching between two uplink carriers

6.3A.3.3.3	Time mask for switching between two uplink carriers with two transmit antenna connectors

6.3A.3.3.4	Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors

6.3A.3.3.5	Time mask for switching between two uplink bands with two transmit antenna connectors

For the requirements structure, in case only one band pair is involved, those requirements can be basically reused. It can be considered that those clauses would be reused, and some clarification can be added in the general section, e.g., clarification in general clause may be added to say that those requirements are also applied to a band pair.

Proposal 5: In case only one band pair would be involved during Tx Switching, the current requirements clauses can be reused with some clarifications.

In case multiple band pairs were involved in the Tx Switching, new requirements clauses may be needed. Since the current requirements may not be directly applied for multiple pair cases. One of the basic principles can be longer switching period can be chosen if different requirements are applied between different band pairs. The details can be FFS.

In addition, RAN1 is currently doing a series of complexity reduction study, which may introduce restrictions that also have an impact of RAN4 requirements. T

Proposal 6: In case multiple band pairs are involved during the Tx switching process, new requirements clauses may be added, which would also be impacted by RAN1’s undergoing complexity reduction discussion.

Conclusion

In this paper, continue discussion was done for Tx Switching up to 3 or 4 bands for single TAG case.

Proposal 1: Either select option 2 (different values can be reported for each band pair for Rel-18 and Rel-16/17) or wait until more information comes from RAN1’s ongoing study.

Proposal 2: Do not define other optional features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period.

Proposal 3: Further discuss whether UL support on the corresponding band pair(s) could be set to a sufficient condition of support of concurrent UL transmission, at least from RAN4’s point of view.

Observation 1: RAN4 requirements currently only consider “switched UL” case, and doesn’t consider “dualUL” option which is defined in RAN1.

Proposal 4: For Rel-18, RAN4 can keep the REl-16/17 way that only define requirements “switched UL” case.

Proposal 5: In case only one band pair would be involved during Tx Switching, the current requirements clauses can be reused with some clarifications.

Proposal 6: In case multiple band pairs are involved during the Tx switching process, new requirements clauses may be added, which would also be impacted by RAN1’s undergoing complexity reduction discussion.

- References

[1] RP-222251, Multi-carrier enhancements for NR, RAN#97e

[2] R1-2205502, LS on UL Tx switching across 3 or 4 bands in Rel-18, RAN1#109-e

[3] R4-2214464, Reply LS on UL Tx switching across 3 or 4 bands, China Telecom, RAN4#104-e

[4] R4-2214463, WF on UL Tx switching across 3/4 bands with single TAG, China Telecom, RAN4#104-e

[5] R4-2214249, Email Discussion Summary for [104-e][138] NR_MC_enh, Moderator (China Telecom), RAN4#104-e