# 3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216135

# Electronic Meeting, 10th - 19th October, 2022

Source: 	vivo

Title: 	Further discussion on interference modelling for full duplex from UE aspect

Agenda Item:	6.17.2.2.1

Document for:	Approval

Introduction

In the last meeting, the discussion for SI NR_Duplex_Evo started in RAN4, focusing on interference modelling for self-interference at gNB, co-channel inter-subband and adjacent channel gNB-gNB/UE-UE CLI. Several high-level agreements were achieved for interference modelling [1] and a reply LS providing partial answers on these aspects [2] was sent out to RAN1. In this contribution, we provide our further thoughts on interference modelling for full duplex from UE aspect.

Discussion

Co-channel inter-subband CLI

Co-channel inter-subband UE-UE CLI

In the last meeting, these candidate Tx/Rx model for co-channel inter-subband UE-UE CLI model were proposed as follows. It is still FFS whether ACLR/ACS can be applied for co-channel inter-subband UE-UE CLI.

- TX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2

  - In band emission as starting point

  - FFS is not precluded for other candidates such as ACLR

- RX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2

  - Maximum input power as threshold based on above specification

  - FFS is not precluded for other candidates such as ACS, ICI, and estimated RX model based on legacy UE.

From our understanding, ACLR/ACS requirement are used in channel bandwidth level, however not in subband level for UE. For subband Tx transmission, whether ACLR can be used depends on whether Tx subband can be configured as UE channel bandwidth. Similarly, for subband Rx reception, whether ACS can be used depends on whether Rx subband can be configured as UE channel bandwidth. The key is whether a subband can be configured as UE channel bandwidth.

For UE channel bandwidth definition, these points are captured as follows:

- From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS.

- From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

- The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

From the definition of UE channel bandwidth, UE carrier bandwidth is configured flexible as long as it is within the BS channel bandwidth. Also, the UE carrier bandwidth can be configured with the width of subband. These following two cases are possible:

- Case 1: UE CHBW is equal to BS CHBW, the subband is within UE CHBW

- Case 2: UE CHBW is less than BS CHBW, the subband is equal to UE CHBW

For Case 1, UE and BS share the same channel bandwidth. UE only transmit in the Subband 1. In this situation, UE only transmits partial RF signal within the UE channel bandwidth, and In-band emission model can be used for Tx model.

Figure 1. IBE for Tx Model (Case 1)

For Case 2, UE CHBW is within BS CHBW and BS subband is equal to UE CHBW. In this case, UE transmits in full RBs of the UE CHBW, no need to consider UE IBE. The leakage can be modeled by UE ACLR.

Figure 2. ACLR for Tx model (Case 2)

For Rx model, the situation is also different for Case 1 and Case 2. For Case 1, UE receives the whole channel including the subband, without any in-channel selectivity. In this case, maximum input power is used as threshold whether or not blocking happens. However, for case 2, UE receives the whole UE channel, which is equal to the UE subband. For the interference on the adjacent subband, adjacent channel selectivity applies.

In the last meeting, we agreed to use existing UE RF requirements to estimate UE performance and if needed extrapolating them for system level studies [1]. In the discussion for UE channel bandwidth configuration case 1 and case2, they all consider the existing UE RF requirements without more complexity to UE architecture. However, with different UE channel bandwidth configuration, different Tx/Rx model may be applied for each case.

Proposal 1: To discuss whether a BS subband can be configured as UE CHBW from UE’s perspective.

Proposal 2. To consider the following as Tx/Rx model for co-channel inter-subband interference modelling.

| Configuration | UE Tx Model | UE Rx Model |
| --- | --- | --- |
| - Case 1: UE CHBW is equal to BS CHBW, the subband is within UE CHBW | - IBE | - Maximum input level |
| - Case 2: UE CHBW is less than BS CHBW, the subband is equal to UE CHBW | - ACLR | - ACS |

Conclusion

This contribution discusses interference modelling issues for co-channel inter-subband UE-UE CLI for full duplex. The following proposals are made:

Proposal 1: To discuss whether a subband can be configured as UE CHBW from UE’s perspective.

Proposal 2. To consider the following as Tx/Rx model for co-channel inter-subband interference modelling.

| Configuration | UE Tx Model | UE Rx Model |
| --- | --- | --- |
| - Case 1: UE CHBW is equal to BS CHBW, the subband is within UE CHBW | - IBE | - Maximum input level |
| - Case 2: UE CHBW is less than BS CHBW, the subband is equal to UE CHBW | - ACLR | - ACS |

- References

[1] R4-2214377, WF on feasibility study from RF perspective, Samsung, RAN4#104-e.

[2] R4-2214376, Reply LS on interference modelling for duplex evolution, Samsung, RAN4#104-e.