# 3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216137

# Electronic Meeting, 10th - 19th October, 2022

Source: 	vivo

Title: 	Further discussion on RF requirement impacts for 700800900MHz CA band combinations

Agenda Item:	6.2.1

Document for:	Approval

Introduction

In the last meeting, we started the discussion on the feasibility and solutions to enable CA_n8-n20-n28, CA_n5-n8, CA_n5-n28. These following aspects related to low band CA were discussed, RF requirement framework, spectrum restriction, UE antenna and RF architecture, CA configurations, impacted RF parameters, feasibility issues [1]. In this contribution, we will further discuss potential RF requirement impacts for low band CA combinations considering smartphone UE type.

Discussion

For low band CA band combinations SI, the following agreements were reached for CA_n5-n8, CA_n5-n28, CA_n8-n20-n28, as shown in Table 1.

Table 1. Summary on low band CA band combinations

| Issues | CA_n5-n8 | CA_n5-n28 | CA_n8-n20-n28 |
| --- | --- | --- | --- |
| Frequency Restriction | Frequency 1 (800MHz) UL:824MHz - 835MHz DL:869MHz - 880MHz Frequency 2 (900MHz) UL: 904MHz - 915MHz DL:949MHz - 960MHz | N.A | Use the follow frequency ranges for further discussion for n28 spectrum restriction to support CA_n8-n20-n28. UL: 703MHz~733MHz DL: 758MHz~788MHz |
| Antenna&RF | - 2-antenna One Main Tx/Rx Antenna+One Diversity Rx Antenna - 3-antenna Two Main Tx/Rx Antenna+One Diversity Rx Antenna | - 2-antenna One Main Tx/Rx Antenna+One Diversity Rx Antenna - 3-antenna Two Main Tx/Rx Antenna+One Diversity Rx Antenna | - 2-antenna One Main Tx/Rx Antenna+One Diversity Rx Antenna - 3-antenna Two Main Tx/Rx Antenna+One Diversity Rx Antenna - 4 antenna Three Main Tx/Rx Antenna+One Diversity Rx Antenna |
| CHBW config | n5: 5, 10 n8: 5, 10 | n5: 5, 10, 15, 20 n28: 5, 10, 15, 20, 25, 30 | n8: 5, 10, 15 n20: 5, 10, 15, 20 n28: 5, 10, 15, 20 |
| UL configuration | The CBW of UL configuration CA_n5-n8 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point. | The CBW of UL configuration CA_n5-n28 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point. | UL configuration of CA_n8-n20, CA_n8-n28, CA_n20-n28 are supported for CA_n8-n20-n28. The CBW assumption for UL and DL are the same. The same RB allocation with single band REFSENS UL configuration is considered as the starting point. |
| Feasibility issues | - The MSD due to IMD and cross band isolation. - The impact on n5 out-of-band blocking requirement. | - The MSD due to cross band isolation - No IMD for 2UL CA n5-n28 | o UL CA_n8-n20 (IMD3 and IMD5), UL CA_n20-n28 (IMD3). o The impact on n20 and n28 out-of-band blocking requirement |

It was also agreed the following RF requirement framework applies to CA_n5-n8, CA_n5-n28 and CA_n8-n20-28.

- Operating bands for CA

- Channel bandwidths per operating band for CA

- UE co-existence requirement

- ∆TIB and ∆RIB values

- REFSENS requirements (MSD)

- Maximum output power

For operating bands for CA, there are frequency restrictions included for CA_n5-n8 and CA_n8-n20-n28. For CA_n5-n8, only partial frequency spectrum for band n5 and n8 is assumed avoiding the conflict for band n5 DL and n8 UL. For CA_n8-n20-n28, only band n28A is considered for this band combination. Thus, the operating bands for CA can be updated as:

Table 2: Inter-band CA operating bands

| CA Band Combinations | Uplink configuration | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n20-n281 | CA_n8-n20 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | CA_n8-n28 | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | CA_n20-n28 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| CA_n5-n82 | CA_n5-n8 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  |  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| CA_n5-n28 | CA_n5-n28 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  |  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| NOTE 1: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 2: The frequency range in band n5 is restricted for this band combination to 824 - 835MHz for the UL and 869-880 MHz for the DL; the frequency range in band n8 is restricted for this band combination to 904 - 915MHz for the UL and 949-960 MHz for the DL. |  |  |  |  |  |  |  |  |  |

For NR CA configurations, it is summarized in table 3 with supported channel bandwidths per CA band combinations.

Table 3. NR CA configuration for CA_n8-n20-n28, CA_n5-n8, CA_n5-n28

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | NR band | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n8A-n20A-n28A | CA_n8A-n20A | n8 | 5,10,15 | 35 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |  |
|  | CA_n8A-n28A | n8 | 5,10,15 | 35 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  | CA_n20A-n28A | n20 | 5, 10, 15, 20 | 40 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5,10 | 20 | 0 |
|  |  | n8 | 5,10 |  |  |
| CA_n5A-n28A | CA_n5A-n28A | n5 | 5, 10, 15, 20 | 50 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |  |

For CA band combination CA_n8-n20-n28 with UL CA_n8-n20 configuration, the coexistence study is in Table 4. We can observe that IMD3 of band n8 and n20 would fall into n8 DL and n20 DL. IMD5 of band n8 and n20 would fall into n8, n20, n28 DL. Further study on MSD requirement is needed after RF architecture assumption is decided.

Table 4: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 826 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |

For CA band combination CA_n8-n20-n28 with UL CA_n8-n28 configuration, the coexistence study is in Table 5. We can observe that IMD3 of band n8 and n28 would not interfere with the DL reception for this CA band combination.

Table 5: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 733 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1027 | 1127 | 491 | 586 |

For CA band combination CA_n8-n20-n28 with UL CA_n20-n28 configuration, the coexistence study is in Table 6. We can observe that IMD3 of band n20 and n28 would fall into n8 DL. Further study on MSD requirement is needed after RF architecture assumption is decided.

Table 6: Band n20 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 733 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 931 | 1021 | 544 | 634 |

For CA_n5-n8, frequency restriction was introduced. The co-existence study is in Table 7. It can be observed that for CA_n5-n8, with 2UL bands transmission, there is no potential interference for DL n5 and n8.

Table 7: Band n5 and Band n8 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 835 | 904 | 915 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 973 | 1006 |

For CA_n5-n28, the 2UL bands coexistence study is shown in Table 8.

Table 8: Band n5 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 703 | 748 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 995 | 557 | 672 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2446 | 2230 | 2345 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 411 | 1141 | 976 |

For CA band combination CA_n5-n28, IMD component is outside the band. There is no potential interference for DL n5 and DL n28. Therefore, no extra MSD is needed for DL n5 and DL n28.

Conclusion

This contribution further clarifies the potential impacted RF requirements for 700/800/900MHz low band CA and summaries the operating bands for CA, CA bandwidths configurations and co-existence studies. The following proposal is made:

Proposal 1: To endorse the following contents for 700/800/900 MHz NR inter-band CA in the TR:

- Operating bands for CA (Table 1)

- Channel bandwidths per operating band for CA(Table 2)

- UE co-existence requirement (Table 3~Table 8)

- ∆TIB and ∆RIB values

- REFSENS requirements (MSD)

- Maximum output power

- References

[1] R4-2214445, WF on study on FS_NR_700800900, CATT, Approved.