3GPP TSG-RAN WG4 Meeting #104-bis-e                                                    	R4-2216155

Online, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 37.718-11-11: DC_1_n1

Agenda item:			5.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-11-11 to include DC_1_n1.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_1_n1

### 5.x.1	Configurations for DC

Table 5.x.1-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n1A | DC_1A_n1A | Yes |

Table 5.x.1-2: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_1A_n1A | DC_1A_n1A | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |

Table 5.x.1-3: Maximum output power for EN-DC (non-continuous sub-blocks

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n1A |  |  |  |  | 23 | +2/-3 |

### 5.x.2	Co-existence studies

No additional co-existence requirements needed.

### 5.x.3	∆TIB and ∆RIB values

No additional relaxations required.

### 5.x.4	Reference sensitivity exceptions

The reference sensitivity requirements for CA_n1(2A) in 38.101-1 have been re-used here.

Table 5.x.4-1: Intra-band non-contiguous EN-DC with one uplink configuration on E-UTRA for reference sensitivity (E-UTRA uplink carrier is closer to the NR downlink carrier than it is to the E-UTRA downlink carrier)

| DC configuration | Aggregated channel bandwidth |  | Wgap / (MHz) | UL E-UTRA allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA | NR |  |  |  |  |
| DC_1A_n1A | 5 MHz | 5 MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |

<End of Text Proposal>