3GPP TSG-RAN WG4 Meeting #104-bis-e                                                    	R4-2216159

Online, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 37.718-21-11: DC_1-3_n1

Agenda item:			5.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-21-11 to include DC_1-3_n1.

The proposed TIB,c and RIB values were derived from those which have already been specified for DC_3_n1 in 38.101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_1-3_n1

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n1A | DC_1A_n1A DC_3A_n1A |

### 5.x.2	Co-existence studies

Table 5.x.2-1 lists the Band 3A + Band n1A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2-1: Band 3 and Band n1 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 1920 | 1980 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 3840 – 3960 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 5760 – 5940 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 135 – 270 |  |  | 3630 – 3765 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1440 – 1650 |  |  | 2055 – 2250 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5340 – 5550 |  |  | 5550 – 5745 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1660 – 1835 |  |  | 1900 – 2000 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 – 3435 |  |  | 3975 – 4230 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 270 – 540 |  |  | 7260 – 7530 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 – 7335 |  |  | 7470 – 7725 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5895 – 6210 |  |  | 4860 – 5220 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2190 – 2520 |  |  | 1170 – 1515 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 – 9705 |  |  | 8760 – 9120 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 – 9510 |  |  | 8970 – 9315 |  |  |

Based on Table 5.x.2-1,

-	2nd order harmonics may fall into Rx frequencies of band 22, 42, 46, 47, 48, 49, 77 and 78.

-	3rd order harmonics may fall into Rx frequencies of band 46 and 77

-	2nd order IMD may fall into Rx frequencies of bands 43, 48, 49, 77 and 78.

-	3rd order IMD may fall into Rx frequencies of bands 1, 4, 10, 11, 21, 23, 24, 32, 45, 46, 50, 65, 66, 74, 75, 92 and 94.

-	4th order IMD may fall into Rx frequencies of bands 31, 42, 52, 72, 73, 77, 78, 87 and 88

-	5th order IMD may fall into Rx frequencies of bands 11, 21, 23, 30, 32, 40, 41, 45, 46, 47, 50, 51, 53, 65, 66, 74, 75, 76, 79, 90, 91, 92, 93 and 94.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 5.x.2-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 5.x.2-2: 2UL Band 3 + Band n1 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, 3rd harmonic, IMD3, IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 3rd harmonic, IMD3, IMD5 |
|  | 5470 | - | 5725 | Yes |  | IMD3 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 3rd harmonic, IMD3, IMD5 |

The requirements for spurious emission band UE coexistence already exist in 38.101-3 for DC_3_n1.

### 5.x.3	∆TIB and ∆RIB values

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n1 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n1 | 0.3 |

### 5.x.4	Reference sensitivity exceptions

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n1A | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |

<End of Text Proposal>