3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216194

# Electronic meeting, 10th - 19th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | draftCR | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.115-2 with updates and corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Perf |  |  |  |  |  | Date: |  |  | 2022-30-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This draftCR provides general updates and corrections to Repeater cofnormance test specification TS 38.115-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General corrections to typos and other errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Final specification would include errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4, 6.4.4.2, 6.5.2.4.2, tab. 6.5.2.5-1a, 6.5.4.3, 6.6.1.4.2, 6.8.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of propose changes>

## 6.3	OTA frequency stability

### 6.3.1	Definition and applicability

Frequency stability is the ability to maintain the same frequency on the output signal with respect to the input signal.

### 6.3.2	Minimum Requirement

The minimum requirement is in TS 38.106 [x], clause 7.3.2.

### 6.3.3	Test purpose

The test purpose is to verify that frequency stability is within the limit specified by the minimum requirement.

### 6.3.4	Method of test

Requirement is tested together with error vector magnitude test, as described in clause 6.6.

### 6.3.5	Test Requirements

The frequency deviation of the output signal with respect to the input signal shall be accurate to within ±(0.01 ppm + 12 Hz) observed over 1 ms.

## 6.4	OTA out of band gain

### 6.4.1	Definition and applicability

Out of band gain refers to the gain of the repeater outside the passband.

### 6.4.2	Minimum Requirement

The minimum requirement is in TS 38.106 [x], clause 7.4.2.

### 6.4.3	Test purpose

The test purpose is to verify that out of band gain is within the limit specified by the minimum requirement.

### 6.4.4	Method of test

#### 6.4.4.1	Initial conditions

Test environment:

-	Normal, see annex B.2,

#### 6.4.4.2	Procedure

For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the repeater at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	Orient the positioner (and repeater and test signal generator) in order that the direction to be tested aligns with the test antenna and the correct angle of arrival for the input signal is achieved.

4)	Set the CW generator power at the RIB as shown in annex D.1.1 with a power equivalent to Pin,p,EIRP, in the correct direction in respect to the repeater.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the radiated power for any two orthogonal polarizations (denoted by p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2, where EIRPp1 is the EIRP value of polarization p1.

7)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I), whilst maintaining the input signal in the correct direction with respect to the repeater.

8)	Calculate TRP using the EIRP measurements. The out of band gain at the frequency under test is given by the difference in dB between the measured output TRP and the EIRP of the input signal.

9) Repeat steps 2-8, shifting the offset frequency of the CW from the edge of the passband from 200kHz to 10MHz in steps of 200kHz for each passband and, for the case of multi-band connectors each operating band

< Next modified section>

### 6.5.2	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 6.5.2.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON state.

#### 6.5.2.2	Minimum requirement

The minimum requirement in TS 38.106 [x], clause 7.5.2.2.

#### 6.5.2.3	Test purpose

To verify that the adjacent channel leakage ratio requirement shall be met as specified by the minimum requirement.

#### 6.5.2.4	Method of test

##### 6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

##### 6.5.2.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex I.

1)	Place the repeater at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	The measurement devices characteristics shall be:

- measurement filter bandwidth: defined in clause 6.7.3.5.

- detection mode: true RMS voltage or true power averaging.

4a)	Set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NR-FR1-TM 1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

4b) Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I) whilst maintaining the correct direction of arrival for the test signal.

6)	Measure the absolute power of the assigned channel frequency and the adjacent channel frequency.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate for the absolute total radiated power of the wanted channel and the adjacent channel using the measurements made in Step 7.

9)	Calculate relative ACLR estimate.

NOTE 1:	ACLR is calculated by the ratio of the absolute TRP of the assigned channel frequency and the absolute TRP of the adjacent frequency channel.

10)	 Measure OTA ACLR for the frequency offsets in both sides of channel frequency as specified in table 6.5.2.5-1. In multiple carrier case only the offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

11)	 For the OTA ACLR requirement applied inside sub-block gap for non-contiguous spectrum operation or inside Inter RF Bandwidth gap for multi-band operation:

a)	Measure OTA ACLR inside sub-block gap or Inter RF Bandwidth gap, if applicable.

b)	Measure OTA CACLR inside sub-block gap or Inter RF Bandwidth gap, if applicable.

< Next modified section>

Table 6.5.2.5-1a: Repeater type 2-O ACLR limit for UL for LA class

| Repeater nominal channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | Repeater adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| min(400MHz, BWpassband) | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 14.7 (Note 3) 13.4 (Note 4) 13.2 (Note 5) |
| NOTE 1: BWChannel and BWConfig are the nominal repeater bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides nominal bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz |  |  |  |  |

< Next modified section>

### 6.5.4	OTA transmitter spurious emissions

#### 6.5.4.1	Definition and applicability

For repeater type 2-O, the OTA transmitter spurious emission limits apply from 30 MHz to 2nd harmonic of the upper frequency edge of the downlink operating band, excluding the frequency range from ΔfOBUE below the lowest frequency of the downlink operating band, up to ΔfOBUE above the highest frequency of the downlink operating band, where the ΔfOBUE is defined in table 6.5.1-1.

#### 6.5.4.2	Minimum requirement

The minimum requirement is defined in TS 38.106 [x], clause 7.5.4.2.2.

#### 6.5.4.3	Test purpose

This test measures radiated spurious emissions while the transmitter is in operation.

#### 6.5.4.4	Method of test

##### 6.5.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	B when testing the spurious emissions below FDL_low - ΔfOBUE,

-	T when testing the spurious emissions above FDL_high + ΔfOBUE; see clause 4.9.1.

##### 6.5.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annex I.

1)	Place the repeater at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the test system.

3)	The measurement devices characteristics shall be:

- detection mode: true RMS.

4a)	Set the input signal at the RIB according to the applicable test configuration and direction in clause 4.8 using the corresponding test models NR-FR1-TM 1.1 in clause 4.9.2 at the input power intended to produce the maximum rated output power, Pin,p,EIRP + 10dB.

4b) Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit.

5)	Orient the positioner (and repeater and test signal source) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed with the specified measurement bandwidth (see annex I) whilst maintaining the correct direction of arrival for the test signal.

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate using the measurements made in Step 7.

< Next modified section>

##### 6.6.1.4.2	Procedure

1a)	Place the repeater at the positioner.

1b) Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit.

2)	Align the manufacturer declared coordinate system orientation [(D.2)] of the repeater with the test system.

3)	Orient the positioner (and repeater) in order that the direction to be tested aligns with the test antenna.

For repeater type 2-O declared to be capable of single carrier operation only, set the repeater to transmit a signal according to the applicable test signal configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test models on all carriers configured:

-	RDL-FR2-TM3.1a with 256QAM signal if 256QAM is supported by repeater without power back off, or

-	RDL-FR2-TM3.1a at manufacturer's declared rated output power if 256QAM is supported by repeater with power back off, and RDL-FR2-TM3.1 with highest modulation order supported without power back off, or

-	RDL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by repeater without power back off, or

-	RDL-FR2-TM3.1 with highest modulation order without power back off if 64QAM is not supported by repeater, or

-	if 64 QAM is supported by repeater with power back off, RDL-FR2-TM 3.1 with 64QAM at manufacturer's declared rated output power (Prated,c,EIRP) and RDL-FR2-TM3.1 with highest modulation order supported at maximum power.

For repeater type 2-O declared to be capable of multi-carrier, set the repeater to transmit according to:

-	RDL-FR2-TM3.1a with 256QAM signal if 256QAM is supported by repeater without power back off, or

-	RDL-FR2-TM3.1a at manufacturer's declared rated output power if 256QAM is supported by repeater with power back off, and RDL-FR2-TM3.1 at maximum power, or

-	RDL-FR2-TM3.1 with 64QAM signal if 64QAM is supported by repeater without power back off, or

-	RDL-FR2-TM3.1 with highest modulation order supported without power back off if 64QAM is not supported by repeater, or

-	if 64QAM is supported by repeater with power back off, RDL-FR2-TM3.1 with 64QAM signal at manufacturer's declared rated output power (Prated,c,EIRP) and RDL-FR2-TM3.1 with highest supported modulation order at maximum power

For RDL-FR1-TM 3.1a and RDL-FR2-TM 3.1, power back-off shall be applied if it is declared.

4)	For each carrier, measure the EVM and frequency error as defined in annex L.

5)	Repeat steps 3 and 4 for RDL-FR2-TM2 if 256QAM is not supported by repeater type 2-O or for RDL-FR2-TM2a if 256QAM is supported by repeater type 2-O. For RDL-FR2-TM2 the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in clause 6.4.3.4.2 and test requirements in clause 6.4.3.5.2.

6)	Adjust the input power to the repeater to create the maximum nominal repeater output power at maximum gain.

7)	Measure the EVM and frequency error as defined in TS 38.141-2 [x] Annex [F].

8)	Repeat the procedure with all the narrower bandwidths of NR-FR2-TM3.1

< Next modified section>

< Next modified section>

##### 6.8.2.3.2	Procedure

1a)	Place the repeater with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.7.

1b) Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit.

2)	Align the manufacturer declared coordinate system orientation of the repeater with the test system.

3)	Align the repeater with the test antenna in the declared direction to be tested.

4)	Align the repeater to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Set the signal generator to transmit a signal modulated with test model XX for downlink and  test model XX for uplink as defined in section 4.9 at the first or last channel with channel offset from frequency range of passband defined in section 6.8.2.3.3 within the pass band.

6)	Adjust the input power to the Repeater to create the maximum nominal Repeater output power at maximum gain

7)	Orient the positioner (and repeater) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

8)	Measure the absolute power of the assigned channel frequency.

9)	Repeat step 7-8 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex XX).

10)	  Calculate TRPEstimate for the absolute total radiated power of a carrier of the repeater in the passband and  using the measurements made in Step 9.

11)	  Set the signal generator to transmit the same signal and the same input power at one of the channel offsets outside the repeater pass band according to Tables in section 6.8.2.3.3.

12)	  Repeat step 7) to 9) for the measurement of the input signal outside the repeaterpassband.

13)	  Calculate TRPEstimate for the absolute total radiated power of a carrier over an adjacent channel outside the repeater passband and  using the measurements made in Step 9.

14)	  Calculate relative ACRR estimate.

15)  Repeat step 5) to 14) until all channel offsets in Tables in section 6.8.2.3.3 are measured.

<End of proposed changes>