3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216217

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on timing requirements for RedCap UEs

Agenda item:	4.6.3.1.3   Timing requirements   [NR_redcap-Core]

Document for:	Discussion

Introduction

The discussion on RedCap core requirements was continued at RAN4 #104-e [1][2]. With regard to timing requirements, the following issue was treated [1].

Issue 3-1-1: Timing requirements when SSB is not in the active BWP Background: Following was agreed at RAN4#103-e [R4-2210592]: Whether SSB has to be in UE active BWP for meeting the UE transmit timing requirements For core requirement, Redcap UE should meet the existing Te and Tq requirements provided that the SSB is available at the UE at least once every 160 ms on the following conditions that - The SSB should be within active BWP, or - The SSB is not within active BWP, and the gap is configured - Capture the condition in the section for RedCap timing of the specification. - Proposals - Option 1 (QC): UE shall meet UL Tx timing accuracy requirement based on intra-freq reference SSB outside active BWP if max (MGRP, SMTC period) x CSSFintra_RedCap <= 160 ms.

In the WF [3], it is noted that there was no consensus to add conditions.

Agreement: Timing requirements when SSB is not in the active BWP No consensus reached to add the conditions to existing requirements.

In this contribution, we discuss the above open issue from last meeting.

Discussion

The above open issue from last meeting is discussed in this section.

Timing requirements when SSB is not in the active BWP

Referring to the agreement at RAN4#103-e, it is specified in section 7.1A.2 in 38.133:

### 7.1A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1A.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission. The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE during the last 160 ms on the condition that: the SSB is within the UE’s active BWP, or the SSB is not within the UE’s active BWP, and the measurement gap is configured

It is clear, that in case the SSB is not in the active BWP, then the network will provide the UE in RRC_CONNECTED with a measurement gap for measuring CD-SSB of the serving cell, for obtaining the reference timing, and other cells on intra-/inter-frequency NR layers and possibly inter-RAT EUTRAN frequency layers. In this case the measurement gap is shared, as UE has to measure serving cell and all configured frequency layers. This means that intra-frequency measurement with gap scales with the configured CSSF. In case of large MGRP or SMTC, such as 160 ms,  CSSFintra_RedCap > 1 may thus lead to the issue, that due to measurement gap sharing the UE cannot identify and measure CD-SSB of the serving cell within 160 ms. Such scenario is possible, and hence Tx timing requirements should still cover it.

The following proposal is made.

RAN4 to ensure that Tx timing requirements cover the scenario of active BWP without SSB and need for measurement gap for monitoring CD-SSB in case of MG sharing and large MGRP/SMTC.

At RAN4 #104-e, it was proposed in [2] to add the condition

max (MGRP, SMTC period) x CSSFintra_RedCap <= 160 ms

for requiring the UE to meet the Tx timing requirements in section 7.1A.2. While this is one option to solve the above issue, there is an alternative in specifying that the UE shall measure the serving cell immediately to ensure CSSF intra_RedCap = 1, so that the UE can obtain the reference timing of the serving cell within 160 ms even in case of measurement gap sharing and large MGRP/SMTC. In our view, this is preferable and is aligned to some companies’ views expressed during RAN4 #104-e [1]. The corresponding change to TS 38.133 is depicted below.

### 7.1A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1A.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission. The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured, and the UE prioritizes SSB measurement for obtaining reference timing of the serving cell over other configured measurements in case of measurement gap sharing.

The following proposal is made.

Existing Tx timing requirements are reused for the scenario of active BWP without SSB and need for measurement gap for monitoring CD-SSB in case of MG sharing and large MGRP/SMTC under the condition that UE prioritizes SSB measurement for obtaining reference timing of the serving cell over other configured measurements.

Conclusion

The open issue from last meeting on Tx timing requirements for RedCap UE in case of active BWP without SSB has been addressed in this contribution and the following proposals are made.

- RAN4 to ensure that Tx timing requirements cover the scenario of active BWP without SSB and need for measurement gap for monitoring CD-SSB in case of MG sharing and large MGRP/SMTC.

Existing Tx timing requirements are reused for the scenario of active BWP without SSB and need for measurement gap for monitoring CD-SSB in case of MG sharing and large MGRP/SMTC under the condition that UE prioritizes SSB measurement for obtaining reference timing of the serving cell over other configured measurements.

The companion CR is submitted in [4].

- References

- R4-2214274, “Email discussion summary for [104-e][223] NR_redcap_RRM_1”, Moderator (Ericsson)

- R4-2214074, “Timing requirements for RedCap UEs”. Qualcomm Incorporated

- R4-2214485, “WF on RedCap RRM requirements”, Ericsson

- R4-2216218, “Correction to Tx timing requirements for active BWP without SSB for RedCap”, Nokia, Nokia Shanghai Bell