3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2216238

Electronic meeting, October 10 – October 19, 2022

Source: 	Huawei, HiSilicon

Title: 	Feasibility study from RF perspective

Agenda Item:	6.17.2.2.1

Document for:	Approval

- Introduction

At last RAN4#104-e meeting, WF on feasibility study from RF perspective was approved in [1]. In this contribution, we provide further analysis and our view on some remaining FFS parts.

- Discussion

[2] Agreement on RSI dependency on Blocking and AGC（Question 1-4）:

- Proposal 1: The in-band blocking is suggested to applied as starting point to ensure the receiver of UL sub-band is not blocked due to DL sub-band transmission

  - Besides blocking, LNA and dynamic range can be FFS for receiver side

- Proposal 2: AGC may be applied to adjust the receiver gain to avoid ADC saturation if spatial isolation and analog IC, if found feasible, don’t provide enough reduction to self-interference. This may result in cost of an impact on sensitivity and potentially reduced coverage. However, it seems infeasible to model this in SLS.

Note: above proposal will not preclude other study regarding this issue. The size of any impact of coverage associated with AGC should in particular be elaborated.

As discussed in [3], by adopting the representative parameters, the overall noise caused by -43 dBm blocking is -95.5 dBm which contributes ~0.5 dB degradation. Hence we can assume -43 dBm can be adopted to ensure the receiver of UL sub-band is not blocked and maintain an acceptable reference sensitivity.

Proposal 1: -43 dBm can be assumed as the maximum blocking level to ensure the receiver of UL sub-band is not blocked and maintain an acceptable reference sensitivity.

[2] Agreement on feasibility and how to model co-site inter-sector gNB-gNB CLI modelling: similar modelling as for self-interference(RSI) can be applied but may with different parameters especially on antenna isolation

  - FFS on possibility to apply digital IC for this case

On digital IC aspect, in our view, since the information of non-linear product is already got in the digital domain, and it can be exchanged between sectors within a BBU, digital IC can be applied for this case.

Proposal 2: Digital IC can be applied for co-site inter-sector case.

[2] On feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity:

Agreement:

- Proposal: Same Transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage candidate: gNB ACLR

  - Receiver impairment candidate: gNB ACS

  - RAN4 will further study the possibility of improved performance/requirements compared to existing referred requirements list above.

From the gNB transmitter or receiver perspective, the ACLR or ACS modelling is independent to propagation channel. Hence the ACLR and ACS model are the same as for RSI. For ACLR we can assume 45 dB to be reused. Regarding ACS, as discussed in [3], we can achieve 62 dB overall selectivity. Hence the following parameters are proposed for the BS capable of SBFD operation.

  - TX leakage candidate: 45 dB

  - Receiver impairment candidate: 62 dB

Proposal 3: The following parameters are proposed for the BS capable of SBFD operation.

  - TX leakage candidate: 45 dB

  - Receiver impairment candidate: 62 dB

- Conclusion

In the contribution, we provide discussion on feasibility study from RF perspective.

Proposal 1: -43 dBm can be assumed as the maximum blocking level to ensure the receiver of UL sub-band is not blocked and maintain an acceptable reference sensitivity.

Proposal 2: Digital IC can be applied for co-site inter-sector case.

Proposal 3: The following parameters are proposed for the BS capable of SBFD operation.

  - TX leakage candidate: 45 dB

  - Receiver impairment candidate: 62 dB

- Reference

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC

- R4-2214377, “WF on feasibility study from RF perspective”, Samsung

- R4-2216239, Evolution of receiver blocking and AGC, Huawei, HiSilicon