3GPP TSG-RAN WG4 Meeting # 104-bis-e 	R4-2216240

Electronic Meeting, October 10 – October 19, 2022

Source: 	Huawei, HiSilicon

Title: 	Definition of FR2 multi-band BS

Agenda Item:	6.4.2

Document for:	Approval

1 Introduction

New SID on NR BS RF requirement evolution was approved in RAN#95-e [1]. In last meeting, the WF on the definition of FR2 multi-band BS was approved [2].

Definition of FR2 multi-band BS

- To investigate if the FR1 definition of multi-band RIB can be re-used for FR2

- To revisit the definition of multi-band BS for FR2, and clarify whether the following scenarios should be considered as multi-band BS

1) Multi-band transmitter and/or receiver with common active RF components

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware

4) BS covers full-band or sub-band of band A and band B

5) BS covers consecutive spectrums with different band number, for example, n258+n261

6) BS covers overlapping spectrums with different band number, for example, n258+n257

- FFS whether need to introduce the definition of multi-band BS for FR2 into specification

In this contribution, we continue the discussion on the definition of multi-band RIB and multi-band BS.

2 Discussion

In TR37.812 [3], the definition of MB-MSR BS is given as:

MB-MSR Base Station: MSR Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s).

From the WF [2], it was tasked to clarify whether the following scenarios should be considered as multi-band BS.

1) Multi-band transmitter and/or receiver with common active RF components

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware

4) BS covers full-band or sub-band of band A and band B

5) BS covers consecutive spectrums with different band number, for example, n258+n261

6) BS covers overlapping spectrums with different band number, for example, n258+n257

Scenario 1

It follows existing MB-MSR approach, and should be considered as a multi-band BS

Scenario 2

Single band requirements and conformance test apply, so it does not need to be considered as a multi-band BS.

Scenario 3

The BS can be configured as band A or B at different time/stage. Single band requirements and conformance test can be still applied hence it also does not need to be considered as a multi-band BS.

Scenario 4

In existing specification for single band BS, it is allowed to declare a supported frequency range within the operating band. Hence BS covers full-band or sub-band of band A and band B should be considered as a multi-band BS.

Scenario 5, scenario 6

For a specific region, non-overlapping spectrums are allocated in two bands. The requirements are defined referring to Base Station RF Bandwidth. Hence consecutive spectrums and partially overlapping spectrums should be considered for multi-band BS. In these cases, there is no inter-band gap and inter RF bandwidth is similar as sub-block gap for single band non-contiguous spectrum.

Based on above discussion, scenarios 1~3 are discussing about BS architecture and scenarios 4~6 are about spectrum. On BS architecture aspects, if separate active RF components are used and the one band can be off for the conformance test on the other band, single band requirements can be still applied. While for multi-band operation with common active RF components, multi-band requirements are needed for it cannot switch one of the bands. Hence FR2 multi-band BS can be defined as below. Whether the definition is introduced in the specification can be discussed later in the WI phase when defining the requirements.

FR2 multi-band BS: FR2 Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

Proposal 1: Scenario 2 and 3 should not be considered as multi-band BS.

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware

2.3 Conclusion

In this contribution, we provide the discussion on the definition of multi-band RIB and multi-band BS.

Proposal 1: Scenario 2 and 3 should not be considered as multi-band BS.

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware

FR2 multi-band BS can be defined as below.

FR2 multi-band BS: FR2 Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

References

[1] RP-221001, “New SID: NR BS RF requirement evolution”, Huawei, HiSilicon