3GPP TSG-RAN WG4 Meeting # 104-bis-e                	R4-2216257

Electronic Meeting, October 10 – 19, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on RRM requirements with CCA in FR2-2

Agenda item: 	4.3.5.3

Document for: 	Discussion and Decision

Introduction

This paper presents Nokia’s view on RRM requirements with CCA for the operation between 52.6 GHz and 71 GHz.

Discussion

UE assumption on whether CCA is used by neighbor cells

In RAN4 #103, RAN4 discussed whether the UE in FR2-2 should assume that CCA is always used by neighbor cells. An LS to RAN2 [1] asking whether and how the information about the channel access mode used by neighbour cells is made available to the UE in RRC_IDLE and RRC_CONNECTED state.

In RAN2 #119, RAN2 sent an LS reply [2] concluding that it is feasible to indicate whether to apply CCA for neighbor cell measurements in Rel-17 and attached a draft CR R2-2209234 showing how it can be done.

RAN2 has concluded that it is feasible to indicate whether the neighbour cells are subject to CCA.

Considering that it is feasible to assume that the network will indicate whether the neighbour cells are subject to CCA, RAN4 requirements shall be applicable considering that this information is available the UE. Since the requirements for operation with and without CCA had been already defined, it is now up to RAN4 to define the conditions in which requirements with and without CCA apply. Therefore, we propose to capture that as an agreement in RAN4 and send an LS to RAN2, so that RAN2 can proceed with the CRs.

RAN2 draft CR attached to the LS Reply already includes signalling that enables the UE to know the CCA configuration of neighbors both in RRC connected and in RRC idle/inactive modes.

The RRM requirements defined in TS 38.133 for FR2-2 were defined under the assumption that the UE knows the channel access mode in use by neighbor cells

The requirements with CCA apply if the UE is signaled that the neighbor cells are configured with CCA, and requirements without CCA apply otherwise.

Send a reply LS to RAN2 informing that the UE shall receive indication about the channel access mode in use by neighbor cells.

Separation between consecutive measurements

In the RAN4 #103 meeting the following was discussed [2]:

- Consider in the maintenance stage between option 1 and option 2 on how to specify the maximum separation between two consecutive measurements. - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. - Option 2: the requirement only applies when - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured

No agreements were reached in the last RAN4 meeting. In the discussion paper in [4], we provided an analysis of the duration of consecutive failures if there was no limitation about the separation between two consecutive measurements – only using the maximum number of SMTC occasion group failures and a beam sweeping scaling factor equal to 12. In that paper we show that this time can be up to 13.44 seconds even when DRX is not configured. When DRX is configured, this time can be up to 23.04 seconds. This situation is different from NR-U, since in NR-U the time was limited by the maximum number of LBT failures for a given procedure, and it would be up to 1.92s in PSS/SSS detection for example. However, in the last meeting there was a clear majority to option 1.

In our view, we can compromise to the view of the majority, since Option 1 follows the definition of NR-U, even though in NR-U the limitation on the time between samples occurred due to maximum number of LBT failures and not due to the requirements being discussed here.

The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known.

Conclusions

In this document, Nokia’s views on the RRM requirements with CCA. The following observations and proposals are discussed:

- RAN2 has concluded that it is feasible to indicate whether the neighbour cells are subject to CCA.

RAN2 draft CR attached to the LS Reply already includes signalling that enables the UE to know the CCA configuration of neighbors both in RRC connected and in RRC idle/inactive modes.

The RRM requirements defined in TS 38.133 for FR2-2 were defined under the assumption that the UE knows the channel access mode in use by neighbor cells

- The requirements with CCA apply if the UE is signaled that the neighbor cells are configured with CCA, and requirements without CCA apply otherwise.

Send a reply LS to RAN2 informing that the UE shall receive indication about the channel access mode in use by neighbor cells.

The requirements apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known.

References

- R4-2211171, LS on signalling of CCA configurations of neighbour cells, RAN4

- R2-2209235, LS on CCA configurations of neighbour cells”, RAN2

- R2-2209234, Draft CR - FR2-2 and CCA configuration, Nokia

- R4-2212911, General discussion on RRM requirements with CCA in FR2-2, Nokia

- R4-2215159, WF on NR extension to 71 GHz RRM requirements (Part 1), RAN4 #104, Huawei, HiSilicon