GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216274

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.9.2.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2 SCell activation delay reduction

Document for:	Discussion

# Introduction

The Rel-18 WI of even further RRM enhancement was approved in [1] and further revised in [2]. One of the objectives is to study the SCell activation delay reduction in FR2. In RAN4#104e meeting, the scope and solutions were initially discussed with agreements and open issues captured in the WF [3]. In this paper, we further provide our views of RRM impact of the objective and provide further analysis on the solutions.

# Discussion

In last RAN4 meeting, apart from enhancement targeting components in L3/L1 procedures, there were solutions proposed from different angles for SCell activation delay reduction. The status are summarized as follows:

- Sub-topic 2-4 Other potential enhancement for FR2 SCell activation Issue 2-4-1: Other possible enhancement of FR2 unknown SCell activation FFS: - Proposal 1 (LG, Nokia, ZTE): Consider long term measurement or aperiodic measurement to detect target SCell before SCell activation command - Proposal 2 (Nokia, LGE, Ericsson): - Proposal 2a: The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be revisited to reduce the FR2 SCell activation delay. - Proposal 2b: Some alignment on the up-to-date known/unknown status of the to-be-activated SCell shall be considered to determine the FR2 SCell activation delay. - Proposal 2c (this is not only for L3 part but for the whole activation procedure): The potential to enable earlier data transmission within the activation period needs to be studied - Proposal 3 (MediaTek): - Contention based random access (CBRA) can be used to activate first unknown SCell in one band. - Proposal 4 (Huawei, Apple, CTC, ZTE): - Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced (e.g., Tactivation_time=3ms). - Proposal 5 (ZTE, CTC): - To realize energy saving, similar as scellWithoutSSB defined for intra-band CA in Rel-16, the SCell without SSB or SSB-less can be considered for inter-band CA case. The corresponding SCell activation procedure needs some update for such SCell withou SSB or SSB-less.

One distinctive angle for delay reduction is about changing the known the conditions for SCell activation. The current known conditions for FR2 SCell is shown as follows:

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

It could be observed that the known conditions requires that UE shall send L3 measurement report with index for the deactivated SCell before activation command no earlier than 3/4 seconds. Companies proposed the timing duration of 3/4 seconds can be revised to reduce the activation delay. However, we have different understanding on this proposed changing. The known conditions basically mean UE can utilize the historic information (e.g. timing and beam) and the condition remain valid until UE completes the SCell activation. The existing known conditions imply the implementation of gNB that UE is configured with measurement and report of the target SCell, and the report interval shall be less than the required time period (3s/4s).

By extending the time period of 3s/4s in the known conditions, there may be benefits on power saving as UE can report measurement results less frequently. But it is not targeting the delay reduction since the time period that the cell can remain detectable does not depend on how we define the known conditions, instead, it is depends on the realistic channel conditions. For example, if the target cell is stable for more than 10 seconds, no matter whether the known conditions is defined as 3 seconds or 10 seconds, the known conditions can always been met. However, if the Cell is just merging for 5 seconds, if the known conditions is modified as 10 seconds, it simply means UE cannot take the cell as a known one as the cell is not measured/reported 10 seconds ago. In summary, if a known SCell activation is expected, gNB will configure more frequency measurement report to make sure that the known conditions can be met in more cases. By extending the time period in the known conditions, the benefits is that gNB can configure long report interval in the case that the SCell can remain detectable for a long time which may have power saving gain. But it will not speed up the SCell activation procedure.

Observation 1: Extending the time duration in known conditions implies that gNB can configure long report interval in the case that the SCell can remain detectable for a long time which may have power saving gain.

However, we believe that the direction of reconsidering the known conditions is worthy further investigation. Based on current known conditions, if gNB want to activate the SCell as a known one, one approach is to configure measurement and periodic report all the time for the target cell even there is not much data for transmission. Another option is that gNB just configure the measurement and report when data is arrival. It means before activating the SCell, gNB shall first perform RRC Reconfigurations to configure the measurement and report, and then UE will perform measurement and report. After receiving the measurement report, gNB can activate the SCell follow the known branch. It is obvious that the former one will lead to undesired power consumption. The latter one will have longer delay as the pure L2 SCell activation procedure, will become L2+L3 procedure. Apart from the legacy approach based on measurement and report, we are open to discuss the extension of known conditions to facilitate SCell activation.

Proposal 1: RAN4 to further discuss the known condition extension to facilitate SCell activation.

Further enhancement on SCell activation

In current specification, the activation delay (Tactivation_time) of the intra-band to-be-activated SCell can be reduced to 3ms under certain side conditions, e.g., for FR1 the conditions are,

-	The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

-	The difference of the reception power with the contiguous active serving cell is <= 6dB, and

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

For FR2, the conditions are,

- the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

However if the to-be-activated SCell is the first cell in one band (no other active serving cell on the same band), the activation delay would be very large as per current requirements. In the following, we try to seek for the further enhancement the FR2 inter-band SCell activation which can also been largely reduced to 3ms (like intra-band scenario).

In our understanding in FR2 under certain network deployment, e.g.,

- If FR2 inter-bands are collocated and the time alignment between two bands is close to that of the intra-band CA, then the RS (s) of SCell being activated can be QCL-TypeA/C with RS (s) of the active serving cell on another FR2 band.

- If the power difference between two bands is not larger than [Y]dB, it can guarantee no AGC is required;

- If the RS(s) of SCell being activated can be QCL type-D with the RS (s) of the active serving cell on another FR2 band, then for both CBM capable UE and IBM capable UE, identical beams can be used on SCell being activated and the beams on another band.

In such above scenario UE can utilize the timing/beam information from active serving cell on FR2 inter-band. Therefore the cell detection, AGC and L1-RSRP procedures can be skipped. Then SCell activation delay only needs to take decoding SCell activation command time into account. We think this further enhancement would be appealing as the inter-band FR2 SCell activation delay would be enormously reduced. Moreover this enhancement would also enable fast SCell activation if the SCell is the first to-be-activated serving cell on inter-band.

Proposal 2: Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

# Conclusions

Observation 1: Extending the time duration in known conditions implies that gNB can configure long report interval in the case that the SCell can remain detectable for a long time which may have power saving gain.

Proposal 1: RAN4 to further discuss the known condition extension to facilitate SCell activation.

Proposal 2: Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC

[3] R4-2214345, WF on R18 eFeRRM