3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216282

Electronic Meeting, October 10 – 19, 2022

Title: 	Discussion on RRM remaining issues for R17 inter-cell BM

Source: 	Huawei, HiSilicon

Agenda item:	4.5.1.2

Document for:	Discussion

Introduction

In this contribution, we will further discuss the definition of sharing factor used for inter-cell L1-RSRP measurement requirements.

Discussion

Sharing factor

For inter-cell beam management in R17, the definition of sharing factor between SSB of SC and SSB of CDP was discussed for overlapping scenario. RAN4 agreed that the new sharing factors for inter-cell L1-RSRP measurements are introduced without impacts on the existing L3 measurements.

In the existing L1-RSRP measurements, the sharing factor P is defined for sharing between L1 measurements and L3 measurements. After punctured by L3 measurements, the remaining L1-RSRP measurement opportunities are further shared between SC SSB and CDP SSB.

For L1-RSRP measurements, the sharing factor between L1 and L3 can be assumed as PL3,SC for SC SSB and PL3,CDP for CDP SSB. Then, the remaining SSB periodicity after punctured by L3 measurements can be assumed as T’SSB,SC for SC SSB and T’SSB,CDP for CDP SSB, where T’SSB,SC is equal to (PL3,SC×TSSB,SC) and T’SSB,SC is equal to (PL3,CDP×TSSB,CDP). The sharing strategy between SC SSB and CDP SSB can be separately discussed for the following 3 cases:

- Case 1: T’SSB,SC = T’SSB,CDP

- Case 2: T’SSB,SC < T’SSB,CDP

- Case 3: T’SSB,SC > T’SSB,CDP

For case 1, the remaining SSB periodicity of SC is equal to the remaining SSB periodicity of CDP. Then, the remaining L1-RSRP measurement opportunities are equally shared between SC SSB and CDP SSB.

For case 2, the remaining SSB periodicity of SC is shorter than the remaining SSB periodicity of CDP. Then, the L1-RSRP measurements on SC SSB can be further punctured by L1-RSRP measurements on CDP SSB.

For case 3, the remaining SSB periodicity of SC is longer than the remaining SSB periodicity of CDP. Then, the L1-RSRP measurements on CDP SSB can be further punctured by L1-RSRP measurements on SC SSB.

In last RAN4 meeting, the following two options were considered for defining the sharing factor between SC SSB and CDP SSB.

  - Option 1:

| # | Scenario | PSC | PCDP |
| --- | --- | --- | --- |
| 1 | T’SSB,SC = T’SSB,CDP | 2 | 2 |
| 2 | T’SSB,SC < T’SSB,CDP | 11-T'SSB,SCT'SSB,CDP | 1 |
| 3 | T’SSB,CDP < T’SSB,SC | 1 | 11-T'SSB,CDPT'SSB,SC |

  - Option 2:

| # | Scenario | PSC | PCDP |
| --- | --- | --- | --- |
| A | SC SSB occasions outside MG are fully overlapping with CDP SSB occasions outside MG | 2 | 2 |
| B | SC SSB occasions outside MG are partially overlapping with CDP SSB occasions outside MG | 2 | 1 |
| C | Scenario C: CDP SSB occasions outside MG are partially overlapping with SC SSB occasions outside MG. | 1 | 2 |

It can be observed that either option 1 or option 2 is aligned with the above sharing strategy, and option 2 is just defined based on the worst case.

Proposal 1: After punctured by L3 measurements, the sharing strategy between SC SSB and CDP SSB for L1-RSRP measurements can be defined as follows:

- When the remaining SSB periodicity of SC is equal to the remaining SSB periodicity of CDP, the remaining L1-RSRP measurement opportunities are equally shared between SC SSB and CDP SSB.

- When the remaining SSB periodicity of SC is shorter than the remaining SSB periodicity of CDP, the L1-RSRP measurements on SC SSB can be further punctured by L1-RSRP measurements on CDP SSB.

- When the remaining SSB periodicity of SC is longer than the remaining SSB periodicity of CDP, the L1-RSRP measurements on CDP SSB can be further punctured by L1-RSRP measurements on SC SSB.

Proposal 2: The sharing factors PSC and PCDP for inter-cell L1-RSRP measurements can be defined as option 1:

  - Option 1:

| # | Scenario | PSC | PCDP |
| --- | --- | --- | --- |
| 1 | T’SSB,SC = T’SSB,CDP | 2 | 2 |
| 2 | T’SSB,SC < T’SSB,CDP | 11-T'SSB,SCT'SSB,CDP | 1 |
| 3 | T’SSB,CDP < T’SSB,SC | 1 | 11-T'SSB,CDPT'SSB,SC |

SC SSB is not only configured for L1-RSRP measurements but also configured for RLM/BFD/CBD measurements. CDP SSB is not only configured for L1-RSRP measurements but also configured for BFD/CBD measurements. So, it is possible that SC SSB configured for RLM/BFD/CBD measurements is overlapped with CDP SSB configured for L1-RSRP measurements and CDP SSB configured for BFD/CBD measurements is overlapped with SC SSB configured for L1-RSRP measurements.

For these cases, whether to apply measurement restrictions or the same sharing strategy for inter-cell L1-RSRP measurements need to be discussed in RAN4. If the same sharing strategy is applied, then there will be a big impact on the existing specification. Hence, we suggest to apply measurement restrictions.

Proposal 3: The measurement restrictions are applied between SC SSB for RLM/BFD/CBD and CDP SSB for L1-RSRP.

Proposal 4: The measurement restrictions are applied between CDP SSB for BFD/CBD and SC SSB for L1-RSRP.

Conclusions

This contribution provides our discussion on remaining issues for Rel-17 inter-cell beam management. The following are provided:

Proposal 1: After punctured by L3 measurements, the sharing strategy between SC SSB and CDP SSB for L1-RSRP measurements can be defined as follows:

- When the remaining SSB periodicity of SC is equal to the remaining SSB periodicity of CDP, the remaining L1-RSRP measurement opportunities are equally shared between SC SSB and CDP SSB.

- When the remaining SSB periodicity of SC is shorter than the remaining SSB periodicity of CDP, the L1-RSRP measurements on SC SSB can be further punctured by L1-RSRP measurements on CDP SSB.

- When the remaining SSB periodicity of SC is longer than the remaining SSB periodicity of CDP, the L1-RSRP measurements on CDP SSB can be further punctured by L1-RSRP measurements on SC SSB.

Proposal 2: The sharing factors PSC and PCDP for inter-cell L1-RSRP measurements can be defined as option 1:

  - Option 1:

| # | Scenario | PSC | PCDP |
| --- | --- | --- | --- |
| 1 | T’SSB,SC = T’SSB,CDP | 2 | 2 |
| 2 | T’SSB,SC < T’SSB,CDP | 11-T'SSB,SCT'SSB,CDP | 1 |
| 3 | T’SSB,CDP < T’SSB,SC | 1 | 11-T'SSB,CDPT'SSB,SC |

Proposal 3: The measurement restrictions are applied between SC SSB for RLM/BFD/CBD and CDP SSB for L1-RSRP.

Proposal 4: The measurement restrictions are applied between CDP SSB for BFD/CBD and SC SSB for L1-RSRP.

Reference

- R4-2214482,  “WF on FeMIMO RRM requirements for inter-cell beam management and TRP-specific link recovery”, Huawei, HiSilicon