3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216284

# Online, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on maintaining TCI state switching requirements for R17 unified TCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the signalling design, most of general test parameters are not TRP-specific configured except BFD-RS and CBD-RS resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To update most general test parameters as non-TRP specific configured for TRP-specific BFD and link recovery tests. - To update the SNR values of BFD-RS or CBD-RS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test configurations of TRP-specific BFD and link recovery tests are not aligned with the signalling design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.5.7, A.5.5.5.8, A.6.5.5.7, A.7.5.5.9, A.7.5.5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode

##### A.4.5.5.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the TRP specific SSB-based beam failure in the set (q0,0), (q0,1) configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set (q1,0) and (q1,1). The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.4.5.5.7.1-1, A.4.5.5.7.1-2, A.4.5.5.7.1-3 and A.4.5.5.7.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the active PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.5.7.1-1 shows the variation of the downlink SNR of the PSCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure A.4.5.5.7.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.4.5.5.7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.5.7.1-2: General test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  |  |  |  |  | Unit |  |  | Value |  |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Test 1 |  |  |  |  |  |
| Active E-UTRA PCell |  |  |  |  |  |  |  |  |  |  |  | Cell 1 |  |  |  |  |  |
| E-UTRA RF Channel Number |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| Active PSCell |  |  |  |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |  |
| RF Channel Number for PSCell |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| Duplex mode |  | Config 1, 4 |  |  |  |  |  |  |  |  |  |  | FDD |  |  |  |  |
|  |  | Config 2, 3, 5, 6 |  |  |  |  |  |  |  |  |  |  | TDD |  |  |  |  |
| BW channel |  | Config 1, 4 |  |  |  |  |  |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  | MHz |  |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |
| TDD Configuration |  | Config 1, 4 |  |  |  |  |  |  |  |  |  |  | Not Applicable |  |  |  |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  |  |  |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | TDDConf.2.1 |  |  |  |  |
| CORESET Reference Channel |  | Config 1, 4 |  |  |  |  |  |  |  |  |  |  | CR.1.1 FDD |  |  | A.3.1.2 |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | CR.2.1 TDD |  |  |  |  |
| SSB Configuration for TRP0 with serving PCI |  | Config 1, 4 |  |  |  |  |  |  |  |  |  |  | SSB.3 FR1 |  |  | A.3.10 |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  |  |  |  | SSB.3 FR1 |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | SSB.4 FR1 |  |  |  |  |
| SSB Configuration for TRP1 with non-serving PCI |  | Config 1, 4 |  |  |  |  |  |  |  |  |  |  | SSB.7 FR1 |  |  | A.3.10 |  |
|  |  | Config 2, 5 |  |  |  |  |  |  |  |  |  |  | SSB.7 FR1 |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | SSB.8 FR1 |  |  |  |  |
| SMTC Configuration |  | Config 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  | SMTC.1 |  |  | A.3.11 |  |
| PDSCH/PDCCH |  | Config 1, 2, 4, 5 |  |  |  |  |  |  |  | kHz |  |  | 15 |  |  |  |  |
| subcarrier spacing |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | 30 |  |  |  |  |
| PRACH Configuration |  | Config 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  | Table A.3.8.2.2-1 |  |  |  |  |
|  |  | Config 3, 6 |  |  |  |  |  |  |  |  |  |  | Table A.3.8.2.2-1 |  |  |  |  |
| SSB Index assigned as BFD RS (q0,0) |  |  |  |  |  |  |  |  |  | 0 |  |  |  |
| SSB Index assigned as CBD RS (q1,0) |  |  |  |  |  |  |  |  |  | 1 |  |  |  |
| SSB Index assigned as BFD RS (q0,1) |  |  |  |  |  |  |  |  |  | 2 |  |  |  |
| SSB Index assigned as CBD RS (q1,1) |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| OCNG parameters |  |  |  |  |  |  |  |  |  | OP.1 |  | A.3.2.1 |  |
| CP length |  |  |  |  |  |  |  |  |  | Normal |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  |  |  |  |  | 1-0 |  |  |  |
|  |  | Number of Control OFDM symbols |  |  |  |  |  |  |  | 2 |  |  |  |
|  |  | Aggregation level |  |  |  |  |  | CCE |  | 8 |  |  |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  |  |  | dB |  | 0 |  |  |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  |  |  | dB |  | 0 |  |  |  |
|  |  | DMRS precoder granularity |  |  |  |  |  |  |  | REG bundle size |  |  |  |
|  |  | REG bundle size |  |  |  |  |  |  |  | 6 |  |  |  |
| DRX |  |  |  |  |  |  |  |  |  | OFF |  |  |  |
| Gap pattern ID |  |  |  |  |  |  |  |  |  | gp0 |  |  |  |
| gapOffset |  |  |  |  |  |  |  |  |  | 0 |  |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  |  |  |  |  | absent |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |  |
| rsrp- |  |  |  |  |  |  | Config 1, 2, 4, 5 |  |  | dBm/SCS |  |  | -98 |  |  | Threshold used |  |
| ThresholdBFR |  |  |  |  |  |  | Config 3, 6 |  |  |  |  |  | -95 |  |  | for Qin_LR_SSB |  |
| powerControlOffsetSS |  |  |  |  |  |  |  |  |  | db0 |  | Used for deriving rsrp-ThresholdCSI-RS |  |
| beamFailureInstanceMaxCount |  |  |  |  |  |  |  |  |  | n1 |  | see TS 38.321 [7], clause 5.17 |  |
| beamFailureDetectionTimer |  |  |  |  |  |  |  |  |  | pbfd4 |  | see TS 38.321 [7], clause 5.17 |  |
| SSB Index assigned as RLM RS |  |  |  |  |  |  |  |  |  | 0,1,2,3 |  |  |  |
| T310 Timer |  |  |  |  |  |  |  | ms |  | 1000 |  |  |  |
| N310 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |
| T1 |  |  |  |  |  |  |  | s |  | 1 |  | During this time the the UE shall be fully synchronized to cell 1 |  |
| T2 |  |  |  |  |  |  |  | s |  | TBD |  |  |  |
| T3 |  |  |  |  |  |  |  | s |  | TBD |  |  |  |
| T4 |  |  |  |  |  |  |  | s |  | TBD |  |  |  |
| T5 |  |  |  |  |  |  |  | s |  | TBD |  |  |  |
| D1 |  |  |  |  |  |  |  | s |  | TBD |  |  |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.5.7.1-3: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0,0 | Config 1, 4 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2, 5 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3, 6 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q0,1 | Config 7,10 | dB | 5 | 5 | 5 | 5 | 5 |
|  | Config 8, 11 |  | 5 | 5 | 5 | 5 | 5 |
|  | Config 9, 12 |  | 5 | 5 | 5 | 5 | 5 |
| SNR_SSB of set q1,0 | Config 1, 4 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 2, 5 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 3, 6 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| SNR_SSB of set q1,1 | Config 7, 10 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 8, 11 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 9, 12 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| SSB_RP of set q1,0 | Config 1, 4 | dBm/SCS | -108 | -108 | -88 | -88 | -88 |
|  | Config 2, 5 | kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 3, 6 |  | -105 | -105 | -85 | -85 | -85 |
| SSB_RP of set q1,1 | Config 7, 10 | dBm/SCS | -108 | -108 | -88 | -88 | -88 |
|  | Config 8, 11 | kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 9, 12 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1, 4 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2, 5 |  | -98 |  |  |  |  |
|  | Config 3, 6 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.4.5.5.7.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.4.5.5.7.2	Test Requirements

Test requirements are applied to TRP specific report respectively on (q0,0), (q0,1) for TRP 1 and (q1,0), (q1,1)  for TRP 2 repectively as Figure A.4.5.5.7.1-1.

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 2 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 2.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1,0 and q1,1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 1>

<Start of Change 2>

#### A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.5.5.5.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the TRP specific CSI-RS-based beam failure in the set (q0,0), (q0,1) configured for a serving PSCell and a cell with PCID different from the serving cell, and that the UE performs correct CSI-RS-based link recovery based on beam candidate set  (q1,0) and (q1,1). The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.5.5.5.8.1-1, A.5.5.5.8.1-2, A.5.5.5.8.1-3, and A.5.5.5.8.1-4 below. There are three cells, cell 1 is the E-UTRAN PCell, cell 2 is the PSCell and cell 3 is the cell with different PCID in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.5.8.1-1 shows the variation of the downlink SNR of the PSCell and the SNR of the CSI-RS in set q0 in the active PSCell to emulate CSI-RS based beam failure. Figure A.5.5.5.8.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.5.5.5.8.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.5.5.5.8.1-2: General test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Test Config. | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  | 1-2 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  | 1-2 |  | 1 |  |
| Active PSCell |  | 1-2 |  | Cell 2 |  |
| RF Channel Number |  | 1-2 |  | 2 |  |
| Duplex mode |  | 1-2 |  | TDD |  |
| TDD Configuration |  | 1-2 |  | TDDConf.3.1 |  |
| BWchannel |  | 1-2 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1-2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | 1-2 | kHz | 120 |  |
| DL initial BWP configuration |  | 1-2 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | 1-2 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | 1-2 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | 1-2 |  | ULBWP.1.1 |  |
| PDSCH Reference Channel |  | 1-2 |  | SR.3.2 TDD |  |
| RMSI CORESET Reference Channel |  | 1-2 |  | CR.3.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1-2 |  | CCR.3.1 TDD |  |
| OCNG parameters |  | 1-2 |  | OP.1 |  |
| CP length |  | 1-2 |  | Normal |  |
| PDSCH/PDCCH TCI state |  | 1-2 |  | TCI.State.0 |  |
| CSI-RS for tracking |  | 1-2 |  | TRS.2.1 TDD |  |
| SSB Configuration |  | 1-2 |  | SSB.1 FR2 |  |
| SMTC Configuration |  | 1-2 |  | SMTC.3 |  |
| PRACH Configuration |  | 1-2 |  | FR2 PRACH configuration 4 | A.3.8.3.4 |
| DRX configuration |  | 1-2 |  | DRX.3 | A.3.3.3 |
| CSI-RS configuration for TRP0 |  | 1-2 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for TRP1 |  |  |  | CSI-RS.3.6 TDD | A.3.14.2 |
| CSI-RS index assigned as BFD RS (q0,0) |  | 1-2 |  | CSI-RS#0 |  |
| CSI-RS index assigned as CBD RS (q1,0) |  | 1-2 |  | CSI-RS#1 |  |
| CSI-RS index assigned as BFD RS (q0,1) |  | 1-2 |  | CSI-RS#2 |  |
| CSI-RS index assigned as CBD RS (q1,1) |  | 1-2 |  | CSI-RS#3 |  |
| CSI-RS index assigned as RLM RS |  | 1-2 |  | 0,1,2,3 |  |
| Beam failure detection transmission parameters | DCI format | 1-2 |  | 1-0 |  |
|  | Number of Control OFDM symbols | 1-2 |  | 2 |  |
|  | Aggregation level | 1-2 | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 1-2 | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 1-2 | dB | 0 |  |
|  | DMRS precoder granularity | 1-2 |  | REG bundle size |  |
|  | REG bundle size | 1-2 |  | 6 |  |
| Gap pattern ID |  | 1-2 |  | N/A |  |
| rlmInSyncOutOfSyncThreshold |  | 1-2 |  | absent | Value 0 is applied. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | 1-2 | dBm/SCS | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  | 1-2 |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  | 1-2 |  | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  | 1-2 |  | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | 1-2 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| reportConfigType |  | 1-2 |  | periodic |  |
| reportQuantity |  | 1-2 |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | 1-2 | slot | 40 |  |
| CSI reporting offset |  | 1-2 | slot | 4 |  |
| T310 |  | 1-2 | ms | 1000 |  |
| N310 |  | 1-2 |  | 2 |  |
| T1 |  | 1-2 | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | 1-2 | s | TBD |  |
| T3 |  | 1-2 | s | TBD |  |
| T4 |  | 1-2 | s | TBD |  |
| T5 |  | 1-2 | s | TBD |  |
| D1 |  | 1-2 | s | TBD |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.5.5.5.8.1-3: Cell specific test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 defined in A.3.15 |  |  |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | Config 1-2 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1,0 | Config 1-2 | dB | 5 | 5 | 5 | 5 | 5 |
| SNR_CSI-RS of set q0,1 | Config 1-2 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| SNR_CSI-RS of set q1,1 | Config 1-2 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1,0 | Config 1-2 | dBm/SCS | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| CSI-RS_RP of set q1,1 | Config 1-2 | dBm/SCS | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 1-2 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.5.5.5.4.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |

Figure A.5.5.5.8.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.5.5.5.8.2	Test Requirements

Test requirements are applied to TRP specific report respectively on (q0,0), (q0,1) for cell-2 and (q1,0), (q1,1)  for cell-3 repectively as Figure A.5.5.5.8.1-1.

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 2>

<Start of Change 3>

#### A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.6.5.5.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects TRP specific CSI-RS-based beam failure in the sets q0,0  and q0,1  configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1,0 and q1,1. The purpose is to test the downlink monitoring for beam failure detection on TRP1 within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based TRP specific beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.18.

The test parameters are given in Tables A.6.5.5.X7.1-1, A.6.5.5.X7.1-2, A.6.5.5.X7.1-3, and A.6.5.5.X7.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.X7.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0,0  and q0,1  for TRP1 and TRP2 respectively to emulate CSI-RS based beam failure on TRP1. Figure A.6.5.5.X7.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1,0 and q1,1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5  ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.X7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.X7.1-2: General test parameters for FR1 PCell for CSI-RS-based TRP specific beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  |  |  |  |  | Unit |  |  | Value |  |  |  |  | Comment |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Test1 |  |  |  |  |  |  |  |  |  |
| Active PCell |  |  |  |  |  |  |  |  |  |  |  | Cell 1 |  |  |  |  |  |  |  |  |  |
| RF Channel Number |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |
| Duplex mode |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 2, 3 |  |  |  |  |  |  |  |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD Configuration |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  | A.3.1.2 |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | CCR.1.1 FDD |  |  |  |  | A.3.1.3 |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | SSB.3 FR1 |  |  |  |  | A.3.10 |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | SSB.3 FR1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | SSB.4 FR1 |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  | Config 1, 2 |  |  |  |  |  |  |  |  | SMTC.1 |  |  |  |  | A.3.11 |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  |  |  |  |  |  |  |  | 15 KHz |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | 30 KHz |  |  |  |  |  |  |  |  |  |  |
| PRACH Configuration |  |  |  | Config 1, 2,3 |  |  |  |  |  |  |  |  | FR1 PRACH configuration 4 |  |  |  |  | A.3.8.2 |  |  |  |  |  |
| OCNG parameters |  |  |  |  |  |  |  |  |  |  |  |  | OP.1 |  |  |  |  | A.3.2.1 |  |  |  |  |  |
| CP length |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |  |  |  |  |  |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  |  |  |  |  |  |  |  | 1-0 |  |  |  |  |  |  |  |  |  |  |
|  | Number of Control OFDM symbols |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  | Aggregation level |  |  |  |  |  |  |  |  | CCE |  |  | 8 |  |  |  |  |  |  |  |  |  |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  |  |  |  |  |  | dB |  |  | 0 |  |  |  |  |  |  |  |  |  |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  |  |  |  |  |  | dB |  |  | 0 |  |  |  |  |  |  |  |  |  |  |
|  | DMRS precoder granularity |  |  |  |  |  |  |  |  |  |  |  | REG bundle size |  |  |  |  |  |  |  |  |  |  |
|  | REG bundle size |  |  |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| DRX |  |  |  |  |  |  |  |  |  |  |  |  | DRX.7 |  |  |  |  | A.3.3.7 |  |  |  |  |  |
| Gap pattern ID |  |  |  |  |  |  |  |  |  |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  |  |  |  |  |  |  |  | absent |  |  |  |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |  |  |  |  |  |
| rsrp-ThresholdSSB |  |  |  |  | Config 1, 2 |  |  |  |  |  | dBm/SCS kHz |  |  | -98 |  |  |  |  | Threshold used for Qin_LR_SSB |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | -95 |  |  |  |  |  |  |  |  |  |
| powerControlOffsetSS |  |  |  |  |  |  |  |  |  |  |  |  |  | db0 |  |  |  |  | Used for deriving rsrp-ThresholdCSI-RS |  |  |  |  |
| beamFailureInstanceMaxCount |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | see clause 5.17 of TS 38.321 [7] |  |  |  |  |
| beamFailureDetectionTimer |  |  |  |  |  |  |  |  |  |  |  |  |  | pbfd4 |  |  |  |  | see clause 5.17 of TS 38.321 [7] |  |  |  |  |
| CSI-RS configuration for TRP0 |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | CSI-RS.1.2 FDD |  |  |  |  | A.3.14.1 |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | CSI-RS.1.2 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | CSI-RS.2.2 TDD |  |  |  |  |  |  |  |  |  |
| CSI-RS configuration for TRP1 |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | CSI-RS.1.7 FDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | CSI-RS.1.6 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | CSI-RS.2.7 TDD |  |  |  |  |  |  |  |  |  |
| CSI-RS Index assigned as BFD RS (q0,0) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |
| CSI-RS Index assigned as CBD RS (q1,0) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |
| CSI-RS Index assigned as BFD RS (q0,1) |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |
| CSI-RS Index assigned as CBD RS (q1,1) |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |
| CSI-RS index assigned as RLM RS |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,1,2,3 |  |  |  |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | CSI-RS.1.1 FDD |  |  |  |  | A.3.14.1 |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  |  |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 2 |  |  |  |  |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Config 3 |  |  |  |  |  |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |
| T310 Timer |  |  |  |  |  |  |  |  |  |  | ms |  |  | 1000 |  |  |  |  |  |  |  |  |  |
| N310 |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |
| T1 |  |  |  |  |  |  |  |  |  |  | s |  |  | 1 |  |  |  |  | During this time the the UE shall be fully synchronized to cell 1 |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  | s |  |  | [10.81] |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  | s |  |  | [10.28] |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  | s |  |  | 0 |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  | s |  |  | [0.57] |  |  |  |  |  |  |  |  |  |
| D1 |  |  |  |  |  |  |  |  |  |  | s |  |  | [0.53] |  |  |  |  |  |  |  |  |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.5.X7.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q0,1 | Config 1 | dB | 5 | 5 | 5 | 5 | 5 |
|  | Config 2 |  | 5 | 5 | 5 | 5 | 5 |
|  | Config 3 |  | 5 | 5 | 5 | 5 | 5 |
| SNR_CSI-RS of set q1,0 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 2 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 3 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1,0 | Config 1 | dB/SCS kHz | -110 | -110 | -88 | -88 | -88 |
|  | Config 2 |  | -110 | -110 | -88 | -88 | -88 |
|  | Config 3 |  | -107 | -107 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.6.5.5.X7.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.6.5.5.7.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1,0.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1,0. The UE shall not transmit preamble on a beam associated with the candidate beam set q1,0 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 3>

<Start of Change 4>

#### A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.7.5.5.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects TRP specific SSB-based beam failure and link recovery in the sets q0,0 and q0,1 for TRP1 when DRX is used for an FR2 active Scell requirements in clause 8.18.

The test parameters are given in Tables A.7.5.5.X8.1-1, A.7.5.5.X8.1-2 and A.7.5.5.X8.1-3. There are two cell, cell 1 is the active PCell and cell 2 is the active SCell, in the test. SCell is configured with two TRPs. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.X8.1-1shows the variation of the downlink SNR of the CSI-RS in set q0,0 and q1,0 in the active SCell for TRP1 and TRP2 respectively. Figure A.7.5.5.X8.1-1additionally shows the variation of the downlink L1-RSRP of the CSI-RS in q0,1  for TPR1. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.5.X8.1-1: Supported test configurations for FR2 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.5.X8.1-2: General test parameters for FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  | Unit |  | Value | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Test 1 |  |  |  |
| Active PCell |  |  |  |  |  |  | Cell 1 |  |  |  |
| RF Channel Number for PCell |  |  |  |  |  |  | 1 |  |  |  |
| Active SCell |  |  |  |  |  |  | Cell 2 |  |  |  |
| RF Channel Number for SCell |  |  |  |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 |  |  |  | TDD |  |  |  |
| BW channel |  |  | Config 1 |  |  |  | 100: NRB,c = 66 |  |  |  |
| DL initial BWP configuration |  |  | Config 1 |  |  |  | DLBWP.0.1 |  |  |  |
| DL dedicated BWP configuration |  |  | Config 1 |  |  |  | DLBWP.1.1 |  |  |  |
| UL initial BWP configuration |  |  | Config 1 |  |  |  | ULBWP.0.1 |  |  |  |
| UL dedicated BWP configuration |  |  | Config 1 |  |  |  | ULBWP.1.1 |  |  |  |
| TDD Configuration |  |  | Config 1 |  |  |  | TDDConf.3.1 |  |  |  |
| CORESET Reference Channel |  |  | Config 1 |  |  |  | CR.3.1 TDD | A.3.1.2 |  |  |
| SSB Configuration |  |  | Config 1 |  |  |  | SSB.3 FR2 | A.3.11 |  |  |
| SMTC Configuration |  |  | Config 1 |  |  |  | SMTC.3 | A.3.10 |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1 |  |  |  | 120 KHz |  |  |  |
| PRACH Configuration |  |  | Config 1 |  |  |  | Table A.3.8.3.1-1 |  |  |  |
| CSI-RS configuration for BFD/RLM |  |  | Config 1 |  |  |  | CSI-RS.3.2 TDD | A.3.14.2 |  |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  |  | CSI-RS.3.1 TDD | A.3.14.2 |  |  |
| TRS configuration |  |  | Config 1 |  |  |  | TRS.2.1 TDD |  |  |  |
| TCI configuration |  |  | Config 1 |  |  |  | CSI-RS.Config.0 |  |  |  |
| OCNG parameters |  |  |  |  |  |  | OP.1 |  |  |  |
| CP length |  |  |  |  |  |  | Normal |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  | 2x2 Low |  |  |  |
| OCNG parameters |  |  |  |  |  |  | OP.1 | A.3.2.1 |  |  |
| CP length |  |  |  |  |  |  | Normal |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  |  | 2x2 Low |  |  |  |
| Beam failure |  |  | DCI format |  |  |  | 1-0 |  |  |  |
| detection transmission parameters |  |  | Number of Control OFDM symbols |  |  |  | 2 |  |  |  |
|  |  |  | Aggregation level |  | CCE |  | 8 |  |  |  |
|  |  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB |  | 0 |  |  |  |
|  |  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB |  | 0 |  |  |  |
|  |  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |  |  |
|  |  |  | REG bundle size |  |  |  | 6 |  |  |  |
| DRX |  |  |  |  |  |  | DRX.3 |  |  |  |
| Gap pattern ID |  |  |  |  |  |  | N.A. |  |  |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  |  |  |  | Configured |  |  |  |
| Periodicity of PUCCH for SR configuration for BFR on SCell |  |  |  |  | Slot |  | 40 | 5ms |  |  |
| Offset of PUCCH for SR configuration for BFR on SCell |  |  |  |  | Slot |  | 5 |  |  |  |
| PUCCH parameters for SR configuration for BFR on SCell |  |  |  |  |  |  | Table 8.3.3.1.2-1 in [13] |  |  |  |
| csi-RS-Index assigned as BFD RS in set (q00) |  |  |  |  |  |  | 0 |  |  |  |
| csi-RS-Index assigned as BFD RS in set (q01) |  |  |  |  |  |  | 1 |  |  |  |
| CSI-RS index assigned as RLM RS |  |  |  |  |  |  | 0, 1 |  |  |  |
| SSB Index assigned as CBD RS in set (q10) |  |  |  |  |  |  | 0 |  |  |  |
| SSB Index assigned as CBD RS in set (q11) |  |  |  |  |  |  | 1 |  |  |  |
| SSB index assigned as RLM RS |  |  |  |  |  |  | 0, 1 |  |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |  |  |
| rsrp-ThresholdBFR |  | Config 1 |  |  | dBm/SCS |  | -95 | Threshold used for Qin_LR_SSB |  |  |
| powerControlOffsetSS |  |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |  |  |
| beamFailureInstanceMaxCount |  |  |  |  |  |  | n1 | see TS 38.321 [7], clause 5.17 |  |  |
| beamFailureDetectionTimer |  |  |  |  |  |  | pbfd4 | see TS 38.321 [7], clause 5.17 |  |  |
| T310 Timer |  |  |  |  | ms |  | 1000 |  |  |  |
| N310 |  |  |  |  |  |  | 2 |  |  |  |
| T1 |  |  |  |  | s |  | 1 | During this time the the UE shall be fully synchronized to cell 1 |  |  |
| T2 |  |  |  |  | s |  | [10.81] |  |  |  |
| T3 |  |  |  |  | s |  | [10.28] |  |  |  |
| T4 |  |  |  |  | s |  | 0 |  |  |  |
| T5 |  |  |  |  | s |  | [0.57] |  |  |  |
| D1 |  |  |  |  | s |  | [0.53] |  |  |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.5.X8.1-3: Cell specific test parameters for FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter | Unit | Cell1 | Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | Setup 3 as specified in clause A.3.15 Note 11 |  |  |  |  |  |
| Assumption for UE beams Note 10 |  | Rough | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1,0 | dB | 5 | 5 | 5 | 5 | 5 | 5 |
| SNR_CSI-RS of set q0,1 | dB | 0.2 | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q0,1 | dBm/ SCS kHz | -104.5 | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| Noc | dBm/120 kHz | -104.7 | -104.7 |  |  |  |  |
| Propagation condition |  | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: AoA1 for PCell and TRP1 of SCell , AoA2 for TRP2 of SCell |  |  |  |  |  |  |  |

Figure A.7.5.5.X8.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing for SCell in DRX mode

##### A.7.5.5.9.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 in A.7.5.5.X8.1 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1 for TRP1 and TRP2.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 2 TRP1 and TRP2 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3, T4, T5, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1 for TRP2.

During T3 the UE shall detect beam failure and initiate link recovery for TRP1. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q0,1.

No later than time point F occurring no later than D1 = [520]+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q0,1. The UE shall not transmit PUCCH with an LRR with the candidate beam set q0,1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.7.5.5.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects TRP specific SSB-based beam failure and link recovery in the sets q0,0 and q0,1 for TRP1 when no DRX is used for an FR2 serving cell requirements in clause 8.18.

The test parameters are given in Tables A.7.5.5.X9.1-1, A.7.5.5.X9.1-2, A.7.5.5.X9.1-3 and A.7.5.5.X9.1-4 below. There is one active serving cell configured with two TRPs in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.X9.1-1 shows the variation of the downlink SNR of the SSB in set q0,0 and q1,0 for TRP1 and TRP2 respectively. Figure A.7.5.5.X9.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q0,1  for TPR1.

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.7.5.5.X9.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.7.5.5.X9.1-2: General test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  | Unit |  |  | Value |  |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Test 1 |  |  | Same configuration for both TRP whereever applicbale |  |  |
| Active PCell |  |  |  |  |  |  | Cell 1 |  |  |  |  |  |
| RF Channel Number for PCell |  |  |  |  |  |  | 1 |  |  |  |  |  |
| Duplex mode |  |  | Config 1,2 |  |  |  | TDD |  |  |  |  |  |
| TDD Configuration |  |  | Config 1,2 |  |  |  | TDDConf.3.1 |  |  |  |  |  |
| BW channel |  |  | Config 1,2 |  |  |  | 100: NRB,c = 66 |  |  |  |  |  |
| Data RBs allocated |  |  | Config 1,2 |  |  |  | 66 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz |  |  | 120 |  |  |  |  |  |
| DL initial BWP configuration |  |  | Config 1,2 |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration |  |  | Config 1,2 |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration |  |  | Config 1,2 |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration |  |  | Config 1,2 |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| PDSCH Reference Channel |  |  | Config 1 |  |  |  | SR.3.2 TDD |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | SR.3.3 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  |  |  | CR.3.1 TDD |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | CR.3.2 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  |  |  | CCR.3.1 TDD |  |  | A.3.1.2 |  |  |
|  |  |  | Config 2 |  |  |  | CCR.3.7 TDD |  |  |  |  |  |
| OCNG parameters |  |  | Config 1,2 |  |  |  | OP.1 |  |  |  |  |  |
| CP length |  |  | Config 1,2 |  |  |  | Normal |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  | Config 1,2 |  |  |  | TCI.State.0 |  |  |  |  |  |
| CSI-RS for tracking |  |  | Config 1,2 |  |  |  | TRS.2.1 TDD |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1,2 |  |  |  | CSI-RS.3.1 TDD |  |  |  |  |  |
| reportConfigType |  |  | Config 1,2 |  |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | Config 1,2 |  |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity |  |  | Config 1,2 | slot |  |  | 40 |  |  |  |  |  |
| CSI reporting offset |  |  | Config 1,2 | slot |  |  | 4 |  |  |  |  |  |
| SSB Configuration |  |  | Config 1 |  |  |  | SSB.1 FR2 |  |  |  |  |  |
|  |  |  | Config 2 |  |  |  | SSB.2 FR2 |  |  |  |  |  |
| SMTC Configuration |  |  | Config 1,2 |  |  |  | SMTC.3 |  |  | A.3.10 |  |  |
| PRACH Configuration |  |  | Config 1,2 |  |  |  | FR2 PRACH configuration 2 |  |  | A.3.8.3.2 |  |  |
| DRX configuration |  |  | Config 1,2 |  |  |  | OFF |  |  |  |  |  |
| SSB Index assigned as BFD RS (q0,0) |  |  | Config 1,2 |  |  |  | 0 |  |  |  |  |  |
| SSB Index assigned as CBD RS (q1,0) |  |  | Config 1,2 |  |  |  | 1 |  |  |  |  |  |
| SSB Index assigned as BFD RS (q0,1) |  |  | Config 1,2 |  |  |  | 2 |  |  |  |  |  |
| SSB Index assigned as CBD RS (q1,1) |  |  | Config 1,2 |  |  |  | 3 |  |  |  |  |  |
| SSB Index assigned as RLM RS |  |  | Config 1,2 |  |  |  | 0,1,2,3 |  |  |  |  |  |
| Beam failure detection transmission parameters |  |  | DCI format |  |  |  | 1-0 |  |  |  |  |  |
|  |  |  | Number of Control OFDM symbols |  |  |  | 2 |  |  |  |  |  |
|  |  |  | Aggregation level | CCE |  |  | 8 |  |  |  |  |  |
|  |  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB |  |  | 0 |  |  |  |  |  |
|  |  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB |  |  | 0 |  |  |  |  |  |
|  |  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |  |  |  |  |
|  |  |  | REG bundle size |  |  |  | 6 |  |  |  |  |  |
| Gap pattern ID |  |  |  |  |  |  | gp0 |  |  |  |  |  |
| gapOffset |  |  |  |  |  |  | 0 |  |  |  |  |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  |  | absent |  |  | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |  |  |
| rsrp-ThresholdBFR | Config 1 |  |  | dBm/SCS |  |  | -95 |  |  | Threshold used for Qin_LR_SSB |  |  |
|  | Config 2 |  |  |  |  |  | -92 |  |  |  |  |  |
| powerControlOffsetSS |  |  |  |  |  |  | db0 |  |  | Used for deriving rsrp-ThresholdCSI-RS |  |  |
| beamFailureInstanceMaxCount |  |  |  |  |  |  | n1 |  |  | see TS 38.321 [7], clause 5.17 |  |  |
| beamFailureDetectionTimer |  |  |  |  |  |  | pbfd4 |  |  | see TS 38.321 [7], clause 5.17 |  |  |
| T310 Timer |  |  |  | ms |  |  | 1000 |  |  |  |  |  |
| N310 |  |  |  |  |  |  | 2 |  |  |  |  |  |
| T1 |  |  |  | s |  |  | 1 |  |  | During this time the the UE shall be fully synchronized to cell 1 |  |  |
| T2 |  |  |  | s |  |  | 2.61 |  |  |  |  |  |
| T3 |  |  |  | s |  |  | 1.64 |  |  |  |  |  |
| T4 |  |  |  | s |  |  | 0 |  |  |  |  |  |
| T5 |  |  |  | s |  |  | 1.01 |  |  |  |  |  |
| D1 |  |  |  | s |  |  | 0.97 |  |  |  |  |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.5.X9.1-3: Cell specific test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 as specified in clause A.3.15 Note 12 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0,0 | Config 1-2 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SNR_SSB of set q1,0 | Config 1-2 | dB | 5Note 11 | 5Note 11 | 5Note 11 | 5Note 11 | 5Note 11 |
| SNR_SSB of set q0,1 | Config 1-2 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 1 | dBm/ SCS | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| SSB_RP of set q0,1 |  |  |  |  |  |  |  |
|  | Config 2 |  | -101.5 | -101.5 | -81.5 | -81.5 | -81.5 |
|  | Config 1,2 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband Note 12: AoA1 for TRP1 and AoA2 for TRP2 |  |  |  |  |  |  |  |

Table A.7.5.5.X9.1-4: Void

Figure A.7.5.5.X9.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.7.5.5.10.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1 for TRP 1 and TRP2.

During the period from time point A to time point B the UE shall transmit uplink signal for TRP 1 and TRP2 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3, T4, T5, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission for TRP 2.

During T3 the UE shall detect beam failure and initiate link recovery for TRP 1. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q0,1.

No later than time point F occurring no later than D1 = [960]+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q0,1 for TRP1. The UE shall not transmit preamble on a beam associated with the candidate beam set q0,1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 4>