3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216328

Electronic Meeting, 10 – 19 October, 2022

Title: 	On measurement accuracy for PDC enhancements

Source: 	Huawei, HiSilicon

Agenda item:	4.7.2.2

Document for:	Discussion

Introduction

RRM requirements for PDC measurement are discussed in RAN4#104-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- Accuracy numbers for TRS based PDC

- Separation of BB and RF error

In this paper we will provide our views on remaining issues for PDC accuracy requirements.

Discussion

Accuracy numbers for TRS based PDC

RAN4 has agreed to define accuracy requirements for TRS based PDC based on 4 sample and AWGN. In [2] the moderator provided the summary of simulation results submitted to RAN4#104-e. It is noted that some companies have also provided simulation results in RAN4#103-e [3][4], and Table 1 summarizes all the results submitted so far.

Table 1: Summary of UE Rx-Tx accuracy for AWGN with

| FR | SCS (kHz) | RB | Accuracy (Tc) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | E/// | Nokia | QC | vivo | HW | Average |
| FR1 | 15 | 24 | 114 | 64 | 121 | 128 | 88 | 103 |
|  |  | 52 | 59 | 32 | 57 | 62 | 56 | 53 |
|  |  | 104 | 29 | 16 | 30 | 30 | 24 | 26 |
|  | 30 | 24 | 58 | 88 | 56 | 62 | 40 | 61 |
|  |  | 48 | 29 | 68 | 28 | 31 | 28 | 37 |
|  |  | 132 | 14 | 40 | 15 | 15 | 16 | 20 |
|  | 60 | 24 | 29 | 64 | [29] | 30 | [24] | 37 |
|  |  | 52 | 14 | 40 | [14] | 15 | [14] | 21 |
|  |  | 132 | 7 | 32 | [7] | 7 | [8] | 14 |
| FR2 | 60 | 24 | 29 | 20 | 29 | 30 | 24 | 26 |
|  |  | 64 | 14 | 8 | 14 | 15 | 14 | 13 |
|  |  | 132 | 7 | 4 | 7 | 7 | 8 | 7 |
|  | 120 | 32 | 14 | 8 | 14 | 15 | 16 | 13 |
|  |  | 64 | 7 | 4 | 7 | 7 | 7 | 6 |
|  |  | 128 | 4 | 2 | 4 | 4 | 4 | 4 |

It is noted that two companies have not provided results for 60kHz FR1, but since it is AWGN channel, the results for FR1 and FR2 should be rather similar, so in Table 1 the results for FR2 are used in the cells for  FR1 with []. The table can be further updated if there are new results coming.

It is suggested to use the average value in Table 1 for defining the accuracy number for TRS based PDC.

Proposal 1: Use the average value in Table 1 for defining the accuracy number for TRS based PDC.

Separation of BB and RF error

In last meeting, it is agreed in the Rel-16 POS discussion to separate BB and RF calibration error for UE Rx-Tx in different tables [5], and the reason is that the RF error does not depend on PRS BW but also on SRS BW. We understand the same applies for UE Rx-Tx for PDC, so it is suggested to capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC.

Proposal 2: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC.

Conclusions

In this paper we provided our views on remaining issues for PDC accuracy requirements.

Proposal 1: Use the average value in Table 1 for defining the accuracy number for TRS based PDC.

Proposal 2: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC.

Reference

- R4-2214338, WF on NR_IIOT_URLLC_enh, Moderator (Nokia)

- R4-2214339, Collection of simulation result for NR_IIOT_URLLC_enh, Nokia, Nokia Shanghai Bell

- R4-2208821, Updated simulation results for TRS based PDC, vivo

- R4-2209237, Simulation results for TRS based PDC, Huawei, Hisilicon

- R4-2214689, DraftCR – Margins for UE Rx-Tx measurement accuracy requirements, Qualcomm Inc.