3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216333

# Electronic Meeting, 10 – 19 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to introduce SDT RRC TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test cases for SDT needs to be introduced to verify the core requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases for SDT RRM are introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM core requirements are not verified for SDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.X (new), A.7.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.2.X	Small Data Transmission

A.6.2.X.1	TA validation for CG-SDT in FR1

A.6.2.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly performs TA validation for CG-SDT. This test will partly verify the TA validation requirements in clause 5.5.3.

Supported test configurations are shown in table A.6.2.X.1.1-1. The test parameters are given in Tables A.6.2.X.1.1-2 and A.6.2.X.1.1-3.

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.2.X.1.1-1 shows the variation of the RSRP level in the active cell during the test.

During T1 UE is in CONNECTED mode, and T1 ends at the slot #n when the last PDSCH containing RRCRelease message #1 (containing CG-SDT configuration) is transmitted, which is denoted as point A. During T1, the RSRP level of the serving cell is changed from rsrp_low to rsrp_high at point X, and point X is earlier than point A by [680]ms.

T2 starts at point A and ends at the slot #m when the last PDSCH containing RRCRelease message #2 (containing CG-SDT configuration) is transmitted, which is denoted as point D. During T2 UE is INACTIVE mode. UE is triggered to transmit UL data at point B, which is [AB] ms later than point A. The test equipment will observe whether UE transmits on CG-SDT occasion after point B, and the time point when UE transmits using CG-SDT is denoted as point C. During the subsequent transmission after point C, test equipment will transmit RRCRelease message #2 at point D, which is [CD] ms after point C.

T3 starts at point D, i.e. the slot #m when the last PDSCH containing RRCRelease message #2 is transmitted. During T3 UE is INACTIVE mode. UE is triggered to transmit UL data at point E, which is [1360]ms later than point D. In between point D and E, the RSRP level is changed from rsrp_high to rsrp_low at point Y. The distance between point Y and D is [680]ms, and the distance between point Y and E is [680]ms. The test equipment will observe whether UE transmits on CG-SDT occasion after point E until the end of T3.

Table A.6.2.X.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.2.X.1.1-2: General test parameters for TA validation for CG-SDT in FR1

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |
|  | Config 2 |  | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| PRACH Configuration | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  | Config 3 |  | Table A.3.8.2.1-1 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  | s | 1.28 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | [2] |
| cg-SDT-TimeAlignmentTime |  |  | infinity |
| CG-SDT resource period |  | ms | 40 |
| T1 |  | s | [1] |
| T2 |  | s | [1] |
| T3 |  | s | [2.5] |

Table A.6.2.X.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR1

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Before X | X to Y | After Y |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
|  | Config 1 | dB | 0 | 5 | 0 |
|  | Config 2 |  | 0 | 5 | 0 |
|  | Config 3 |  | 0 | 5 | 0 |
|  | Config 1 | dB | 0 | 5 | 0 |
|  | Config 2 |  | 0 | 5 | 0 |
|  | Config 3 |  | 0 | 5 | 0 |
| SS-RSRP | Config 1 | dBm/SCS | -98 | -93 | -98 |
|  | Config 2 |  | -98 | -93 | -98 |
|  | Config 3 |  | -95 | -90 | -95 |
| Io | Config 1 | dBm/9.36 MHz | -67.04 | -63.85 | -67.04 |
|  | Config 2 | dBm/9.36 MHz | -67.04 | -63.85 | -67.04 |
|  | Config 3 | dBm/38.16 MHz | -60.94 | -57.76 | -60.94 |
| Propagation condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Figure A.6.2.X.1.1-1: RSRP variation for TA validation for CG-SDT

A.6.2.X.1.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

The UE should transmit UL data using a CG-ST occasion within [700]ms after point B.

The UE should not transmit UL data using CG-SDT occasion within [700]ms after point E.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 1>

<Start of Change 2>

A.7.2.X	Small Data Transmission

A.7.2.X.1	TA validation for CG-SDT in FR2

A.7.2.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly performs TA validation for CG-SDT. This test will partly verify the TA validation requirements in clause 5.5.3.

Supported test configurations are shown in table A.7.2.X.1.1-1. The test parameters are given in Tables A.7.2.X.1.1-2 and A.7.2.X.1.1-3.

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.2.X.1.1-1 shows the variation of the RSRP level in the active cell during the test.

During T1 UE is in CONNECTED mode, and T1 ends at the slot #n when the last PDSCH containing RRCRelease message #1 (containing CG-SDT configuration) is transmitted, which is denoted as point A. During T1, the RSRP level of the serving cell is changed from rsrp_low to rsrp_high at point X, and point X is earlier than point A by [520]ms.

T2 starts at point A and ends at the slot #m when the last PDSCH containing RRCRelease message #2 (containing CG-SDT configuration) is transmitted, which is denoted as point D. During T2 UE is INACTIVE mode. UE is triggered to transmit UL data at point B, which is [AB] ms later than point A. The test equipment will observe whether UE transmits on CG-SDT occasion after point B, and the time point when UE transmits using CG-SDT is denoted as point C. During the subsequent transmission after point C, test equipment will transmit RRCRelease message #2 at point D, which is [CD] ms after point C.

T3 starts at point D, i.e. the slot #m when the last PDSCH containing RRCRelease message #2 is transmitted. During T3 UE is INACTIVE mode. UE is triggered to transmit UL data at point E, which is [1040]ms ms later than point D. In between point D and E, the RSRP level is changed from rsrp_high to rsrp_low at point Y. The distance between point Y and D is [520]ms, and the distance between point Y and E is [520]ms. The test equipment will observe whether UE transmits on CG-SDT occasion after point E until the end of T3.

Table A.7.2.X.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.2.X.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 DD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 kHz |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  | s | 1.28 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | [2] |
| cg-SDT-TimeAlignmentTime |  |  | infinity |
| CG-SDT resource period |  | ms | 40 |
| T1 |  | s | [0.8] |
| T2 |  | s | [1.2] |
| T3 |  | s | [2.5] |

Table A.7.2.X.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Before X | X to Y | After Y |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 1 | dB | 0 | 8.5 | 0 |
|  | Config 1 | dB | 0 | 8.5 | 0 |
| SS-RSRP | Config 1 | dBm/SCS | -98 | -89.5 | -98 |
| Io | Config 1 | dBm/95.04 MHz | -66.00 | -59.94 | -66.00 |
| Propagation condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |

Figure A.7.2.X.1.1-1: RSRP variation for TA validation for CG-SDT

A.7.2.X.1.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

The UE should transmit UL data using a CG-ST occasion within [1000]ms after point B.

The UE should not transmit UL data using CG-SDT occasion within [1000]ms after point E.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 2>