3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216367

# Electronic Meeting, Oct. 10 – Oct. 19, 2022

Source: 	vivo

Title: 	Discussion on RRM aspects in R18 L1L2 mobility

Agenda Item:	6.20.3

Document for:	Discussion

Introduction

In RAN 94e meeting, WID on further NR mobility enhancements is agreed in [1]. In RAN4 #104e meeting, on L1/L2 mobility, the work plan [2] was approved with the following clarification.

- Discuss RRM core part requirements on

  - L1/L2-based inter-cell mobility (focus on requirements that have no dependency on RAN1/2 progress)

- Reply RAN1/2 LS, if any

According to RAN2 discussion, one LS was sent from RAN2 [3]. And 2 email discussions were handled after RAN2 #119e meeting, with moderator proposals captured in [4] and [5].

Moreover, some initial discussion on L1 L2 mobility was handled in RAN4 RRM session, and the agreements were captured in [6].

Based on all above information, we provide our views on the RRM aspects for L1L2 mobility.

Discussion

<On the delay components of inter-cell mobility>

In RAN2 discussion, some conclusions on time chart of inter-cell mobility before R18 enhancement were achieved in [4]. As captured in P2 of [4], it is assumed that TRS tracking and CSI-RS measurements are already included in current interruption model, which is within the scope of enhancements. However, based on RAN4 RRM requirements, the understanding of this conclusion would restrict the scope of R18 discussion. To illustrate the issue, firstly let us consider two kinds of mobility for which the legacy RRM requirements were specified.

- Type 1: Handover or PSCell addition/change, where PRACH transmission is generally considered as the end point for RRM requirements.

- Type 2: SCell activation, or activation of TCI that associated to the addition cell for R17 inter-cell beam managements(ICBM), where normally high-performance data transmission is generally considered as the end point for RRM requirements.

For Type 1, normally the high-performance data transmission is not ensured at the end point of the HO procedure. This is partly due to the lack of DL CSI information at gNB side, before any DL CSI measurement and feedback is done by the UE, which may be further discussed in RAN1. From RAN4 RRM perspective, note that when high-performance data transmission is considered, as the Type 2 does, TRS tracking and CSI feedback delay needs to be counted in the procedure. Note that in R17 ICBM, since serving cell is not changed, CSI feedback of additional cell can be done, as long as the TCI of CSI-RSs configured in CSImeasConfig is associated to the SSB of additional cell.

Observation 1  In SCell activation and R17 ICBM, high-performance data transmission can be ensured at the end of the related procedures, from RRM requirements perspective.

In the discussion of R17 ICBM, RAN4 has only specified RRM requirements for the case when the additional cell is known. The known condition of additional cell is similar to the SCell activation and PSCell addition, except that timing offset between serving cell and the cell with the additional PCI is restricted to be within CP. The main part of known conditions is that L3 measurements has to be done within a specific duration before the cell is actually used.

As in legacy RRM requirements for SCell activation, when the corresponding cell is known and the AGC state is not out-of-date or can be re-used from other intra-band serving cell(s), one SSB for tracking can be counted in the RRM requirements for the SCell activation, which is similar to the TCI activation. In this case, the SSB is actually used for fine timing sync and the procedure is actually the TRS tracking. Note that in the known condition of a FR2 SCell, the TCI to be used in the SCell is restricted to be selected based on one of the latest reported SSB indexes in MR.

When the corresponding cell is unknown, TRS tracking and CSI measurement reporting need to be counted in the related procedure when necessary. Note that in this case, normally 3 or 4 SSBs in the SMTC are used for AGC settling and cell search. The TCI state indication can be a separate procedure after L1 measurements of SCell are performed by UE and the corresponding L1 report is available at gNB side, as quoted below. These L1 procedures are normally considered after the cell search procedure. Since the cell is unknown, normally the TCI state of the cell is also unknown, especially for FR2, i.e. the SSB index of the cell to be used by the gNB is not sent by UE in the measurement report in advance.

TS 38.133 v17.7.0, clause 8.3.2: If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Observation 2  From RAN4 RRM perspective, the TRS tracking can be merged with the T_firstSSB only when the corresponding cell is known, i.e. corresponding L3 measurements report is sent by UE within a pre-defined period before the L1L2 activation signalling is received. For FR2, the known conditions of corresponding cell to be activated by L1L2 signaling have further restricted that, the SSB index associated to the TCI state of the SCell has to be one of the reported SSB indexes in MR.

Note that for HO or PSCell addition/change, at least for FR2, the known condition of target cell does not include the restriction that the SSB index associated to the TCI state of the target cell, has to be one of the reported SSB indexes in MR. Moreover, the end point also shows that the high-performance data transmission is not always assumed to be ensured.

Observation 3 High-performance data transmission is not always ensured after legacy HO or PSCell addition/change, from RRM requirements perspective.

Proposal 1  RAN4 to align common understanding on whether TRS tracking for high-performance data transmission is always considered in legacy mobility procedure, including HO, PSCell addition/change, SCell activation, activation of TCI from cell with additional PCI.

For R18 L1L2 based mobility, in our understanding, the enhancement should be discussed to shorten the overall delay, i.e. the latency from the time when L1L2 mobility command is received to the time when the high-performance data transmission can be ensured. This would cover the scenario that assumed in R17 ICBM. In other word, the enhancements should always consider the TRS tracking for high-performance data transmission, no matter it is considered in legacy mobility latency or not.

Proposal 2  TRS tracking for high-performance data transmission should always be considered in R18 L1L2 mobility related enhancements, no matter it is considered in legacy mobility latency or not.

<On the definition of ‘inter-frequency’>

In WID, the ‘inter-frequency’ is explicitly mentioned. For R18 L1L2 mobility, in our understanding, the enhancements compared to R17 inter-cell BM, regarding ‘inter-frequency’, would be that the scenario of inter-frequency inter-cell mobility would need to be supported in R18. As stated in RAN2 LS[3], RAN2 will focus on ‘PCell mobility’, and therefore we take PCell mobility in example, as illustrated in Figure 1.

Figure 1 Illustration of inter-frequency L1L2 based moblity

As shown in Figure 1, in our understanding, in R18, for the L1L2 mobility, the source cell and target cell are not necessarily to be on the same frequency layer, i.e. the SSB frequency for the serving cell and candidate cell can be different. In this case, it is called inter-frequency L1L2 based PCell mobility.

Observation 4  In R17, only ‘intra-frequency’ ICBM, i.e. the case when SSBs of serving cell and the cell with additional PCI are on the same frequency layer, is supported. However, in R18, both ‘intra-frequency’ and ‘inter-frequency’ L1L2 based mobility, i.e. the cases when SSBs of serving cell and the candidate target cell are either on the same frequency layer or on different frequency layers, need to be supported according to the WID.

Note that in R17 ICBM, the active BWP of the source cell and target cell should also be the same. This restriction is not needed in R18. However, we do not think inter-frequency definition is related to the active BWPs of the source/target cells.

Proposal 3  In R18 FeMob WI, inter-frequency L1L2-based mobility should be supported according to the WID, where the SSBs of active serving cell(s) and the corresponding candidate target cell(s) are on different frequency layers.

As stated in the WID, and agreed in RAN2 as in [3], L1L2-based mobility is based on the configuration of candidate cells. UE is expected to perform L1 measurements on the candidate cells, and send L1 report to the gNB, so that gNB can make the corresponding mobility decision. Based on current RAN1/2 status, it is still not clear how the L1 measurement and reporting should be done for the candidate cells based on the agreed/discussed mechanisms. For example, whether L1 measurement can be performed for an ‘inter-frequency SSB frequency layer’, which has different SSB frequency from the serving cell(s), before such candidate cell(s) configuration is provided, as discussed in [5]. Without RAN1/2 input, it is difficult for RAN4 to proceed, and the valuable RAN4 TU might be wasted. Therefore, we suggest RAN4 to identify the urgent issue that should be prioritized in RAN1/2 discussion, and send LS to RAN1/2, asking RAN1/2 to prioritize such discussion.

Proposal 4  RAN4 to discuss the definition/necessity of inter-frequency L1 measurements after RAN1/2 achieve conclusions on the L1 measurement/reporting mechanism for the potential L1L2 mobility. RAN4 may further discuss the issues that RAN1/2 should prioritize to discuss for RAN4 to proceed, and provide feedback to RAN1/2 via LS.

<On the Rx Timing Difference (RTD) assumption>

In last RAN4 meeting, the following issue was discussed and captured in [6].

<Way forward >: Issue 3-1-5: Timing requirements

  - Revised Option 1 (Ericsson): Specify MRTD of serving cell and neighbour cell if covering the scenario that time offset of serving cell and neighbour cell is more than CP.

  - Option 2 (Apple, QC, HW, Nokia) : FFS, need further clarification

In R17 ICBM, the timing difference between serving cell and additional cell is assumed to be within CP for both FR1 and FR2. This is reasonable because, normally for beam managements/M-TRP transmission, the RTD is assumed to be within CP, no matter whether the signal is transmitted from the same TRP or not. In R16 CSI-RS based L3 measurement, the RTD is also assumed to be within CP for the applicability of RRM requirements, which becomes a big restriction of this feature. Note that no restriction on the RTD of SSB based L3 measurements has been introduced in RAN4.

For R18 L1L2 mobility, the RTD assumption issue needs to be analysed. Actually, in our understanding, it is also related to the L1 measurement and reporting mechanism, which is supposed to be discussed in RAN1 firstly. In our understanding, if intermediate product of L3 measurement results can be used in L1 reporting, as discussed in R17, then there is no need to restrict the RTD assumption to be less than CP at least for SSB based inter-cell L1 measurements. Probably the MRTD specified for the inter-band CA case can be re-used.

Proposal 5  No need to restrict the RTD between serving cell and neighbour cell to be within CP, at least for the case when SSB-based inter-cell L1 measurement is performed.

<On the simultaneous Rx/Tx with both source cell and target cell >

In R16 DAPS HO, simultaneous Rx/Tx with both source cell and target cell is assumed to be supported by a UE, no matter whether the synchronized or asynchronized scenario is assumed.

In R17 ICBM, simultaneous Rx may not be supported on the intra-frequency layer by a UE without such capability in FR2, even for the synchronized scenario. However, in R17 ICBM, UE is assumed to ‘connect’ to both serving cell and the cell with additional PCI, while Rx beam is used for one cell at a time. On the data transmission, UE is only assumed to receive PDCCH and PDSCH, and to transmit SRS/PUSCH/PUCCH to one cell at a time, which is based on the indicated active TCI to be used by UE for the physical data channel or RSs.

In R18 L1L2 mobility, for the intra-frequency synchronized scenario, the same methodology as R17 ICBM can be re-used, especially if the inter-cell M-TRP, which is discussed in R18 MIMO evo. WI, is not considered in this WI. In other word, simultaneous Rx/Tx with both source cell and target cell is not considered for a UE without such capability in FR2, but simultaneous Rx for RS signal can be considered in FR1. For data transmission, the UE is assumed to Rx/Tx to one cell between serving cell and intra-frequency candidate cell based on the active TCI of the UE.

For intra-frequency async scenario, in our understanding it refers to the case when SFN boundary between serving cell and intra-frequency neighbour cell is not aligned. For this case, it would be difficult to achieve simultaneous Rx/Tx with two difference cells. Normally UE only considers to support such large MRTD in DC scenario, which would be the same as DAPS HO, and is not preferred.

For inter-frequency scenario to be assumed in L1L2 mobility, since mobility within one CG is considered in this release, in our understanding UE would be able support simultaneous Rx/Tx if it supports the corresponding CA band combination, and if the RTD between cells is within MRTD/MTTD.

Proposal 6  Do not consider the case when UE always support simultaneous Rx/Tx with both source cell and target cell without any restriction to the scenario in R18, which is similar to the case of DAPS HO.

Observation 5  UE is able to simultaneous Rx with both source cell and target cell for the scenarios when

- the RTD between source cell and target cell is within MRTD for inter-band CA in FR1, or

- the RTD between source cell and inter-frequency target cell is within MRTD for inter-band CA in FR2, and IBM is assumed, or

- the RTD between source cell and intra-frequency target cell is within MRTD for inter-band CA, and UE support simultaneous Rx with different QCL-D in FR2, or

- the RTD between source cell and inter-frequency target cell is within MRTD for inter-band CA, and UE support simultaneous Rx with different QCL-D in FR2, if CBM is assumed between the source cell and target cell.

Observation 6  UE is able to simultaneous Tx with both source cell and target cell for the scenarios at least when

- the Tx timing difference (TTD) between source cell and inter-frequency target cell is within MTTD for inter-band CA in FR1, or

- the TTD between source cell and inter-frequency target cell is within MTTD for inter-band CA in FR2, and IBM is assumed, or

- the RTD between source cell and intra-frequency target cell is within MTTD for inter-band CA, and UE support simultaneous Tx with different panels in FR2, or

- the RTD between source cell and inter-frequency target cell is within MTTD for inter-band CA, and UE support simultaneous Tx with different panels in FR2, if CBM is assumed between the source cell and target cell.

For the issue whether the case UE support simultaneous Rx with different QCL-D in FR2 is discussed in this WI, in our understanding RRM requirements for R18 feature should be discussed based on R17 features firstly. It is not good to have parallel discussion in different WIs.

Proposal 7  Simultaneous multi-panel Rx/Tx in FR2 is not considered in this WI.

<Other issues>

In last meeting, the following issue was discussed.

<Way forward >: Issue 3-2-6: Whether to support L1 measurement on multiple cells with PCI different from serving cell

  - Option 1(Apple, HW, vivo, Ericsson, MTK, Nokia): to discuss the number of supported cells with PCI different from serving cell.

    - Option 1a: To discuss the number of supported cells with PCI different from serving cell on a FR2 band

    - Option 1b: To discuss the total number of cells to be monitored on all bands

    - Option 1c: To discuss the total number of cells to be monitored per frequency layer

  - Option 2 (QC): discuss this issue later

In our understanding, the terminology of ‘cells with PCI different from serving cell’ would be mainly for R17 ICBM. For R18 L1L2 mobility, the terminology of ‘candidate cell’ would be better. Based on latest RAN1/2 progress, it would be difficult to discuss the number of supported candidate cells that supported by UE, since it is not clear yet whether UE needs to perform L1 measurements on all the configured candidate cell according to the mechanism specified in R18.

Proposal 8  Whether to support L1 measurement on multiple candidate cells should be firstly discussed in RAN1/2.

In last meeting, the following issue is discussed.

<Way forward >: Issue 3-3-1: Intra-frequency L1-RSRP measurement accuracy requirements

  - Option 1 (Nokia, vivo): Discuss whether intra-frequency L1-RSRP measurement accuracy can be improved for L1/L2 mobility

  - Option 2 (MTK, HW): reuse the legacy intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

  - Option 3 (Apple, Ericsson, QC): Discuss this issue later, e.g. in performance part

In our understanding, this is related to the scenario that assumed for L1L2 mobility. Since this is the first time when L1 measurement is considered in the mobility scenario, whether to perform L1 measurements in the lower SNR condition may need to be further discussed. At least we see if the intermediate product of L3 measurement results is re-used in L1 reporting, the accuracy of L1 measurement can be improved compared to legacy.

Proposal 9  RAN4 further clarify the scenario of L1 measurement in the context of mobility, i.e. whether lower SNR would be assumed compared to the case when L1 measurements are only used for beam managements. If no clear consensus, LS to RAN1/2 can be considered.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  In SCell activation and R17 ICBM, high-performance data transmission can be ensured at the end of the related procedures, from RRM requirements perspective.

Observation 2  From RAN4 RRM perspective, the TRS tracking can be merged with the T_firstSSB only when the corresponding cell is known, i.e. corresponding L3 measurements report is sent by UE within a pre-defined period before the L1L2 activation signalling is received. For FR2, the known conditions of corresponding cell to be activated by L1L2 signaling have further restricted that, the SSB index associated to the TCI state of the SCell has to be one of the reported SSB indexes in MR.

Observation 3 High-performance data transmission is not always ensured after legacy HO or PSCell addition/change, from RRM requirements perspective.

Proposal 1  RAN4 to align common understanding on whether TRS tracking for high-performance data transmission is always considered in legacy mobility procedure, including HO, PSCell addition/change, SCell activation, activation of TCI from cell with additional PCI.

Proposal 2  TRS tracking for high-performance data transmission should always be considered in R18 L1L2 mobility related enhancements, no matter it is considered in legacy mobility latency or not.

Observation 4  In R17, only ‘intra-frequency’ ICBM, i.e. the case when SSBs of serving cell and the cell with additional PCI are on the same frequency layer, is supported. However, in R18, both ‘intra-frequency’ and ‘inter-frequency’ L1L2 based mobility, i.e. the cases when SSBs of serving cell and the candidate target cell are either on the same frequency layer or on different frequency layers, need to be supported according to the WID.

Proposal 3  In R18 FeMob WI, inter-frequency L1L2-based mobility should be supported according to the WID, where the SSBs of active serving cell(s) and the corresponding candidate target cell(s) are on different frequency layers.

Proposal 4  RAN4 to discuss the definition/necessity of inter-frequency L1 measurements after RAN1/2 achieve conclusions on the L1 measurement/reporting mechanism for the potential L1L2 mobility. RAN4 may further discuss the issues that RAN1/2 should prioritize to discuss for RAN4 to proceed, and provide feedback to RAN1/2 via LS.

Proposal 5  No need to restrict the RTD between serving cell and neighbour cell to be within CP, at least for the case when SSB-based inter-cell L1 measurement is performed.

Proposal 6  Do not consider the case when UE always support simultaneous Rx/Tx with both source cell and target cell without any restriction to the scenario in R18, which is similar to the case of DAPS HO.

Observation 5  UE is able to simultaneous Rx with both source cell and target cell for the scenarios when

- the RTD between source cell and target cell is within MRTD for inter-band CA in FR1, or

- the RTD between source cell and inter-frequency target cell is within MRTD for inter-band CA in FR2, and IBM is assumed, or

- the RTD between source cell and intra-frequency target cell is within MRTD for inter-band CA, and UE support simultaneous Rx with different QCL-D in FR2, or

- the RTD between source cell and inter-frequency target cell is within MRTD for inter-band CA, and UE support simultaneous Rx with different QCL-D in FR2, if CBM is assumed between the source cell and target cell.

Observation 6  UE is able to simultaneous Tx with both source cell and target cell for the scenarios at least when

- the Tx timing difference (TTD) between source cell and inter-frequency target cell is within MTTD for inter-band CA in FR1, or

- the TTD between source cell and inter-frequency target cell is within MTTD for inter-band CA in FR2, and IBM is assumed, or

- the RTD between source cell and intra-frequency target cell is within MTTD for inter-band CA, and UE support simultaneous Tx with different panels in FR2, or

- the RTD between source cell and inter-frequency target cell is within MTTD for inter-band CA, and UE support simultaneous Tx with different panels in FR2, if CBM is assumed between the source cell and target cell.

Proposal 7  Simultaneous multi-panel Rx/Tx in FR2 is not considered in this WI.

Proposal 8  Whether to support L1 measurement on multiple candidate cells should be firstly discussed in RAN1/2.

Proposal 9  RAN4 further clarify the scenario of L1 measurement in the context of mobility, i.e. whether lower SNR would be assumed compared to the case when L1 measurements are only used for beam managements. If no clear consensus, LS to RAN1/2 can be considered.

- References

[1]  RP-213565	New WID on Further NR mobility enhancements, MediaTek.  RAN #94e

[2]  R4-2214660	Work Plan for Further NR Mobility Enhancements, MediaTek.  RAN4 #104-e

[3]  R2-2209257	LS on L1/L2-based inter-cell mobility, MediaTek.  RAN2 #119-e

[4]  R2-2209256	Report of [Post119-e][036][feMob] Time Chart, MediaTek.  RAN2 #119-e

[5]  R2-2210329	[Post119-e][048][feMob] Candidate target configurations for L1/L2 mobility, Ericsson.  RAN2 #119-e

[6]  R4-2214348	WF on Further NR Mobility Enhancements, MediaTek.  RAN4 #104-e