3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-2216394

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item: 	4.2.7.3.1

Source:	Ericsson

Title:	Discussion on remaining issues for PDSCH requirement for NTN

Document for:	Discussion

# 1	Introduction

For UE demodulation of NTN solution, most of the test parameters are settled except the modulation order and SCS/CBW set.

In this contribution, we will give our views on whether it is necessary to specify PDSCH performance requirement for 64QAM and 30kHz SCS scenario.

# 2	Discussion

## 2.1 Modulation order

As for the modulation order, we have previously agreed on introducing performance requirement for QPSK and 16QAM. Now the remaining issue is whether to specify additional requirement for 64QAM. Proposals are captured from the last meeting’s WF [1]:

Issue 2-5: Modulation order - Proposals: to confirm the following tentative agreements: - Option 1: Do not specify PDSCH for performance requirements for 64QAM - Option 2: Specify PDSCH for performance requirements for 64QAM - Agreement: - FFS whether specify PDSCH requirements for 64QAM in Rel-17, operators’ feedback on the demand are encouraged.

In our view, there are two reasons makes us reluctant to consider additional requirement for 64QAM:

- Supporting UL/DL 64QAM for NTN operation as optional with per band, which is an agreement in RAN4 102-e meeting.

Further discuss below two options - Option 1: Supporting UL/DL 64QAM for NTN operation as optional with [per UE/per band] - Option 2: Mandatory same as TN operation Agreement: Supporting UL/DL 64QAM for NTN operation as optional with per band Decision: Approved.

- There will be low SNR condition for NTN communications currently so that it is difficult for supporting the transmission of 64QAM.

Considering the reasons above, we find it less meaningful to introducing additional requirements for 64QAM at least for Rel-17. We are open to come back and discuss it in the future release if needed.

Proposal 1: Do not specify PDSCH requirement for 64QAM

## 2.2 SCS/CBW set

Another open issue is whether to specify additional requirements for 30kHz SCS scenario.

Issue 2-6: SCS/CBW set - Proposals: to confirm the following tentative agreements: - Option 1: Do not specify requirements with additional 30KHz SCS for NTN. - Option 2: Specify requirements with additional 30KHz SCS for NTN

From the discussion in the previous RAN4 meetings, 10M/15kHz was agreed to be considered for defining requirement, as it is the most practical configuration of FDD deployment. 30kHz SCS with 20MHz CBW is not very typical and is hardly considered even in R15/16 TN UE FDD requirements. Meanwhile, we don’t see specific needs or planning on such deployment at current stage. We are open to discuss it in the future release if needed.

Proposal 2: Do not specify PDSCH requirement for 30kHz SCS scenario

# 3	Summary

In this contribution, we provided our views on remaining open issues for PDSCH requirement of NTN UE. Proposals are summarized as follows:

Proposal 1: Do not specify PDSCH requirement for 64QAM

Proposal 2: Do not specify PDSCH requirement for 30kHz SCS scenario

# References

[1]	R4-2210661 WF on NTN Demod-general and PDSCH, Qualcomm Incorporated