3GPP TSG-RAN WG4 Meeting #104bis	R4-2216403

Electronic meeting, October 10th – 19th 2022

Agenda Item:	6.12.4

Source:	Ericsson

Title:	UE RF requirements or HST FR2 CA

Document for:	Discussion

# 1	Introduction

During RAN4#104-e, work will begin on enhanced RF requirements for HST in FR2. One of the objectives is to study the tunnel scenario:

- Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4]

# 2	Discussion

During the Rel-17 FR HST WI, two deployment scenarios were identified:

Table 1 Scenario-A

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Ds | 700m |
| Dmin | 10m |
| RRH Tx power | 47dBm |
| RRH height | 15m |
| RRH antenna array | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] |
| Path Loss | RMa LoS |
| UE antenna height | 5m |
| UE antenna array | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] |
| UE noise figure | 10dB |
| ILs | 13 dB |

Table 2 Scenario-B

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Ds | 700m |
| Dmin | 150m |
| RRH Tx power | 47dBm |
| RRH height | 15m |
| RRH antenna array | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] |
| Path Loss | RMa LoS |
| UE antenna height | 5m |
| UE antenna array | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] |
| UE noise figure | 10dB |
| ILs | 13 dB |

The fundamental difference between the two scenarios is that in scenario A, the BS is assumed to be close to the trackside, whereas in scenario B the BS is 150m from the trackside. In terms of requirements, the difference is that the RRM requirements assume more RX beams for scenario B.

Inside a tunnel, the BS will need to be placed close to the trackside and so the tunnel scenario will be similar to scenario A. It is possible that the BS may need to be even closer to the trackside than 10m. This is no fundamental difference though; still the BS antenna will point straight along the track and the UE antenna will point straight along the track.

The tunnel deployment scenario is fundamentally the same as Scenario A in Rel-17.

The channel model assumes LoS between the BS and the UE for scenario A. Inside the tunnel, the channel will also be LoS. It is unlikely that there will be any reflection or dispersion of the LoS beam inside the tunnel; even if the beam is wide enough to reach the tunnel walls and experience reflection, the reflections will be low power and have negligible little time offset from the main LoS signal, since there are no significant differences in pathlengths within the tunnel.

The tunnel pathloss model, fading model and link budget will be the same as scenario A (LoS)

Since the pathloss model will be the same, the link budget will be the same in the tunnel, and there is no need to space the BS any closer than for scenario A.

There is no need for BS to be more closely spaced inside the tunnel than is the case for Scenario A

Based on these observations, we conclude that Scenario A requirements are sufficient to cover the tunnel scenario..

Scenario A requirements are sufficient to cover the tunnel scenario.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The tunnel deployment scenario is fundamentally the same as Scenario A in Rel-17.

Observation 2	The tunnel pathloss model, fading model and link budget will be the same as scenario A (LoS)

Observation 3	There is no need for BS to be more closely spaced inside the tunnel than is the case for Scenario A

Based on the discussion in the previous sections we propose the following:

Proposal 1	Scenario A requirements are sufficient to cover the tunnel scenario.