3GPP TSG-RAN WG4 Meeting #104bis	R4-2216407

Electronic meeting, October 10th – 19th 2022

Agenda Item:	4.1.5.3.3

Source:	Ericsson

Title:	Spectrum purity requirements for FR2

Document for:	Discussion

# 1	Introduction

During RAN4#104-e, a remaining open issue was to decide on the TE spectrum purity requirements for FR2. This document presents some considerations and proposals on spectrum purity.

# 2	Discussion

For E-UTRA repeaters, the requirement on spectral purity from the TE is as follows:

Table D.5-1: Repeater stimulus signal spectral purity requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.15 MHz | 0.015 MHz f_offset < 0.165 MHz | -40 + 20*( f_offset -0.015) dBc | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165 MHz f_offset < 0.215 MHz | -37 dBc | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
|  | 1.015 MHz f_offset < 1.5 MHz | -106 dBm | 30 kHz |
| 1 MHz f < 2.8 MHz | 1.5 MHz f_offset < 2.85 MHz | -78 dBm | 1 MHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -80 dBm | 1 MHz |

It is not entirely clear how the spectrum purity requirement has been derived, although it bears some resemblance to the structure of the E-UTRA OBUE requirements.

The spectrum purity of the TE signal is of interest when measuring ACLR and OBUE. It is important that, during the test, the measured output signal from the repeater consists mainly of IM products from the repeater and not re-amplified unwanted emissions from the test equipment.

To decide the spectrum purity requirement, it is of interest to consider the requirements on out of band gain, ACRR, OBUE and ACLR.

The out of band gain requirement is as follows:

| Frequency offset, f_offset_CW | Maximum gain |
| --- | --- |
| 0.1*Minimum {400MHz, passband BW}  f_offset_CW < 150 MHz | 68 dB |
| 150 MHz  f_offset_CW < 400 MHz | 55 dB |
| 400 MHz  f_offset_CW < f_offset_max | 35 dB |

The most stringent ACRR requirement is for DL and is as follows:

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit |
| --- | --- | --- | --- |
| NR | Wide Area repeater | minimum {400MHz, passband BW}/2 | 28 (Note 2) 26 (Note 3) |
|  | Medium Range repeater | minimum {400MHz, passband BW}/2 | 28 (Note 2) 26 (Note 3) |
|  | Local Area repeater | minimum {400MHz, passband BW}/2 | 28 (Note 1, 2) 26 (Note 1, 3) |
| NOTE 1: This requirement does not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |

The most stringent OBUE requirement is Category B and is as follows:

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within gaps between passbands is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the gap between passbands. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

The most stringent ACLR requirement is as follows:

| Repeater nominal channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | Repeater adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| min(400MHz, BWpassband) | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWChannel and BWConfig are the nominal repeater bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides nominal bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |  |

When measuring OBUE, it is of importance that the emissions from the TE, amplified according to the OOB gain are significantly less than the OBUE limits. We assume that 20dB lower than the OBUE limits is sufficient. A spectrum purity requirement can then be obtained by taking the most stringent OBUE requirement, subtracting the OOB gain and subtracting 20dB.

The OOB gain is not specified for the range 0-0.1 MHz from the carrier, so we assume that the OOB gain in this range could be as high as a typical repeater gain, which we assume to be 90dB. For the range 0.1MHz-150MHz the gain limit is 68dB. For convenience, we assume that the OOB gain is 68dB from 0.1MHz until fB. This is rather pessimistic for carrier bandwidths above 100MHz (2*carrier bandwidth will be more than 150MHz) and optimistic for the 50MHz carrier bandwidth with a single carrier. RAN4 may need to further discuss whether this convenience leads to reasonable requirements.

For >fB, we assume 55dB gain. Again, this may be rather pessimistic, but is convenient.

Using this approach, the spectrum purity limit becomes as follows:

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | -122 dBm | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | -108 dBm | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | -85 dBm | 10 MHz |

The most difficult limit to meet for the TE may be the first one with -122 dBm. It should be noted that the level is the EIS level at the input to the repeater, thus the emissions from the TE will be attenuated by the pathloss in the test chamber. Nonetheless, if e.g. a repeater would have an output power of 30dBm and a 90dB gain then the input level would be -60dBm, and so around 40dB of suppression would be required in this range for a 100MHz carrier (lower suppression for -60dBm spread over a wider bandwidth).

If the limit it too challenging for TE, it could be relaxed somewhat, to e.g. -112dBm. However, if the TE spectral purity is relaxed too far then it would become increasingly likely that a repeater would fail the OBUE test due to re-amplified emissions from the TE (depending on the real repeater OOB gain in the range).

Since OOB gain is not defined in the range, an alternative approach could be to decide the OBUE requirement in the range of 0-0.1MHz to be untestable if the TE could not achieve the required spectral purity.

For the ACLR measurement, if the TE spectrum purity would be the same as indicated in the table then, due to reduced repeater gain over the adjacent channel compared to the wanted channel, the re-amplified unwanted emissions from the repeater would not significantly impact ACLR measurement from the repeater PA.

# Conclusion

Based on the discussion in the previous section, the following is put forward for a spectrum purity mask for the TE (defined as EIS at the repeater RIB):

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | -122 dBm | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | -108 dBm | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | -85 dBm | 10 MHz |

Feedback is requested regarding the feasibility of this mask. In particular the mask in the range 0-0.1MHz may be challenging, but on the other hand if it is not achieved then measurement of OBUE may be infeasible for certain types of repeater.

For the value from 0.1MHz to fB, the solution is straightforward but pessimistic regarding the repeater gain and could be further examined if needed.