3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216419

Electronic Meeting, October. 15-26, 2022

Agenda Item:	7.4.2

Source: 		Qualcomm Incorporated

Title: 	Coexistence study of 5G terrestrial broadcast

Document for:	Approval

# 1. Introduction

5G broadcast coexistence study was agreed to be conducted in [1] in RAN4 #104 meeting. This paper provides our considerations on the coexistence scenarios, topologies, and assumptions.

# 2. Discussion

# 2.1 Co-existence scenario

According to R4-2214441, the following scenarios are agreed to be studied for coexistence:

- Coexistence of 5G broadcast BS and 5G broadcast UE in adjacent channels within the same band

  - Shall include in the study coordinated deployments with co-located 5G broadcast BSs and necessary guard bands

  - Non-coordinated deployments can also be studied to understand the limitations, but the WID indicates coordination is expected

- UE blocking for the coexistence of IMT BS and 5G broadcast in adjacent bands

  - Assumption on UE Rx filter capability should be indicated. Guard bands between adjacent 5G broadcast and IMT should be evaluated and quantified.

There are already broadcast regulations to define to broadcast transmission limits, therefore, the broadcast-to-broadcast coexistence simulation is not necessary.  Three simulation scenarios to study the broadcast and TN coexistence are proposed in below Table 1.

Table 1. Coexistence simulation scenarios

| No. | Combination | Aggressor | Victim | Simulation frequency | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | broadcast with TN | TN UL 10MHz | broadcast DL 6/7/8MHz | 700MHz | Example: n28 UL, n71 UL |
| 2 | broadcast with TN | TN DL 30MHz | broadcast DL 6/78MHz | 700MHz | Example: n71 DL |
| 3 | broadcast with TN | broadcast DL 6/7/8MHz | TN DL 30MHz | 700MHz | Example: n71 DL |

Proposal 1: Three simulation scenarios to study the broadcast and TN coexistence are proposed in Table 1.

# 2.2 Co-existence topology

Considering large coverage area of 5G broadcast, the TN stations are grouped in one cluster (19 cells 57 sectors for each cluster) and randomly dropped in the broadcast coverage. For each snapshot, the TN cluster will change its location. The coexistence topology is proposed in Figure 1.

The border area, whose area of the ring is 5% of the total coverage area shown in the Figure 1, can be used to measure the edge performance.

The broadcast UE is assumed to be the same density with TN UE, and is dropped uniformly in the TN cluster for each snapshot.

Figure 1. Topology for 5G broadcast and IMT

(IMT clusters are dropped randomly in broadcasting coverage area)

# 2.3 System parameters

Table 2. Broadcast BS parameters

| Parameter | Value |
| --- | --- |
| Frequency | 650 MHz |
| Cell radius | 30 km |
| Tx antenna height | 300 m |
| Tx power | 70 dBm |
| Tx antenna gain | 13 dBi |
| Vertical pattern of Tx antenna | ITU-R Rec. F.1336 |
| Noise figure | 5 dB |

Table 3. Broadcast UE parameters

| Parameter | Value |
| --- | --- |
| UE bandwidth | 6/7/8 MHz with 10MHz filter |
| UE Antenna height (note 1) | 1.5 m |
| UE antenna gain | 0 dBi |
| UE Noise figure | 9 dB |
| Active user number (note 2) | 171 |
| Note 1: broadcast UE has 3 types of bandwidths with 10MHz filter. In the coexistence simulation, one bandwidth can be chosen as the exemplary bandwidth, for example 8MHz. Note 2: broadcast UE only have receiving mode. Assuming broadcast UE has the same density with the TN UE, and the broadcast UE are dropped in the same coverage area of TN UE to simulate the interference. |  |

Table 4. TN BS parameters

| Parameter | Value |
| --- | --- |
| Frequency | 700 MHz |
| ISD | Urban 500 m or 1000m |
| Bandwidth | 30 MHz |
| Tx EIRP | 46 dBm |
| Antenna gain | 15 dBi |
| Tx antenna height | 30 m |
| Antenna type | 3 sectors |
| Noise figure | 5 dB |

Table 5. TN UE parameters

| Parameter | Value |
| --- | --- |
| Bandwidth | 10 MHz for UL, 30MHz for DL |
| Max Tx power | 23 dBm |
| Min Tx power | -40 dBm |
| Tx antenna gain | 0 dBi |
| Tx antenna height | 1.5 m |
| Antenna type | Omni |
| Noise figure | 9 dB |
| Active user | 3 UE/sector for UL and 1 UE/sector for DL |

The feeder loss and body loss are not considered here.

# 2.4 ACLR and ACS modelling

The 5G broadcast UE is assumed to have the similar ACS requirement with the TN UE. 3-steps ACLR and ACS modeling are assumed. The scaling factor F_ACLR = 10 × LOG10(B_victim/B_Aggressor) is considered when the aggressor and victim have different bandwidth.

For each proposed test case, the ACLR and ACS modelling is illustrated as below.

- For test case 1, TN UL interfering to the broadcast DL, TN UE ACLR (30, 43, 50)-F_ACLR apply.

- For test case 2, TN DL interfering to the broadcast DL, TN BS ACLR 45dB and broadcast UE ACS 33dB apply.

- For test case 3, broadcast DL interfering to the TN DL, the broadcast BS ACLR and TN UE ACS values apply.  The broadcast BS ACLR value should be consistent with the broadcast regulations.  TN UE ACS 33dB applies.

# 2.5 Propagation model

Below propagation model are proposed to be adopted in the coexistence simulation.

1)	Propagation model used between broadcast BS and broadcast UE, between broadcast BS and TN UE

- Recommendation in P.1546-6

- look-up tables for the 50% time curves and frequency of 700MHz with interpolating method between 600MHz and 2000MHz.

- σ = 5.9 dB

2)	Propagation model used between TN BS and TN UE, between TN BS and broadcast UE

- Propagation model in 36.942

- L = 118.6 + 37.6*log10(d(km)) for marco urban area

- σ = 10 dB

3)	Propagation model used between TN UE and broadcast UE

- Dual slope model in P.1411

- L(d <= dbreak) = 32.44 + 20*log10(d(km)) + 20*log10(f);

- L(d > dbreak) = 32.44 + 40*log10(d(km)) + 20*log10(f) - 20*log10(dbreak);

- dbreak = 4*TxAntennaHeight(m)* RxAntennaHeight(m)/(300/f)/1000;

- σ = 4 dB

Proposal 2:  the coexistence scenarios, topology, system parameters, ACLR/ACS modeling, and the propagation model provided in this paper are proposed to be adopted in the 5G broadcast coexistence simulations.

# 2.6 Transmission power control model

The UL transmission power control model will be used in the test case 1, TN UL interfering to broadcast DL. The TN UL Power control modeling in TS36.942 is proposed to be used as below.

Pt=Pmax×min1, maxRmin, CLCLx-ileγ

The CLx-ile value need to be tuned and calibrated considering the carrier frequency, pathloss and deployment parameters.

Proposal 3: the CLx-ile value in the TN UL transmission power control model needs to be tuned and calibrated considering the carrier frequency, pathloss and deployment parameters.

# 2.7 Performance metric and coexistence criteria

The traditional DTB-2 broadcast planning criteria can be referred to ITU-R BT.2254.  In defining coverage, it is indicated that due to the very rapid transition from near perfect to no reception at all, it is necessary that the minimum required signal level is achieved at a high percentage of locations. These percentages have been set at 95% for “good” and 70% for “acceptable” portable reception. For mobile reception the percentages defined were 99% and 90%, respectively.

Different with the TN performance using throughput performance, the SNR performance is used for the broadcast planning and performance evaluation. I/N < -6dB is used for the coexistence criteria. Converting the I/N to SINR difference, we can roughly get that SINR loss < 1dB.

Proposal 4: the SINR loss < 1dB is proposed to be used as the 5G broadcast coexistence criteria.

# 3. Conclusion

In this paper, we discussed the assumptions that needed for the 5G broadcast coexistence simulations.  We have the following proposals:

Proposal 1: Three simulation scenarios to study the broadcast and TN coexistence are proposed in Table 1.

Proposal 2:  the coexistence scenarios, topology, system parameters, ACLR/ACS modeling, and the propagation model provided in this paper are proposed to be adopted in the 5G broadcast coexistence simulations.

Proposal 3: the CLx-ile value in the TN UL transmission power control model needs to be tuned and calibrated considering the carrier frequency, pathloss and deployment parameters.

Proposal 4: the SINR loss < 1dB is proposed to be used as the 5G broadcast coexistence criteria.

# 4. References

[1] R4-2214441, “ WF on the scope of a coexistence study for HPHT 5G broadcast”, 3GPP TSG RAN Meeting #104e, Electronic Meeting, August 15-26, 2022