3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216450

Electronic Meeting, 10 – 19 Oct, 2022

Title:	Views on FR2 OTA enhanced test method

Source:	OPPO

Agenda item:	6.5.2

Document for:	Approval

- Introduction

The new study item of NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements was started from the last RAN4 meeting, i.e. RAN4#104-e. One of the main objectives of the SI is to define FR2-1 OTA test method for UEs with multi-panel reception and 4 downlink layers. Several potential test methods were proposed in the last meeting and summarized in the WF [1]. And the final selected test method will highly depend on how the RF requirement specified for multi-Rx chain reception in the core requirement WI. In the core requirement discussion in the last RAN4 meeting, 4 options of candidate AoA pair were captured in the WF [2] for further discussion.

This contribution presents our views on the potential FR2 test methods.

- Discussion

6 options as potential test methods for RF testing are proposed in several contributions [3][4][5], and preliminarily discussed. The basic characteristics of the proposed test methods are summarized and listed as below.

| Potential test methods | Basic characteristic of the test methods | Example illustration |
| --- | --- | --- |
| Test method 1 | IFF + IFF with two moving reflectors. Both of the two IFFs have full rotational degrees of freedom. |  |
| Test method 2 | IFF + DFF, replaced IFF with DFF compared with Test method 1 Both the IFF and the DFF have full rotational degrees of freedom. |  |
| Test method 3 | IFF + fixed DFF, several DFF are fixed at scheduled positions IFF has full rotational degrees of freedom, while DFFs does not. |  |
| Test method 4 | Sequential tests of each UE antenna panel, active antenna in the UE are fixed during the test by a new test command. There are no two active AoAs in this test method. |  |
| Test method 5 | IFF + rotating UE and anchor probe as a whole IFF has full rotational degrees of freedom due to dual-axis turntable, while the anchor probe has not rotation freedom. |  |
| Test method 6 | Reuse the legacy RRM test setup The DUT is placed on the 3D rotational positioner, while test antennas of the system are fixed. |  |

Besides, in the core part discussion on the RF requirement of the multi-Rx reception, there are four candidate AoA pairs proposed and agreed for further discussion, shown as below.

As summarized above, some potential test methods have full rotational degrees of freedom on the two AoAs, and other test methods don’t. Based on the characteristic of these potential test methods, they may be applied for measuring different kinds of candidate AoA pairs.

| Potential test methods | Suitable for which kind of Candidate AoA pairs |
| --- | --- |
| Test method 1 | Candidate 1/2/3/4 |
| Test method 2 | Candidate 1/2/3/4 |
| Test method 3 | Candidate 3 |
| Test method 4 | Not support two active AoAs |
| Test method 5 | Candidate 1, probably can support Candidate 2 |
| Test method 6 | Candidate 3 |

Test method 1 and 2 have full flexibility on the rotation freedom of the two AoAs, can consequently support all the four candidate AoA pairs. However, the complexity of the test system is greatly increased compared with the legacy one single AoA test system. And furthermore, test time for Candidate 4 (Full set of AoA1 + Full set of AoA2) is unaffordable for UE RF measurement.

With these considerations, it is proposed to study Test method 3/5/6 with high priority. And further simplifications on Test method 1/2 are not precluded.

Proposal: It is proposed to study Test method 3/5/6 with high priority. And further simplifications on Test method 1/2 are not precluded.

- Conclusions

This paper discussed about the potential FR2 enhanced test methods. And got the following conclusions.

Proposal: It is proposed to study Test method 3/5/6 with high priority. And further simplifications on Test method 1/2 are not precluded.

- References

[1] R4-2214357, WF on NR FR2 OTA testing enhancements, Qualcomm Incorporated

[2] R4-2214457, WF on FR2 multi-Rx chain DL reception, Sony

[3] R4-2211549, Views on FR2-1 RF OTA test for a device with multi-panel reception, Anritsu Corporation

[4] R4-2211991, Considerations on FR2 multiple AoA test, Samsung

[5] R4-2213627, Discussion on FR2 methods for UEs with multi-panel reception, Rohde & Schwarz