3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216465

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on configuration of HO aspects for NTN

Agenda item:	4.2.6.3

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. Following the progress of RAN4 on the topic, it was approved an extension for the WI activities, in order to address eventual open issues. The WI exception list provided in [2] includes RAN4 RRM requirements, with provision for clarifications and addressing possible inconsistencies.  This contribution proposes further clarification for issues regarding the consistency of GNSS-related operation by UEs in NTN.   In [3] and  [4]several issues are listed, of which we discuss several in the following sections.

Discussion

During HO discussions in previous meetings, it was agreed that the legacy HO test cases for TN are the baseline to be used as reference for the HO in NTN. The differences in the test cases shall account for specificities of NTN HO mechanism.

The HO requirements for a UE connected to a NTN cell is described in TS 38.133, in Section 6.1C [5]. And some of the requirements captured in specifications are:

- Handover Delay: When the SAC UE receives a RRC message implying handover to NR SAN cell, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover msec from the end of the last TTI containing the RRC command.

- Interruption Time: If the valid parameters of ephemeris information, epoch time of the ephemeris, common TA, validity timer information, DL and UL Polarization information, Koffset , and Kmac  for target NR SAN cell are send to UE, the interruption time shall be less than Tinterrupt

However, as we understand, it is not  always possible for the UE to comply with these requirements. The reason is related to additional delays forcibly introduced in the handover execution in NR as the UE prepares for the (random access) transmission in the target cell caused by the validity timer of the satellite assistance information for the target cell.

This is caused by the following aspects:

- A UE that does not have a validity timer running towards the target cell, is assumed out-of-sync and cannot perform UL transmissions.

#### From 3GPP TS 38.331 #### T430 expiry The UE shall: 1> if in RRC_CONNECTED: 2> inform lower layers that UL synchronisation is lost; 2> acquire SIB19 as defined in clause 5.2.2.3.2; 2> upon successful acquisition of SIB19: 3> inform lower layers that UL synchronisation is obtained; #### From 3GPP TS 38.133: The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. and the UE has a validity time running for NTA,common and NTA,UE-specific.

- T430 (validity timer) can only be initiated at the epoch time provided in the satellite assistance information.

#### From 3GPP TS 38.331 #### “Upon receiving SIB19, the UE shall: #### 1> start or restart T430 with the duration ntn-UlSyncValidityDuration from the subframe indicated by epochTime; NOTE: UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation. #### From 3GPP TS 38.133: The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. and the UE has a validity time running for NTA,common and NTA,UE-specific.

- There is only one instance of the timer T430 running at the UE side – and it is associated to the serving cell. The UE is not expected to have T430 running towards the target cell.

#### From RAN 2 #119 agreements: “RAN2 does not need to capture T430 for neighbour cells in all RRC states, and it is up to UE implementation on how to re-acquire SIB19 for neighbour cells (e.g. maintain one or multiple timers for serving cell and neighbour cells). UE needs to know information about neighbour cell’s”

- Upon receiving the HO command the UE must:

  - stop the running T430 in the serving cell

  - then start the new T430 for the target cell before performing the Random Access procedure.

#### From RAN 2 #119 agreements: “During HO/CHO execution upon applying target cell configuration, UE should: 1. Stop the current T430 (if it is running); 2. Start T430 for the target cell as indicated by ntn-UlSyncValidityDuration and epochTime of the target cell”

In short, the UE does not have a validity timer running for the target cell until it receives the HO command. And the transition between 4a and 4b is not instantaneous, as the UE must follow the satellite assistance information for the target cell to initiate the HO procedure. Regardless when the HO command is received by the UE, the new T430 can only be initiated when the epoch time for the target cell is reached.

Observation 1: Upon receiving the HO command, the UE must stop T430 and wait until the epoch time of the target cell is reached to initiate a new T430.

Observation 2: In the NTN HO, additional delay and/or interruption time is introduced by the waiting time until epoch time is reached.

Observation 3: In some cases, the epoch time of the target cell falls within the RRC processing time (20 ms), and no additional action is needed by the UE.

Therefore, based on the observations above, we propose:

Proposal 1: RAN4 to decide the best way to deal with the additional delay introduced in the HO procedure for NTN caused by the initiation of T430 towards the target cell.

Proposal 2:

- If the UE can initiate a valid T430 within the processing interval of the HO delay, no additional action is needed.

-  If the epoch time happens after the processing interval of the HO delay, increase HO interruption time.

Conclusion

Based on our discussions and investigations regarding the validity timer during the handover procedure, we propose:

Observation 1: Upon receiving the HO command, the UE must stop T430 and wait until the epoch time of the target cell is reached to initiate a new T430.

Observation 2: In the NTN HO, additional delay and/or interruption time is introduced by the waiting time until epoch time is reached.

Observation 3: In some cases, the epoch time of the target cell falls within the RRC processing time (20 ms), and no additional action is needed by the UE.

Therefore, based on the observations above, we propose:

Proposal 1: RAN4 to decide the best way to deal with the additional delay introduced in the HO procedure for NTN caused by the initiation of T430 towards the target cell.

Proposal 2:

- If the UE can initiate a valid T430 within the processing interval of the HO delay, no additional action is needed.

-  If the epoch time happens after the processing interval of the HO delay, increase HO interruption time.

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-220209, Rel-17 Work Item Exception for NR-NTN-solutions, Thales

- R4-2214471, WF on NR NTN RRM requirements, Qualcomm Inc.

- R4-2214473, WF on performance part for NR NTN, Xiaomi

- TS 38.133, NR; Requirements for support of radio resource management, V17.7.0.

- TS 38.213, NR; Physical layer procedures for control, V17.3.0.

- R4-2211638, Discussion on RRM performance requirements for NTN, CATT

- R4-2214135, Email Discussion Summary for [104-e][215] NR_NTN_solutions_RRM_2, Xiaomi