3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216553

E-meeting,10th Oct – 19th Oct,2022

Agenda item:	6.24.2

Source: 	ZTE Corporation

Title: 	Discussion on RF requirements for NCR in Rel-18

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In this contribution, we want to share initial analysis on RF requirement impacts from RAN4 perspective based on the listed feature from RAN1/RAN2 perspective.

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2]

- Beamforming

- UL-DL TDD operation

- ON-OFF information

Note: Power control aspect will be checked in RAN#98e.

Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4]

Study and specify the RF and EMC requirements of NCR if necessary [RAN4]

Note: The existing requirements defined in RAN4 can be reused if applicable.

Note: The work in RAN4 for beam related is expected to start on FR2 first.

Discussion

## 2.1. RF architecture for NCR

Firstly of all, before going into the details of Rel-18 NCR RF requirement discussion, it’s better to understand its RF architecture of NCR and then further discuss how to specify its RF requirements. Compared with Rel-17 RF repeater, one NCR-MT entity is agreed by RAN1 to receive the control information from parent gNB to control the beamforming, UL-DL TDD operation and ON-OFF information of NCR-Fwd link. In addition, NCR-MT transmitter in UL is also necessary to conduct the random access to the network, provide the HARQ feedback for the received DL signal,etc.

In the following Figure 1/ 2, we illustrate how the NCR-MT entity is integrated into the legacy RF repeater in both FR1 and FR2.

For NCR-MT receiver in the downlink direction, NCR-MT is supposed to decode the in-band signals within the pass-band to obtain its side control information to assist the transmission/reception of NCR-Fwd link. For the following figure, NCR-MT could share some RF components with NCR-Fwd link from the cost efficiency perspective, however the dedicated/separated RF chain for NCR-MT receiver is also one possible solution which should be up to the vendor’s implementation.

For NCR-MT transmitter in the uplink direction, NCR-MT is supposed to transmit the in-band uplink signal as necessary behaviors (e.g. to conduct the random access to the network, provide the HARQ feedback for the received DL signal. etc). Similar as NCR-MT receiver, NCR-MT could somehow share some RF components with NCR-Fwd link from the cost efficiency perspective, however as we mentioned before, the dedicated/separated RF chain for NCR-MT receiver is also one possible solution which should be up to the vendor’s implementation.

Figure 1. the generic illustration of FR1 NCR

Figure 2. the generic illustration of FR2 NCR

## 2.2. RF requirements for NCR

From RF perspective, compared with the legacy Rel-17 NR RF repeater, dynamic beamforming capability is supposed to be supported for Rel-18 NCR-Fwd link with assistance by side control information received by NCR-MT. In Rel-17, since RF repeater don’t have dynamic beamforming capability, therefore we reached the following consensus [3] in RAN4#102e meeting which is illustrated in the following Figure 3. For Rel-18 NCR, since dynamic beamforming capability is assumed, therefore we think that these kind of restriction as agreed in Rel-17 could be removed at least. For example, from BS-side RIB, the input signal of DL could be different from the reference direction of UL Tx; from UE-side RIB, the input signal for UL could be different from the reference direction of DL Tx;

Tx Power: On the issue of OTA directions it is agreed: For repeater requirements, the input signal for DL should be the same as the reference direction for UL TX and vice versa. No further input directions declared for Rel-17. This is to be accomplished within the existing beam declaration format.

Figure 3. the generic illustration of the directions for input and output conformance testing in Rel-17

Proposal 1: for Rel-18 NCR-fwd with dynamic beamforming, from BS-side RIB, the input signal of DL could be different from the reference direction of UL Tx; from UE-side RIB, the input signal for UL could be different from the reference direction of DL Tx;

For Rel-18 NCR, ON-OFF information is recommended for network-controlled repeater to control the behaviour of NCR-Fwd. Therefore, the related ON-OFF requirement should be extended from TDD bands to the NCR operating in FDD band. For NCR operating in TDD band, the existing ON-OFF requirement should be still reused.

Proposal 2: for Rel-18 NCR-fwd operating in FDD bands in FR1, the existing ON-OFF requirement should be also applicable; for Rel-18 NCR in TDD bands in both FR1 and FR2, no further updates on ON-OFF requirement are needed.

In addition, as we highlighted in the previous Figure 1/2, it’s expected to have shared/dedicated RF chain for NCR-MT from both received and transmitter perspective, therefore we need to further discuss whether it’s necessary to define the corresponding RF requirements for it. In the following section, some initial analysis for NCR-MT is provided as below.

Firstly for NCR-MT receiver, regardless of shared RF chain or dedicated/separated RF chain for NCR-MT, it seems that refense requirement for NCR-MT might be still necessary to check its noise figure performance which might be different from NCR-Fwd link.

Proposal 3: to define the refense requirement for NCR-MT receiver.

In addition, as shown in the following figure 4a/4b/5, it’s better to define the NCR-MT Rx requirement together with ACRR and Input IMD requirement since these interfering signals of ACRR/Input IMD requirements might also have the impacts on the wanted signal received by NCR-MT. For example, the interfering signal of ACRR requirement could have some carrier leakage into the NCR-MT receiver which is similar as ACS requirement of UE/BS and the 3rd order intermodulation product from two CW signal of input IMD requirement could fall into the passband or wanted signal of NCR-MT carrier.

Proposal 4: to define the Rx requirement for NCR-MT together with ACRR requirement and Input IMD requirement in DL and its sensitivity degradation compared with the refense requirement could be further discussed.

Figure 4a. the illustration of ACRR requirement with NCR-MT Rx requirements

Figure 4b. the illustration of ACRR requirement with NCR-MT Rx requirements

Figure 5. the illustration of Input IMD requirement with NCR-MT Rx requirements

Secondly for NCR-MT transmitter, as shown in Figure 1/2, the signal of NCR-MT transmitter and the analog signals of NCR-Fwd link could be combined together to be transmitted in the uplink direction or still fall within pass-band if separated RF chain is assumed for NCR-MT transmitter and NCR-Fwd. Therefore the combined uplink signal could still conform to the legacy uplink transmitter requirement as defined in Rel-17 for RF repeater (e.g. repeater output in uplink, frequency stability, out of gain, unwanted emission, EVM, ON-OFF requirements).

Proposal 5: for the RF requirement of NCR-MT transmitter, it should be tested together with NCR-Fwd in the uplink and still conform to the legacy uplink transmitter requirement as defined in Rel-17.

Conclusions

In this contribution, we want to share initial views on NCR from both RF architecture and RF requirement perspective and observations and proposals are made as following:

Proposal 1: for Rel-18 NCR-fwd with dynamic beamforming, from BS-side RIB, the input signal of DL could be different from the reference direction of UL Tx; from UE-side RIB, the input signal for UL could be different from the reference direction of DL Tx;

Proposal 2: for Rel-18 NCR-fwd operating in FDD bands in FR1, the existing ON-OFF requirement should be also applicable; for Rel-18 NCR in TDD bands in both FR1 and FR2, no further updates on ON-OFF requirement are needed.

Proposal 3: to define the refense requirement for NCR-MT receiver.

Proposal 4: to define the Rx requirement for NCR-MT together with ACRR requirement and Input IMD requirement in DL and its sensitivity degradation compared with the refense requirement could be further discussed.

Proposal 5: for the RF requirement of NCR-MT transmitter, it should be tested together with NCR-Fwd in the uplink and still conform to the legacy uplink transmitter requirement as defined in Rel-17.

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- TS 38.106, NR Repeater Radio Transmission and Reception; (Release 17)

- R4-2207290, WF on repeater RF requirements, Huawei, approved.

- R4-2205030,Repeater radiated power requirements, Ericsson, Noted

- TS 38.115-2,NR;Repeater conformance testing, Part 2: Radiated conformance testing.

- TS 38.115-1,NR;Repeater conformance testing, Part 1: Conducted conformance testing.