3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216579

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM L3 enhancements for multi Rx DL in FR2

Agenda item:	6.8.3.2

Document for:	Discussion

Introduction

This paper discusses our view on the RRM aspects related to the multi-RX chain reception in FR2 for Rel-18, in particular those concerning L3 measurements. This paper presents an overview on the objectives of the work item and based on those, observations and proposals are derived concerning expected architecture and how it can lead to enhancements for RRM measurements.

Discussion

The WID on Multi Rx has the following objectives related to enhancements for RRM [1]:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs)

As part of those objectives, L3 measurements and scheduling restrictions are addressed, where measurement delay and scheduling restrictions could bennefit from enhancements of multi Rx chain UEs.

One important open item is related to which of the RRM requirements could be enhanced in the multi-Rx chain DL reception WI. As part of this discussion the following issue with open discussion point is captured in the agreed WF [2]:

Issue 1-1-4: Feasibility/necessity of enhancing requirements for L3 measurements - Proposals - Option 1: Following L3 measurements related requirements are enhanced in the multi-Rx chain DL reception WI - RRM measurement requirements for RRC-IDLE state and for RRC_INACTIVE state. - Handover to FR2-1 unknown cell - FR2-1 unknown SCell activation - FR2-1 unknown PSCell addition and release - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG). - Option 1a: Following L3 measurements related requirements are enhanced in the multi-Rx chain DL reception WI - Handover to FR2-1 unknown cell - FR2-1 unknown SCell activation - FR2-1 unknown PSCell addition and release - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG). - Option 2a: Not to enhance L3 measurement requirement in R18 multi-panel WI.

We understand that there are important benefits in developing L3 enhancements for multi Rx UEs in FR2. In higher frequency bands (FR2) the necessity of Rx beam sweeping causes large delays on RRM measurements and delays in general which increases the time when the UE cannot be assumed available. For example, one scenario is when UE is performing L3 measurements including sweeping assumptions and another similar scenario is activation delay.

These delays and their possible negative impacts is also recognized in 3GPP as 3GPP has agreed at least one other WI addressing latency reductions. Hence, our view is that RAN4 should look at how to optimize overall system performance for multi-Rx chain UEs.

From the last WF some of available options were including requirements for both RRC connected and RRC idle/inactive requirements. We understand that there are benefits from improvements for RRC idle/inactive performance in some scenarios (discussed in Mob_enh2 WI), but for this WI we see the ones related to RRC connected more in scope. Therefore, we propose focusing on defining requirements for RRC connected in this WI, at least for a few meetings. Once we have progressed on how to improve RRC connected requirements we can discuss how to develop the requirements for RRC idle/innactive.

RAN4 to focus on L3 enhancements for requirements in RRC connected.

RAN4 to discuss the need of L3 enhancements for requirements in RRC idle and inactive in a later phase of the work item.

RAN4 to consider the following requirements for multi Rx enhancements in RRC connected mode

  - L3 measurement in connected mode

  - Handover to FR2-1

  - RRC re-establishment

  - Intra-frequencey measurements

  - Inter-frequency measurements

As part of the agreements reached during the last meeting, we can highlight the following agreement on the number of searchers [2]:

Issue 1-1-1-2: Number of searchers for cell detection and measurements - 2 searchers are assumed.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 1 Possible scenarios for use of multi Rx in DL where (a) is the current Rel 15-17 scenario, (b) measurements can be performed in multiple beams simultaneously; and (c) measurements and SSB monitoring can be performed on different beams

When considering this agreement and considering the multiple Rx chains capability discussed in this work item, we understand that it must be possible to consider to how L3 measurements may be enhanced accounting the multiple Rx chains available at the UE. Such assumption is significantly different than Rel-15 baseline assumption used when developing the Rel-15 basic UE RRM requirements for FR2 where only 1 active Rx chain was assumed.

During last meeting we had an agreement that 2 searchers can be assumed for cell detection and measurements. Given this agreement, we understand that a multi Rx capable UE is able to perform simultaneous RRM measurements using 2 Rx chains as shown in Figure 1 (b). This could implies reduced beam sweeping scaling factor when compared to the Rel 15 to 16 single Rx assumption shown in Figure 1 (a).

Multi Rx UEs, assumed to have 2 searchers, can be assumed being able to use both searchers simultaneously for cell detection and measurements.

RAN4 to discuss updated Rx beam sweeping scaling factor for multi Rx UEs.

RAN4 to discuss RRM requirements with Rx beam sweeping scaling factor equal to N=4.

Additionally, as part of the discussion it has been raised that multi Rx UEs could be capable of performing RRM measurements on one chain, while performing demodulation tasks in another chain. This is shown in an architecture view in Figure 2, which could influence simultaneous RRM measurements and demodulation of data signals as shown in Figure 1 (a).

Figure 2 UE architecture with multiple Rx chains

As part of the Rel-15 to Rel-17  RRM requirements scheduling restrictions apply when the UE is performing RRM measurements. shows on example of such restrictions when considering pure 120 kHz SCS numerology, i.e. SSB and PDSCH using 120 kHz SCS, according to clause 9.2.5.3.3 of TS 38.133. As described in this clause, one symbol before and after the SSB symbols are not available for the UE to be scheduled, since with one Rx the UE might be changing the spatial filter parameters while doing RRM measurements in other directions. From that only 4 out of 14 symbols are still available for scheduling. If we consider that the SMTC window may be at most 5 ms, that means that if all SSB positions are used inside the SMTC window only 28% of the resources – in time - are available during that window. Additionally, since the SMTC is the same for several UEs in the network, most of the UEs will be restricted to the same symbols and that leaves very small room for the network to schedule UEs. As a result, this restriction can generate quite large network impact when considering a busy network.

Figure 3 Scheduling restrictions example for a slot with 120 kHz SCS SSB pattern considering 120 kHz SCS for PDSCH.

Scheduling restrictions in FR2 can impact most of the OFDM symbols inside a SMTC window for measurements without gaps.

Scheduling restrictions in FR2 can apply to 10 out of the 14 OFDM symbols of a slot with 2 SSB occasions configured.

Most UEs in a network would have the same SMTC configuration, resulting in inefficient use of resources with scheduling restrictions.

Furthermore, one open issue relating simultaneous reception of RS and data is open from the last meeting, which is shown below [2]:

Issue 1-1-6-1: UE capability of simultaneous reception of measured RS and data - Proposals - Option 1: Consider simultaneous reception of L1/L3 measured RS and data if UE supports simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. - Option 2: Consider simultaneous reception of L1 measured RS and data if UE supports simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. - Option 3: For scenario 2 (two RX beams used for different cells) with multi-RX chain capability, UE can receive PDCCH/PDSCH/TRS/CSI-RS for CQI (one beam) and on SSB symbols (another beam) to be measured - Option 4: RAN4 to discuss if additional UE capability for indicating supporting of simultaneous reception with different QCL type D RS and PDSCH/PDCCH is needed.

Considering the open issue above, we understand that simultaneous reception of L3 and RS on a single component carrier can be assumed. Therefore we propose the following:

RAN4 to define requirements considering that a multi Rx UE can receive PDCCH/PDSCH/TRS/CSI-RS for CQI and SSB symbols simultaneously from beams with different QCL type D.

However, we do not see this necessarily limited to another cell but could also be from same cell.

RAN4 to discuss how to reduce scheduling restrictions for multi Rx chain UEs.

Power saving was also raised as a concern for the operation using multiple Rx chains which is addressed in the open issue shown below searchers [2]:

Issue 1-1-2-4: Whether and how to define power saving related requirements - Proposals - Option 1: Power saving can be additional requirements of UE with multiple RX chains - Option 2: Power saving is not part of this study - Option 3: no need to define additional requirements for power saving.

It is clear that the UE might benefit with enhanced RRM and higher throughput when using multiple Rx chains. However, some UE implementations might find it challenging to keep the multiple chains active. Therefore, it may be desirable to discuss whether a UE is always assumed using both Rx chains e.g. for L3 RRM or to adapt the number of Rx chain.

Use of multiple Rx chains may increase the UE power consumption unnecessarily at time periods when the UE is not requiring heavy data traffic.

RAN4 to discuss use of multi-Rx chain UEs dual searchers in an efficient way.

Finally, when we consider the possibility for single/multi Rx, reduced beam sweeping capability, and enhanced measurements, the same UE could be performing different operations at different points of time. Therefore, we understand that some differentiation of when each of those enhancements is active is necessary.

RAN4 to study if use of single Rx chain or multiple Rx chain can be adapted over time.

Conclusion

In this paper we have presented Nokia’s views on L3 measurements for multi Rx chains. As part of this discussion, we have reached the following observations/proposals:

- RAN4 to focus on L3 enhancements for requirements in RRC connected.

RAN4 to discuss the need of L3 enhancements for requirements in RRC idle and inactive in a later phase of the work item.

RAN4 to consider the following requirements for multi Rx enhancements in RRC connected mode

  - L3 measurement in connected mode

  - Handover to FR2-1

  - RRC re-establishment

  - Intra-frequencey measurements

  - Inter-frequency measurements

- Multi Rx UEs, assumed to have 2 searchers, can be assumed being able to use both searchers simultaneously for cell detection and measurements.

RAN4 to discuss updated Rx beam sweeping scaling factor for multi Rx UEs.

RAN4 to discuss RRM requirements with Rx beam sweeping scaling factor equal to N=4.

Scheduling restrictions in FR2 can impact most of the OFDM symbols inside a SMTC window for measurements without gaps.

Scheduling restrictions in FR2 can apply to 10 out of the 14 OFDM symbols of a slot with 2 SSB occasions configured.

Most UEs in a network would have the same SMTC configuration, resulting in inefficient use of resources with scheduling restrictions.

RAN4 to define requirements considering that a multi Rx UE can receive PDCCH/PDSCH/TRS/CSI-RS for CQI and SSB symbols simultaneously from beams with different QCL type D.

RAN4 to discuss how to reduce scheduling restrictions for multi Rx chain UEs.

Use of multiple Rx chains may increase the UE power consumption unnecessarily at time periods when the UE is not requiring heavy data traffic.

RAN4 to discuss use of multi-Rx chain UEs dual searchers in an efficient way.

RAN4 to study if use of single Rx chain or multiple Rx chain can be adapted over time.

- References

- RP-221753, "Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception", Qualcomm

- R4-2214344, WF on FR2 multi-Rx RRM, vivo