3GPP TSG-RAN WG4 Meeting #104bis-e     	R4-2216598

E-meeting, October 10th  – 19th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Rel-17 UE power saving for RedCap

Agenda item:	4.6.3.1.4

Document for:	Discussion

- Introduction

- The RedCap WID in [1] considers that power saving enhancements solutions specified in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) are assumed to be available also to RedCap UEs by default; however, the WID also mentions that no RRM measurement relaxations are specified for the serving cell. In Rel-17 RedCap WID, RRM measurement relaxations were defined only for Idle / Inactive mode.

In RAN4 #104, the following was captured in the way forward:

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17 No consensus reached.

In RAN4 #103, the following options were available:

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17 Candidate options: - Option 1 (QC): RAN4 to define further relaxations (e.g., larger K values) to RLM/BFD measurements for a Rel-17 RedCap UE that is configured with and satisfies the stationary criterion. - Option 2 (HW, Apple, CMCC, vivo, OPPO): Relaxed RLM/BFD based on Rel-17 UE power saving WI is out of RedCap WI scope. Option 3 (E///, MTK, Xiaomi, MTK): FFS under maintenance

- In this document, Nokia’s views regarding UE power saving for RedCap UEs are presented.

- Discussion

In the last RAN4 meeting, different companies were against the introduction of RLM/BFD relaxation for RedCap arguing that the RedCap WID stated that no RRM relaxations are to be specified for the serving cell in Rel-17. Even though there are no RRM relaxation for connected mode measurements in Rel-17, the UE could still benefit from relaxing RLM/BFD.

Even though there are no connected mode RRM measurement relaxations in RedCap Rel-17, the RedCap UEs could still benefit from relaxed RLM/BFD.

Both work items NR_redcap and NR_power_sav_enh have defined new RRM requirements for radio link monitoring (RLM) and beam failure detection (BFD). The baseline for the requirements in both features is the RRM requirements defined in NR. Up to Rel-16, the NR RRM requirements have been defined under the assumption that the UEs have 2 Rx branches. This assumption changed in the RedCap work item, which also considers UEs with 1 Rx branch. Furthermore, requirements were defined for Half-duplex FDD UEs (HD-FDD).

In general, the requirements defined for 2 Rx RedCap UEs are the same as the requirements for NR UEs. The RLM/BFD relaxation factors in the NR_power_sav_enh were defined based on the assumption of 2 Rx UEs. Therefore, in our view, RAN4 effort for introducing requirements for RLM/BFD relaxation in Rel-17 for 2Rx RedCap UEs is low.

For 1 Rx RedCap UEs, there is an accuracy degradation caused by reducing the number of Rx branches, when compared to the accuracy for 2 Rx RedCap UEs, as shown in our contributions in [4] and [5]. Therefore, in the RedCap WI, the following modifications were done when comparing to the baseline requirements:

- The OOS evaluation period and the lower bound of the OOS evaluation period were doubled.

- The PDCCH transmission parameters for in-sync and out-of-sync evaluation were changed. For in sync and out-of-sync, the bandwidth and the aggregation level were doubled.

- It is not straightforward to assume RLM/BFD relaxation for 1 Rx RedCap UEs with the same factors that were carefully designed for the 2 Rx UEs.

It is not straightforward to apply the RLM/BFD requirements to 1 Rx RedCap UEs without simulations, since the evaluation periods were modified in comparison to those defined for 2 Rx RedCap.

Considering that for 2 Rx RedCap UEs, the assumptions under which the requirements have been defined are the same as for NR UEs, we consider that there is not an extensive RAN4 effort required to define requirements for RLM/BFD relaxation in this case. Therefore, we propose the following:

RAN4 to prioritize the definition of RLM/BFD relaxation requirements to 2 Rx RedCap UEs in Rel-17.

The RLM/BFD relaxation factors defined in the NR_power_sav_enh WI are applicable to 2 Rx RedCap UEs.

- Conclusions

In this document, Nokia’s view regarding Rel-17 power saving applicability to RedCap UEs is shared. The following observations and conclusions are made:

- Even though there are no connected mode RRM measurement relaxations in RedCap Rel-17, the RedCap UEs could still benefit from relaxed RLM/BFD.

- In general, the requirements defined for 2 Rx RedCap UEs are the same as the requirements for NR UEs. The RLM/BFD relaxation factors in the NR_power_sav_enh were defined based on the assumption of 2 Rx UEs. Therefore, in our view, RAN4 effort for introducing requirements for RLM/BFD relaxation in Rel-17 for 2Rx RedCap UEs is low.

- It is not straightforward to apply the RLM/BFD requirements to 1 Rx RedCap UEs without simulations, since the evaluation periods were modified in comparison to those defined for 2 Rx RedCap.

- RAN4 to prioritize the definition of RLM/BFD relaxation requirements to 2 Rx RedCap UEs in Rel-17.

- The RLM/BFD relaxation factors defined in the NR_power_sav_enh WI are applicable to 2 Rx RedCap UEs.

References

- RP-220966, “Revised WID on support of reduced capability NR devices”

- R4-2214485 WF on RedCap RRM requirements, Ericsson (RAN4 #104)

- R4-2210592 WF on RedCap RRM requirements, Ericsson (RAN4 #103)