# 3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2216638

# Electronic Meeting, 15th  –26th August, 2022

Source:		Ericsson

Title:			TP for SAN RF requirement

Agenda item:		7.5.4

Document for:		Approval

# Introduction

In this paper, we present the TP for some SAN RF requirement for conducted clause 6.

From our ,coexisting simulation paper, it is fine to reuse the 38.108 ACLR and OBUE.

# TP text

# 6	Conducted transmitter characteristics

## 6.1	General

<Text will be added.>

Unless otherwise stated, the conducted transmitter characteristics are specified at the TAB connector for SAN type 1-H, with a full complement of transceiver units for the configuration in normal operating conditions.

## 6.2	Satellite Access Node output power

<Text will be added.>

The SAN conducted output power requirements in clause 6.2 in TS 38.108 apply.

## 6.3	Output power dynamics

### 6.3.1	General

<Text will be added.>

The requirements in clause 6.3 apply during the transmitter ON period. Transmit signal quality (as specified in clause 6.5) shall be maintained for the output power dynamics requirements of this Clause.

Power control is used to limit the interference level.

### 6.3.2	RE power control dynamic range

<Text will be added.>

The SAN RE power control dynamic range requirements in clause 6.3.2 in TS 38.108 apply.

### 6.3.3	Total power dynamic range

<Text will be added.>

#### 6.3.3.1	General

The SAN total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

For SAN type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the dynamic range is the OFDM symbol power for a SAN when transmitting on all RBs at maximum output power. The lower limit of the total power dynamic range is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

#### 6.3.3.2	Minimum requirement for SAN type 1-H

The downlink (DL) total power dynamic range for each SAN carrier shall be larger than or equal to the level in table 6.3.3.2-1.

Table 6.3.3.2-1: Total power dynamic range

| SAN channel | Total power dynamic range (dB) |
| --- | --- |
| bandwidth (MHz) | 15 kHz SCS |
| 1.4 | 7.7 |

## 6.4	Transmit ON/OFF power

<Void.>

N/A

## 6.5	Transmitted signal quality

### 6.5.1	Frequency error

<Text will be added.>

Frequency error is the measure of the difference between the actual BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

#### 6.5.1.1	Minimum requirement

For E-UTRA, the modulated carrier frequency of each E-UTRA carrier configured by the BS shall be accurate to within the accuracy range given in Table 6.5.1-1 observed over a period of one subframe (1ms).

For NB-IoT, the modulated carrier frequency of each NB-IoT carrier configured by the BS shall be accurate to within the accuracy range given in Table 6.5.1-1 observed over a period of one subframe (1ms).

- Table 6.5.1-1:  Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| SAN Type 1-H | ±0.05 ppm |

### 6.5.2	Modulation quality

<Text will be added.>

The EVM quality requirements in clause 6.5.2 in TS 36.104 apply except for 256QAM and 1028QAM.

### 6.5.3	Time alignment error

<Text will be added.>

N/A

### [6.5.4	DL RS power]

<Text will be added.>

DL RS power requirements in clause 6.5.4 in TS 36.104 apply.

## 6.6	Unwanted emissions

### 6.6.1	General

<Text will be added.>

General requirements in clause 6.6.1 in TS 38.108 apply

### 6.6.2	Occupied bandwidth

The occupied bandwidth requirements in clause 6.6.2 in TS 38.108 apply.

<Text will be added.>

<Text will be added.>

### 6.6.3	Adjacent Channel Leakage Power Ratio

<Text will be added.>

The ACLR requirements in clause 6.6.3 in TS 38.108 apply.

### 6.6.4	Operating band unwanted emissions

<Text will be added.>

The Operating band unwanted emissions requirements in clause 6.6.4 in TS 38.108 apply.

### 6.6.5	Transmitter spurious emissions

The Transmitter spurious emissions requirements in clause 6.6.5 in TS 38.108 apply.

# References

- R4-2215119, Draft spec for TS 36.108, ZTE